- EP8204、EP8208  Cassette OLT

- User Manual

Command Line Reference Manual

- Accessing OLT Command

- Port Configuration Command

- VLAN Configuration Command

- ARP Configuration Command

- DHCP Configuration Command

- Multicast Protocol Configuration Command

- ACL Configuration Command

- QOS Configuration Command

- STP Configuration Command

- 802.1x Configuration Command

- SNTP Client Configuration Command

- Syslog Configuration Command

- SSH Configuration Command

- OLT Management and Maintenance Command

- LLDP Configuration Command

- Flex Links Configuration Command

- CFM Configuration Command

- PPPoE Plus Configuration Command

- ERRP Configuration Command

- EFM Configuration Command

- OLT Slot Management Configuration Command

- PON Configuration Command

- ONU Management Configuration Commands

- PSG Management Configuration Command

- Multicast Profile Managmenet Commands

Content

Chapter 1 OLT Logging in Command	- 1 -

1.1 OLT Logging in Command	- 1 -

1.1.1 cls	- 1 -

1.1.2 configure terminal	- 2 -

1.1.3 enable	- 2 -

1.1.4 end	- 3 -

1.1.5 exit	- 4 -

1.1.6 help	- 4 -

1.1.7 hostname	- 5 -

1.1.8 interface	- 6 -

1.1.9 muser	- 6 -

1.1.10 quit	- 7 -

1.1.11 show muser	- 8 -

1.1.12 show tacacs+	- 8 -

1.1.13 show username	- 9 -

1.1.14 stop	- 9 -

1.1.15 tacacs+	- 10 -

1.1.16 timeout	- 11 -

1.1.17 username	- 11 -

1.1.18 username change-password	- 13 -

Chapter 2 Port Configuration Command	- 15 -

2.1 Ethernet Interface Configuration Command	- 15 -

2.1.1 clear interface	- 16 -

2.1.2 combo	- 17 -

2.1.3 description	- 17 -

2.1.4 duplex	- 18 -

2.1.5 flow-control	- 19 -

2.1.6 ingress acceptable-frame	- 20 -

2.1.7 ingress filtering	- 21 -

2.1.8 priority	- 21 -

2.1.9 show description	- 22 -

2.1.10 show interface	- 23 -

2.1.11 show statistics interface	- 23 -

2.1.12 show statistics higig	- 24 -

2.1.13 clear higig-statistics	- 25 -

2.1.14 shutdown	- 26 -

2.1.15 port-control mode	- 26 -

2.1.16 show port-control mode	- 27 -

2.1.17 speed	- 28 -

2.1.18 OLTport mode	- 29 -

2.1.19 OLTport trunk allowed vlan	- 29 -

2.1.20 OLTport trunk default vlan	- 30 -

2.1.21 show statistics dynamic interface	- 31 -

2.1.22 show utilization interface	- 31 -

2.1.23 local-OLT	- 32 -

2.2 Interface Mirror Configuration Command	- 33 -

2.2.1 mirror destination-interface	- 33 -

2.2.2 mirror source-interface	- 33 -

2.2.3 show mirror	- 34 -

2.2.4 no mirror all	- 35 -

2.3 Port CAR Configuration Command	- 35 -

2.3.1 port-car	- 35 -

2.3.2 port-car-open-time	- 36 -

2.3.3 port-car-rate	- 37 -

2.3.4 show port-car	- 37 -

2.4 Port LACP Configuration Command	- 38 -

2.4.1 channel-group	- 38 -

2.4.2 channel-group mode	- 39 -

2.4.3 channel-group load-balance	- 40 -

2.4.4 lacp system-priority	- 40 -

2.4.5 lacp port-priority	- 41 -

2.4.6 show lacp sys-id	- 42 -

2.4.7 show lacp internal	- 43 -

2.4.8 show lacp neighbor	- 43 -

2.4.9 show statistics channel-group	- 44 -

2.4.10 clear channel-group	- 44 -

2.4.11 show statistics dynamic channel-group	- 45 -

2.4.12 show utilization channel-group	- 45 -

2.5 Port Alarm Configuration Command	- 46 -

2.5.1 alarm all-packets	- 46 -

2.5.2 alarm all-packets threshold	- 47 -

2.5.3 show alarm all-packets	- 48 -

2.5.4 show alarm all-packets interface	- 48 -

2.6 Interface shutdown-control Configuration Command	- 49 -

2.6.1 shutdown-control	- 49 -

2.6.2 shutdown-control-open-time	- 50 -

2.6.3 no shutdown-control-recover	- 50 -

2.6.4 show shutdown-control	- 51 -

Chapter 3 VLAN Configuration Command	- 52 -

3.1 VLAN Configuration	- 52 -

3.1.1 description	- 52 -

3.1.2 show vlan	- 53 -

3.1.3 OLTport	- 53 -

3.1.4 vlan	- 54 -

3.2 GVRP Configuration Command	- 55 -

3.2.1 gvrp	- 56 -

3.2.2 show gvrp	- 56 -

3.2.3 show gvrp interface	- 57 -

3.2.4 garp permit vlan	- 58 -

3.2.5 show garp permit vlan	- 58 -

3.3 QinQ command	- 59 -

3.3.1 dtag	- 59 -

3.3.2 dtag mode	- 60 -

3.3.3 show dag	- 60 -

3.3.4 dtag insert	- 61 -

3.3.5 show dag insert	- 61 -

3.3.6 dtag swap	- 62 -

3.3.7 show dag swap	- 62 -

3.3.8 dtag pass-through	- 63 -

3.3.9 show dag pass-through	- 63 -

3.4 l2-tunnel Configuration	- 63 -

3.4.1 l2-tunnel	- 64 -

3.4.2 show l2-tunnel interface	- 65 -

3.4.3 l2-tunnel drop-threshold	- 65 -

3.4.4 show l2-tunnel drop-threshold	- 66 -

Chapter 4 ARP Configuration Command	- 67 -

4.1 ARP Configuration Command	- 67 -

4.1.1 no arp	- 67 -

4.1.2 show arp	- 68 -

Chapter 5 DHCP Configuration Command	- 69 -

5.1 DHCP Configuration Command	- 69 -

5.1.1 dhcp-relay	- 69 -

5.1.2 dhcp-server	- 70 -

5.1.3 dhcp-snooping	- 71 -

5.1.4 dhcp-snooping trust	- 72 -

5.1.5 dhcp-snooping max-clients	- 72 -

5.1.6 ip-source-guard	- 73 -

5.1.7 ip-source-guard bind ip	- 73 -

5.1.8 show dhcp-relay	- 74 -

5.1.9 show dhcp-snooping interface	- 74 -

5.1.10 show dhcp-snooping vlan	- 75 -

5.1.11 show dhcp-snooping clients	- 76 -

Chapter 6 Multicast Protocol Configuration Command	- 77 -

6.1 Static Multicast Configuration Command	- 77 -

6.1.1 multicast mac-address	- 77 -

6.1.2 multicast mac-address vlan interface	- 78 -

6.1.3 show multicast	- 79 -

6.2 IGMP snooping and GMRP Configuration Command	- 79 -

6.2.1 gmrp	- 80 -

6.2.2 igmp-snooping	- 81 -

6.2.3 igmp-snooping host-aging-time	- 81 -

6.2.4 igmp-snooping max-response-time	- 82 -

6.2.5 igmp-snooping fast-leave	- 82 -

6.2.6 igmp-snooping group-limit	- 83 -

6.2.7 igmp-snooping permit/deny group	- 84 -

6.2.8 igmp-snooping route-port forward	- 84 -

6.2.9 igmp-snooping multicast vlan	- 85 -

6.2.10 show gmrp	- 86 -

6.2.11 show gmrp interface	- 86 -

6.2.12 garp permit multicast mac-address	- 87 -

6.2.13 show garp permit multicast	- 88 -

6.2.14 show igmp-snooping	- 88 -

6.3 IGMP Configuration Command	- 88 -

6.3.1 ip igmp	- 89 -

6.3.2 igmp-proxy	- 90 -

6.3.3 ip igmp access-group	- 91 -

6.3.4 ip igmp last-member-query-interval	- 92 -

6.3.5 ip igmp query-interval	- 93 -

6.3.6 ip igmp query-max-response-time	- 94 -

6.3.7 ip igmp static-group	- 95 -

6.3.8 ip igmp create-group	- 96 -

6.3.9 ip igmp robustness-varible	- 97 -

6.3.10 ip igmp limit-group	- 98 -

6.3.11 ip igmp version	- 99 -

6.3.12 ip multicast-routing	- 100 -

6.3.13 show igmp-proxy	- 101 -

6.3.14 show ip igmp groups	- 101 -

6.3.15 show ip igmp interface	- 102 -

6.3.16 ip igmp ssm-mapping	- 103 -

6.3.17 mroute igmp	- 103 -

6.3.18 ssm-mapping static	- 104 -

6.3.19 show ip igmp ssm-mapping	- 105 -

6.4 PIM Configuration Command	- 105 -

6.4.1 ip pim dense-mode	- 106 -

6.4.2 ip pim neighbor-limit	- 107 -

6.4.3 ip pim neighbor-policy	- 108 -

6.4.4 ip pim query-interval	- 109 -

6.4.5 ip pim sparse-mode	- 110 -

6.4.6 ip pim bsr-border	- 110 -

6.4.7 mroute pim	- 111 -

6.4.8 show ip mroute	- 111 -

6.4.9 show ip pim neighbor	- 112 -

6.4.10 show ip pim interface	- 113 -

6.4.11 show ip pim rp-info	- 113 -

6.4.12 show ip pim bsr	- 114 -

6.4.13 source-policy	- 114 -

6.4.14 static-rp	- 115 -

6.4.15 bsr-candidate	- 116 -

6.4.16 rp-candidate	- 117 -

6.4.17 spt-threshold	- 118 -

6.4.18 ssm	- 119 -

6.4.19 show ip pim ssm range	- 120 -

Chapter 7 ACL Configuration Command	- 122 -

7.1 ACL configuration command list	- 122 -

7.1.1 absolute	- 123 -

7.1.2 access-group	- 124 -

7.1.3 access-list	- 125 -

7.1.4 access-list extended	- 130 -

7.1.5 access-list link	- 131 -

7.1.6 access-list match-order	- 133 -

7.1.7 access-list standard	- 134 -

7.1.8 { permit | deny }	- 135 -

7.1.9 periodic	- 140 -

7.1.10 port-isolation	- 142 -

7.1.11 port-isolation group	- 143 -

7.1.12 show access-list config	- 144 -

7.1.13 show access-list config statistic	- 145 -

7.1.14 show access-list runtime	- 145 -

7.1.15 show access-list runtime statistic	- 146 -

7.1.16 show port-isolation	- 146 -

7.1.17 show time-range	- 147 -

7.1.18 time-range	- 148 -

Chapter 8 QOS Configuration Command	- 149 -

8.1 QoS Configuration Command	- 149 -

8.1.1 clear traffic-statistic	- 150 -

8.1.2 bandwidth egress	- 151 -

8.1.3 mirrored-to	- 152 -

8.1.4 queue-scheduler	- 153 -

8.1.5 queue-scheduler cos-map	- 154 -

8.1.6 queue-scheduler dscp-map	- 155 -

8.1.7 rate-limit	- 156 -

8.1.8 two-rate-policer mode	- 157 -

8.1.9 two-rate-policer set-pre-color	- 158 -

8.1.10 show qos-info all	- 159 -

8.1.11 show qos-info mirrored-to	- 159 -

8.1.12 show qos-info statistic	- 160 -

8.1.13 show qos-info traffic-copy-to-cpu	- 160 -

8.1.14 show qos-info traffic-priority	- 161 -

8.1.15 show qos-info traffic-redirect	- 161 -

8.1.16 show qos-info traffic-statistic	- 162 -

8.1.17 show qos-interface all	- 162 -

8.1.18 show qos-interface line-rate	- 163 -

8.1.19 show qos-interface rate-limit	- 164 -

8.1.20 show qos-interface statistic	- 164 -

8.1.21 show queue-scheduler	- 165 -

8.1.22 show queue-scheduler cos-map	- 165 -

8.1.23 show queue-scheduler dscp-map	- 166 -

8.1.24 show two-rate-policer	- 166 -

8.1.25 storm-control	- 166 -

8.1.26 traffic-copy-to-cpu	- 167 -

8.1.27 traffic-priority	- 168 -

8.1.28 traffic-redirect	- 170 -

8.1.29 traffic-statistic	- 171 -

Chapter 9 STP Configuration Command	- 173 -

9.1 STP Configuration Command	- 173 -

9.1.1 show spanning-tree interface	- 173 -

9.1.2 show spanning-tree remote-loop-detect interface	- 174 -

9.1.3 spanning-tree	- 175 -

9.1.4 spanning-tree cost	- 176 -

9.1.5 spanning-tree forward-time	- 176 -

9.1.6 spanning-tree hello-time	- 177 -

9.1.7 spanning-tree max-age	- 178 -

9.1.8 spanning-tree port-priority	- 179 -

9.1.9 spanning-tree mcheck	- 180 -

9.1.10 spanning-tree point-to-point	- 180 -

9.1.11 spanning-tree portfast	- 181 -

9.1.12 spanning-tree transit-limit	- 182 -

9.1.13 spanning-tree priority	- 183 -

9.1.14 spanning-tree mode	- 183 -

9.1.15 spanning-tree remote-loop-detect	- 184 -

9.1.16 clear spanning-tree	- 185 -

9.2 MSTP Cconfiguration Command	- 185 -

9.2.1 spanning-tree mst max-hops	- 187 -

9.2.2 spanning-tree mst name	- 187 -

9.2.3 spanning-tree mst revision	- 188 -

9.2.4 spanning-tree mst instance vlan	- 189 -

9.2.5 spanning-tree mst instance instance-num priority	- 189 -

9.2.6 spanning-tree mst external cost	- 190 -

9.2.7 spanning-tree mst instance cost	- 191 -

9.2.8 spanning-tree mst instance port-priority	- 191 -

9.2.9 show spanning-tree mst config-id	- 192 -

9.2.10 show spanning-tree mst instance interface	- 192 -

Chapter 10 802.1X Configuration Command	- 194 -

10.1 Domain Configuration Command	- 194 -

10.1.1 aaa	- 194 -

10.1.2 access-limit	- 195 -

10.1.3 default domain-name enbale	- 196 -

10.1.4 domain	- 197 -

10.1.5 show domain	- 198 -

10.1.6 radius host binding	- 198 -

10.1.7 state	- 199 -

10.2 RADIUS Server Configuration Command	- 200 -

10.2.1 accounting-on	- 201 -

10.2.2 acct-secret-key	- 201 -

10.2.3 auth-secret-key	- 202 -

10.2.4 dnrate-value	- 203 -

10.2.5 h3c-cams	- 203 -

10.2.6 nas-ipaddress	- 204 -

10.2.7 primary-acct-ip	- 204 -

10.2.8 primary-auth-ip	- 205 -

10.2.9 show radius attribute	- 206 -

10.2.10 show radius config-attribute	- 206 -

10.2.11 show radius host	- 207 -

10.2.12 uprate-value	- 207 -

10.2.13 radius 8021p	- 208 -

10.2.14 radius accounting	- 209 -

10.2.15 radius attribute	- 209 -

10.2.16 radius bandwidth-limit	- 210 -

10.2.17 radius config-attribute	- 211 -

10.2.18 radius host	- 211 -

10.2.19 radius mac-address-number	- 212 -

10.2.20 radius server-disconnect drop 1x	- 213 -

10.2.21 radius vlan	- 214 -

10.2.22 realtime-account	- 214 -

10.2.23 second-acct-ip	- 215 -

10.2.24 second-auth-ip	- 216 -

10.2.25 username-format	- 217 -

10.3 802.1X Related Configuration Command	- 218 -

10.3.1 dot1x method	- 218 -

10.3.2 dot1x daemon	- 219 -

10.3.3 dot1x eap-finish	- 220 -

10.3.4 dot1x eap-transfer	- 221 -

10.3.5 dot1x max-user	- 222 -

10.3.6 dot1x port-control	- 223 -

10.3.7 dot1x re-authenticate	- 224 -

10.3.8 dot1x re-authentication	- 225 -

10.3.9 dot1x timeout re-authperiod	- 226 -

10.3.10 dot1x user cut	- 227 -

10.3.11 dot1x detect	- 227 -

10.3.12 dot1x quiet-period-value	- 228 -

10.3.13 show dot1x	- 229 -

10.3.14 show dot1x daemon	- 229 -

10.3.15 show dot1x interface	- 230 -

10.3.16 show dot1x session	- 230 -

Chapter 11 SNTP Client Configuration Command	- 232 -

11.1 SNTP client configuration command list	- 232 -

11.1.1 show sntp client	- 232 -

11.1.2 sntp client	- 233 -

11.1.3 sntp client authenticate	- 233 -

11.1.4 sntp client authentication-key	- 234 -

11.1.5 sntp client broadcastdelay	- 235 -

11.1.6 sntp client mode	- 235 -

11.1.7 sntp client multicast ttl	- 236 -

11.1.8 sntp client poll-interval	- 237 -

11.1.9 sntp client retransmit	- 238 -

11.1.10 sntp client retransmit-interval	- 239 -

11.1.11 sntp client valid-server	- 240 -

11.1.12 sntp server	- 240 -

11.1.13 sntp client summer-time	- 241 -

11.1.14 sntp trusted-key	- 242 -

Chapter 12 Syslog Configiration Command	- 244 -

12.1 Syslog Configuration Command	- 244 -

12.1.1 show logging	- 245 -

12.1.2 show logging buffered	- 245 -

12.1.3 show logging flash	- 246 -

12.1.4 show logging filter	- 246 -

12.1.5 show debug	- 247 -

12.1.6 logging	- 247 -

12.1.7 logging sequence-numbers	- 248 -

12.1.8 logging timestamps	- 248 -

12.1.9 logging language	- 249 -

12.1.10 logging monitor	- 250 -

12.1.11 terminal monitor	- 251 -

12.1.12 logging buffered	- 252 -

12.1.13 clear logging buffered	- 253 -

12.1.14 logging flash	- 253 -

12.1.15 clear logging flash	- 255 -

12.1.16 logging host	- 255 -

12.1.17 logging facility	- 256 -

12.1.18 logging source	- 257 -

12.1.19 logging snmp-agent	- 258 -

12.1.20 debug	- 260 -

12.1.21 upload logging	- 261 -

Chapter 13 SSH Configuration Command	- 263 -

13.1 SSH configuration command list	- 263 -

13.1.1 show ssh	- 263 -

13.1.2 show keyfile	- 263 -

13.1.3 ssh	- 264 -

13.1.4 crypto key generate rsa	- 264 -

13.1.5 crypto key zeroize rsa	- 265 -

13.1.6 crypto key refresh	- 265 -

13.1.7 load keyfile	- 265 -

13.1.8 upload keyfile	- 266 -

Chapter 14 OLT Manage and Maintenance Command	- 269 -

14.1 Configuration Files Management	- 269 -

14.1.1 buildrun mode continue	- 269 -

14.1.2 buildrun mode stop	- 269 -

14.1.3 clear startup-config	- 270 -

14.1.4 copy nm-interface-config startup-config	- 270 -

14.1.5 copy running-config startup-config	- 271 -

14.1.6 copy startup-config running-config	- 272 -

14.1.7 show running-config	- 272 -

14.1.8 show startup-config	- 273 -

14.2 Online Loading Upgrade Program	- 273 -

14.2.1 load application ftp	- 274 -

14.2.2 load application tftp	- 275 -

14.2.3 load application xmodem	- 275 -

14.2.4 load configuration ftp	- 276 -

14.2.5 load configuration tftp	- 277 -

14.2.6 load configuration xmodem	- 277 -

14.2.7 load whole-bootrom ftp	- 278 -

14.2.8 load whole-bootrom tftp	- 279 -

14.2.9 load whole-bootrom xmodem	- 279 -

14.2.10 upload alarm ftp	- 280 -

14.2.11 upload alarm tftp	- 281 -

14.2.12 upload configuration ftp	- 281 -

14.2.13 upload configuration tftp	- 282 -

14.2.14 upload logging ftp	- 283 -

14.2.15 upload logging tftp	- 283 -

14.3 Reboot OLT	- 284 -

14.3.1 reboot	- 284 -

14.4 Basic Configuration and Maintenance	- 284 -

14.4.1 broadcast-suppression	- 285 -

14.4.2 clock set	- 286 -

14.4.3 clock timezone	- 287 -

14.4.4 discard-bpdu	- 288 -

14.4.5 discard-l2-tunnel	- 289 -

14.4.6 dlf-forward	- 289 -

14.4.7 loopback	- 290 -

14.4.8 vct run	- 291 -

14.4.9 vct auto-run	- 291 -

14.4.10 show vct auto-run	- 292 -

14.4.11 mac-address-table	- 292 -

14.4.12 mac-address-table age-time	- 293 -

14.4.13 mac-address-table learning	- 294 -

14.4.14 mac-address-table max-mac-count	- 294 -

14.4.15 ping	- 295 -

14.4.16 show broadcast-suppression	- 296 -

14.4.17 show clock	- 296 -

14.4.18 show cpu-utilization	- 297 -

14.4.19 show dhcp-server clients	- 297 -

14.4.20 show discard-bpdu	- 298 -

14.4.21 show dlf-forward	- 298 -

14.4.22 show ip fdb	- 298 -

14.4.23 show mac-address-table	- 299 -

14.4.24 show mac-address-table age-time	- 300 -

14.4.25 show mac-address-table learning	- 300 -

14.4.26 show memory	- 301 -

14.4.27 show system	- 301 -

14.4.28 show users	- 301 -

14.4.29 show version	- 302 -

14.4.30 login-access-list telnet-limit	- 302 -

14.4.31 tracert	- 303 -

14.4.32 cpu-car	- 304 -

14.4.33 show cpu-car	- 305 -

14.4.34 show cpu- statistics	- 305 -

14.4.35 clear cpu- statistics	- 305 -

14.5 SNMP Configuration	- 306 -

14.5.1 show snmp community	- 307 -

14.5.2 show snmp contact	- 307 -

14.5.3 show snmp host	- 308 -

14.5.4 show snmp notify	- 308 -

14.5.5 show snmp location	- 308 -

14.5.6 show snmp engineID	- 309 -

14.5.7 show snmp group	- 309 -

14.5.8 show snmp user	- 310 -

14.5.9 show snmp view	- 310 -

14.5.10 snmp-server community	- 311 -

14.5.11 snmp-server contact	- 312 -

14.5.12 snmp-server host	- 312 -

14.5.13 snmp-server location	- 313 -

14.5.14 snmp-server name	- 314 -

14.5.15 snmp-server enable traps	- 315 -

14.5.16 snmp-server trap-source	- 316 -

14.5.17 snmp-server engineID	- 316 -

14.5.18 snmp-server view	- 318 -

14.5.19 snmp-server group	- 319 -

14.5.20 snmp-server user	- 320 -

14.6 Manage IP Restriction Configuration	- 322 -

14.6.1 login-access-list	- 322 -

14.6.2 show login-access-list	- 323 -

14.7 Telnet Client	- 323 -

14.7.1 telnet	- 324 -

14.7.2 show telnet client	- 324 -

14.7.3 stop telnet client	- 325 -

14.8 CPU Alarm Configuration Command	- 325 -

14.8.1 alarm cpu	- 326 -

14.8.2 alarm cpu threshold	- 326 -

14.8.3 show alarm cpu	- 327 -

14.9 Mail Alarm Configuration	- 327 -

14.9.1 mailalarm	- 328 -

14.9.2 mailalarm server	- 328 -

14.9.3 mailalarm receiver	- 329 -

14.9.4 mailalarm ccaddr	- 329 -

14.9.5 mailalarm smtp authentication	- 330 -

14.9.6 mailalarm logging level	- 331 -

14.9.7 show mailalarm	- 332 -

14.10 Anti-DOS Attack	- 332 -

14.10.1 anti-dos ip fragment	- 333 -

14.10.2 anti-dos ip ttl	- 333 -

14.10.3 show anti-dos	- 334 -

Chapter 15 LLDP Configuration Command	- 335 -

15.1 LLDP Configuration Command	- 335 -

15.1.1 lldp	- 335 -

15.1.2 lldp hello-time	- 336 -

15.1.3 lldp hold-time	- 336 -

15.1.4 lldp { rx | tx | rxtx }	- 337 -

15.1.5 show lldp interface [ <interface-list> ]	- 337 -

Chapter 16 Flex links Configuration Command	- 338 -

16.1 Flex links Configuration Command	- 338 -

16.1.1 OLTport backup	- 338 -

16.1.2 channel-group channel-group-number backup	- 338 -

16.1.3 OLTport backup preemption mode	- 339 -

16.1.4 channel group backup preemption mode	- 339 -

16.1.5 OLTport backup preemption delay	- 340 -

16.1.6 channel-group backup preemption delay	- 341 -

16.1.7 show swithport interface backup	- 341 -

16.1.8 mac-address-table move update transmit	- 342 -

16.1.9 mac-address-table move update receive	- 342 -

16.1.10 show swithport interface backup	- 342 -

Chapter 17 CFM Configuration Command	- 344 -

17.1.1 cfm md	- 344 -

17.1.2 cfm mep	- 345 -

17.1.3 cfm mip	- 346 -

17.1.4 cfm rmep	- 346 -

17.1.5 cfm cc interval	- 347 -

17.1.6 cfm loopback	- 348 -

17.1.7 cfm linktrack	- 348 -

17.1.8 show cfm md	- 349 -

17.1.9 show cfm mp local	- 349 -

17.1.10 show cfm mp remote	- 349 -

17.1.11 show cfm cc database	- 350 -

17.1.12 show cfm errors	- 350 -

Chapter 18 PPPoE PlusConfiguration Command	- 351 -

18.1 PPPoE PlusConfiguration Command	- 351 -

18.1.1 pppoeplus	- 351 -

18.1.2 pppoeplus type	- 351 -

18.1.3 show pppoeplus	- 352 -

Chapter 19 ERRP Configuration Command	- 353 -

19.1 ERRP Configuration Command	- 353 -

19.1.1 errp	- 353 -

19.1.2 errp hello-timer	- 354 -

19.1.3 errp fail-timer	- 354 -

19.1.4 errp domain	- 355 -

19.1.5 control-vlan	- 355 -

19.1.6 ring role primary-port secondary-port level	- 356 -

19.1.7 ring role common-port edge-port	- 357 -

19.1.8 ring { enable | disable }	- 358 -

19.1.9 show errp	- 359 -

19.1.10 ring query-solicit	- 359 -

Chapter 20 EFM Configuration Command	- 361 -

20.1 EFM Configuration Command	- 361 -

20.1.1 efm	- 361 -

20.1.2 efm mode	- 362 -

20.1.3 efm pdu-timeout	- 363 -

20.1.4 efm link-timeout	- 363 -

20.1.5 efm remote-response-timeout	- 364 -

20.1.6 efm link-monitor para	- 365 -

20.1.7 efm remote-failure	- 366 -

20.1.8 efm link-monitor	- 366 -

20.1.9 efm variable-retrieval	- 367 -

20.1.10 efm remote-loopback	- 368 -

20.1.11 efm remote-loopback { start | stop }	- 368 -

20.1.12 efm remote-loopback { ignore | process }	- 369 -

20.1.13 show efm status interface	- 369 -

20.1.14 show efm summary	- 370 -

20.1.15 show efm discovery interface	- 370 -

20.1.16 show / clear efm statistics interface	- 371 -

20.1.17 show efm remote-mib	- 371 -

Chapter 21 OLT Slot Management Configuration Command	- 373 -

21.1 OLT Slot Management Configuration Command	- 373 -

21.1.1 set slot	- 373 -

21.1.2 show slot	- 374 -

21.1.3 show pon	- 374 -

Chapter 22 PON Configuration Command	- 376 -

22.1 PON Configuration Command	- 376 -

22.1.1 onu-authenticate	- 377 -

22.1.2 white-list	- 377 -

22.1.3 black-list	- 378 -

22.1.4 loid-list	- 378 -

22.1.5 hybrid-list	- 379 -

22.1.6 onu-p2p	- 379 -

22.1.7 encryp	- 380 -

22.1.8 dba	- 381 -

22.1.9 mac-address-table	- 381 -

22.1.10 classif	- 382 -

22.1.11 no classif	- 387 -

22.1.12 enable-pon-vlan-isolation	- 387 -

22.1.13 show onu-mac-auth	- 388 -

22.1.14 show white-list	- 389 -

22.1.15 show black-list	- 389 -

22.1.16 show loid-list	- 390 -

22.1.17 show hybrid-list	- 390 -

22.1.18 show onu-p2p	- 391 -

22.1.19 show dba	- 391 -

22.1.20 show mac-address-table	- 392 -

22.1.21 show classif	- 392 -

22.1.22 show enable-pon-vlan-isolation	- 393 -

Chapter 23 ONU Management Configuration Commands	- 394 -

23.1 ONU Management Configuration Commands	- 394 -

23.1.1 onu-description	- 395 -

23.1.2 show onu-description	- 395 -

23.1.3 onu-binding	- 396 -

23.1.4 show onu-status	- 397 -

23.1.5 onu-reboot	- 397 -

23.1.6 onu-bandwidth	- 398 -

23.1.7 onu-encrypt	- 400 -

23.1.8 onu-loopback	- 400 -

23.1.9 onu-flow-control	- 401 -

23.1.10 onu-shutdown	- 401 -

23.1.11 onu-speed auto	- 402 -

23.1.12 onu-bandwidth	- 402 -

23.1.13 onu-bandwidth multicast	- 403 -

23.1.14 onu-bandwidth broadcast	- 403 -

23.1.15 onu-vlan-mode	- 403 -

23.1.16 onu-classification	- 405 -

23.1.17 onu-multicast mode	- 406 -

23.1.18 onu-multicast tag	- 406 -

23.1.19 onu-multicast fastleave	- 406 -

23.1.20 onu-igmp-snooping vlan	- 407 -

23.1.21 onu-igmp-snooping group-number	- 407 -

23.1.22 onu-multicast-ctrl	- 408 -

23.1.23 onu-fec mode	- 408 -

23.1.24 no onu-classification	- 409 -

23.1.25 no onu-igmp-snooping vlan	- 409 -

23.1.26 no onu-multicast-ctrl	- 410 -

23.1.27 no onu-bandwidth multicast	- 410 -

23.1.28 no onu-bandwidth broadcast	- 410 -

23.1.29 onu-mac-address-table max-mac-count	- 411 -

23.1.30 onu-mac-address-table age-time	- 411 -

23.1.31 onu-queue-scheduler	- 412 -

23.1.32 onu-queue-scheduler cos-map	- 413 -

23.1.33 onu-queue-scheduler cos-remap	- 414 -

23.1.34 onu-mac-address-table blackhole	- 414 -

23.1.35 onu-dtag	- 415 -

23.1.36 onu-ip address static	- 417 -

23.1.37 no onu-ip address	- 417 -

23.1.38 onu-vlan	- 418 -

23.1.39 no onu-vlan	- 418 -

23.1.40 onu-swithport	- 419 -

23.1.41 no onu-swithport	- 419 -

23.1.42 onu-swithport access vlan	- 419 -

23.1.43 no onu-swithport access vlan	- 420 -

23.1.44 onu-tag-mode	- 420 -

23.1.45 onu-com-session	- 420 -

23.1.46 no onu-com-session	- 422 -

23.1.47 clear onu-com-statistic	- 422 -

23.1.48 onu-event-alarm loopback	- 422 -

23.1.49 onu-ctc-upgrade	- 423 -

23.1.50 onu-ctc-upgrade-commit	- 423 -

23.1.51 show onu-port-info	- 423 -

23.1.52 show onu-status	- 424 -

23.1.53 show statistics onu	- 424 -

23.1.54 show onu-bandwidth	- 425 -

23.1.55 show onu-encrypt	- 426 -

23.1.56 show onu-loopback oam	- 427 -

23.1.57 show onu-interface	- 427 -

23.1.58 show onu-sn	- 428 -

23.1.59 show onu-firmware	- 428 -

23.1.60 show onu-pon-chip	- 429 -

23.1.61 show onu-capabilities	- 429 -

23.1.62 show onu-bandwidth	- 430 -

23.1.63 show vlan-mode	- 432 -

23.1.64 show onu-classification	- 433 -

23.1.65 show onu-multicast mode	- 433 -

23.1.66 show onu-multicast tag	- 434 -

23.1.67 show onu-multicast fast-leave	- 434 -

23.1.68 show onu-igmp-snooping vlan	- 435 -

23.1.69 show onu-multicast-ctrl local-ctrl	- 435 -

23.1.70 show onu-multicast-ctrl	- 436 -

23.1.71 show onu-fec	- 436 -

23.1.72 show onu-mac-address-table max-mac-count	- 437 -

23.1.73 show onu-mac-address-table age-time	- 438 -

23.1.74 show onu-queue-scheduler	- 438 -

23.1.75 show onu-mac-address-table blackhole	- 439 -

23.1.76 show onu-bandwidth multicast	- 439 -

23.1.77 show onu-bandwidth broadcast	- 440 -

23.1.78 show onu-dtag	- 440 -

23.1.79 show onu-mac-address-table blackhole	- 442 -

23.1.80 show onu-ip address onu	- 442 -

23.1.81 show onu-com-session	- 442 -

23.1.82 show onu-com-statistic	- 443 -

23.1.83 show onu-event-alarm loopback	- 444 -

Chapter 24 PSG Management Configuration Command	- 445 -

24.1 PSG Management Configuration Command	- 445 -

24.1.1 mpcp-delay-time	- 445 -

24.1.2 admin-enable-pon	- 446 -

24.1.3 psg creat	- 447 -

24.1.4 psg delete	- 447 -

24.1.5 psg OLT	- 448 -

24.1.6 admin-enable-psg	- 448 -

24.1.7 show mpcp-delay-time	- 449 -

24.1.8 show admin-enable-pon	- 449 -

24.1.9 show psg	- 450 -

24.1.10 show admin-enable-psg	- 451 -

Chapter 25 Controllable Multicast Profile Managmenet Commands	- 452 -

25.1 Controllable multicast profile management commands	- 452 -

25.1.1 multicast-ctrl profile	- 452 -

25.1.2 multicast-ctrl	- 453 -

25.1.3 show multicast-ctrl profile	- 453 -

25.1.4 enable multicast-ctrl profile	- 454 -

25.1.5 onu-multicast-ctrl profile-binding	- 454 -

# OLT Logging in Command

  - OLT Logging in Command

OLT logging in command includes:

- cls

- configure terminal

- enable

- end

- exit

- help

- hostname

- interface

- muser

- quit

- show muser

- show tacacs+

- show username

- stop

- tcacac+

- timeout

- username

- username change-password

    - cls

Use cls command to clear current screen displaying

cls

【Command configuration mode】

Any configuration mode

【Example】

！Clear current screen displaying

Optiway>cls

    - configure terminal

Use configure terminal command to enter global configuration mode from privileged mode.

configure terminal

【Command configuration mode】

Privileged mode

【Example】

Optiway#configure terminal

Optiway(config)#

【Related command】

exit,end

    - enable

Use enable command to enter privileged mode from user mode.

enable

【Command configuration mode】

User mode

【Example】

！Enter from user mode to privileged mode

Optiway>enable

Optiway#

【Related command】

exit,end

    - end

Use end command to be back from global configuration mode or other superior mode to privileged mode.

end

【Command configuration mode】

Any configuration mode except user mode and privileged mode

【Usage】

5 levels of command line configuration mode, from inferior to superior are:

- User mode

- Privileged mode

- Global configuration mode

- Interface configuration mode, VLAN configuration mode, and                  AAA configuration mode

- Domain configuration mode and radius configuration mode

End command can back from global configuration mode or other superior mode to privileged mode.

【Example】

！Back from global configuration mode to privileged mode

Optiway(config-if-ethernet-2/1)#end

Optiway#

【Related command】

exit

    - exit

Use exit command to be back to inferior mode. For the user mode, exit.

exit

【Command configuration mode】

Any configuration mode

【Usage】

Use exit command can be back to inferior mode

【Example】

！Back to global configuration mode from interface configuration mode

Optiway(config-if-ethernet-2/1)#exit

Optiway(config)#

【Related command】

end

    - help

Use help command to display command help information.

help

【Command configuration mode】

Any configuration mode

【Usage】

Use help command can display any command in current mode, and user can key in “?” at any moment.

【Example】

Optiway(config)#help

    - hostname

Use hostname command to configure host name. Use no hostname command to restore default host name.

hostname hostname

no hostname

【Parameter】

hostname:character strings range from 1 to 32, these strings can be printable, excluding such wildcards as '/',':','*','?','\\','<','>','|','"'etc.

【Default】

Default hostname is OptiWay

【Command configuration mode】

Global configuration mode

【Usage】

Modify system hostname. If the hostname is OptiWay,the hostname in global configuration mode is OptiWay(config)#.

【Example】

！Configure hostname to be OptiWayEL8600-04

Optiway(config)#hostname OptiwayEL8600-04

OptiwayEL8600-04(config)#

    - interface

Use interface command to enter interface configuration mode.

interface ethernet interface-num

【Parameter】

interface-num:The number of the interface

【Command configuration mode】

Global configuration mode

【Usage】

Interface-number is in the form of slot-num/port-num, in which slot-num is in the range of 0 to 2, and port-num is in the range of 1 to 24.

【Example】

！Enter from clobal configuration mode to interface configuration mode

Optiway(config)#interface ethernet 2/1

    - muser

Use muser command to enable user’s RADIUS/TACACS+ remote authentication.

muser { local | { radius radiusname { pap | chap } [ local ] } }| { tacacs+ [ author ] [ account ] [ local ] } }

【Parameter】

radiusname:RADIUS server configuration name

【Command configuration mode】

Global configuration mode

【Usage】

Configure authentication of RADIUS/TACACS+ remote authentication only or using RADIUS/TACACS+ remote authentication first, if RADIUS/TACACS+ fails, local database authentication is used.

RADIUS authentication supports PAP or CHAP ways.

Enable RADIUS/TACACS+ remote authentication needs correct RADIUS/TACACS+ server configuration.

If it is TACACS+ remote authentication, when the authorization is not used, the privilege after authentication is administrator; when the authorization is used, the privilege after authentication is determined by the replied priv_lvl from remote server, if there is no reply, it is administrator; if the authorization fails, it is normal user.

The accounting of TACACS+ is from the beginning to the end.

【Example】

！Enable RADIUS authentication with the way of PAP

Optiway(config)#muser radius radiusserver1 pap

！Enable TACACS+ authentication

Optiway(config)#muser tacacs+ author

    - quit

Use quit command to disconnect with OLT and exit.

quit

【Command configuration mode】

Any configuration mode

【Usage】

If the current connect is in telnet, use quit command to disconnect with the OLT and exit. If the current connect is in serial port, after using quit command, you will re-log in.

【Example】

！Disconnect with the OLT and exit

Optiway#quit

    - show muser

Use show muser command to display user’s authentication.

show muser

【Command configuration mode】

Any configuration mode

【Example】

！Display user’s authentication

Optiway(config)#show muser

    - show tacacs+

Use this command to show TACACS+ server configuration.

show tacacs+

【Command configuration mode】

Any configuration mode

【Example】

！show TACACS+ server configuration

Optiway(config)#show tacacs+

    - show username

Use show username command to display all the users or the user’s privilege or the existed user and his privilege.

show username [ username ]

【Parameter】

username:existed  username ranges from 1 to 32 printable characters such wildcards as '/',':','*','?','\\','<','>','|','"'.

【Command configuration mode】

Any configuration mode

【Example】

！Display the privilege of user “green”

Optiway(config)#show username green

    - stop

Use stop command to stop the session between user and telnet forcibly, that is, after using this command, telnet user with the username of “username” will force to disconnect with telnet.

stop username

【Parameter】

username:Telnet user who has logged in

【Command configuration mode】

Privileged mode

【Usage】

Only administrator can use this command

【Example】

！Force user “green” to disconnect with telnet

Optiway#stop green

    - tacacs+

use this command to configure TACACS+ server.

tacacs+ { priamary | secondary } server ipaddress [key keyvalue] [port portnum] [timeout timevalue]

no tacacs+ { priamary | secondary } server

【Parameter】

ipaddress:ip address of primary and secondary server which cannot be the same.

keyvalue:the key between OLT and server, which is in the length of 1～16 characters.

portnum:TACACS+ is tcp and this value is defaulted to be 49, which is in the range of 1～65535

timevalue:the timeout of tcp connection, which is defaulted to be 5 seconds and in the range of 1～120 seconds.

【Command configuration mode】

Global configuration mode

【Usage】

Only administrators can use this command.

【Example】

！Configure primary server

Optiway#tacacs+ priamary server key 123

    - timeout

Use timeout command to configure the overtime of user’s logging in. Use no timeout command to configure overtime to be non-over timing.

timeout [ minute ]

no timeout

【Parameter】

minute:Range from 1 to 480 minutes

【Default】

Default time is 20 minutes

【Command configuration mode】

User mode, privileged mode

【Usage】

If timeout command without parameter, it configures to be default time. No timeout command means non-overtime. Use no timeout command in telnet, if the user doesn’t exit and the net is smooth, telnet user is non-overtime; if the net is disconnected, the link to telnet will be disconnected in 2 hours.

This command is effective for command line users.

【Example】

！Configure the overtime to be 30 minutes

Optiway#timeout 30

！Configure user to be non-overtime

Optiway#no timeout

    - username

Use username username privilege command to add a user or modify the privilege or password of the existed user. Use no username username privilege command to remove specified user.

username username [ privilege level ] { password encryption-type password }

no username username

【Parameter】

username:User name of new users and existed users ranges from 1 to 32 printable characters excluding such wildcards as '/',':','*','?','\\','<','>','|','"' etc.

privilege:Privilege of new user or the modified privilege of existed user ranges from 0 to 15. 0 to 1 means user while 2 to 15 means administrator. Caution: the privilege of administrator cannot be modified.

encryption-type:  the value of it is 0 or 7. 0 means non-encryption and 7 means encryption( It is not supported now).

password:Log in password for new user and modified password of the existed user ranges from 1 to 16 characters or numbers.

【Command configuration mode】

Global configuration mode

【Usage】

When inputting the privilege of the new user, 0 to 1 means ordinary user and 2 to 15 means administrator. If the privilege doesn’t configure, the default privilege is ordinary user.

If inputting nothing to modify the privilege of existed user, the privilege doesn’t modify. The privilege of Admin cannot be modified.

【Example】

！ Add a new administrator “green”, configure privilege to be 15,and password to be 123456

Optiway(config)#username green privilege 15 password 0 123456

！Modify the privilege of administrator “green” to be 1,and password to be 1234

Optiway(config)#username green privilege 1 password 0 1234

    - username change-password

Administrator “admin” can use username change-password to modify the password of him and others, and other users can use this command to modify his own password. After inputting this command, user will be asked to input as following: original password, the username of the password needs modifying, new password and confirm new password.

username change-password

【Parameter】

Username must be existed.

【Command configuration mode】

Global configuration mode

【Usage】

Only administrator “admin”can modify other user’s password, while others only can modifies his own. If a user forgets his password, administrator “admin” can use this command to give him a new one.

【Example】

！Modify the password of user “green” to be 123456

Optiway(config)#username change-password

please input you login password : ******

please input username :green

Please input user new password :******

Please input user comfirm password :******

chang user green password success.

# Port Configuration Command

  - Ethernet Interface Configuration Command

Ethernet interface configuration command includes:

- clear interface

- combo

- description

- duplex

- flow-control

- ingress acceptable-frame

- ingress filtering

- link-aggregation

- priority

- show description

- show interface

- show statistics interface

- shutdown

- speed

- OLTport access

- OLTport mode

- OLTport trunk allowed vlan

- OLTport trunk

- tag

- show statistics dynamic interface

- show utilization interface

    - clear interface

Use clear interface command to clear the information of the interface.

clear interface [ interface-num | slot-num ]

【Parameter】

interface-num:Means Ethernet port. Interface-num is in the form of interface-type + interface-number. Interface-type is Ethernet and interface-number is slot-num/port-num, in which slot-num is in the range of 0 to 2, and port-num is in the range of 1 to 24.

slot-num:Means slot number which is in the form of ethernet + slot-num, and ranges from 0 to 2

【Command configuration mode】

Global configuration mode, interface configuration mode

【Usage】

The information of the interface includes: numbers of unicast, multicast and broadcast message etc.

Using clear interface command in global mode, if the interface-num and slot-num are not assigned, the information of all interfaces is cleared. If the slot-num is assigned, the port information of the assigned slot is cleared. In interface mode, only the information of the current port can be cleared.

【Example】

！Clear information of all interfaces

Optiway(config)#clear interface

！Clear information of interface 5 in global and interface mode

Optiway(config)#clear interface ethernet 2/5

Optiway(config-if-ethernet-2/5)#clear interface

    - combo

Use this command to configure combo attribute of Ethernet interface.

combo { fiber | copper }

【Parameter】

fiber:FX attribution

copper:TX arrtibution

【Default】

fiber

【Command configuration mode】

Interface configuration mode

【Usage】

Only Ethernet interface 1～4 are combo interfaces. If combo interface is configured to be TX mode, FX cannot be used. If combo interface is configured as FX, TX cannot be used.

【Example】

！Configure combo attribution of current Ethernet interface １to be TX

Optiway(config-if-ethernet-2/１)#combo copper

    - description

Use description command to configure a port description string. Use no description command to remove the port description string.

description description-list

no description

【Parameter】

description-list:Port description string ranges from 1 to 32 characters

【Command configuration mode】

Interface configuration mode

【Example】

！Configure description string “green” for the Ethernet 2/3

Optiway(config-if-ethernet-2/3)#description green

！Clear description of Ethernet 2/3

Optiway(config-if-ethernet-2/3)#no description

【Related command】

show description

    - duplex

Use duplex command to configure the duplex mode of the current port. Use no duplex command to restore the default duplex mode, that is, auto-negotiation.

duplex { half | full | auto }

no duplex

【Parameter】

half:Half duplex mode

full:Full duplex mode

auto:Auto-negotiation mode

【Default】

auto

【Command configuration mode】

Interface configuration mode

【Usage】

When configuring duplex mode, full duplex means receiving and sending messages at the same time; half duplex means receiving or sending message at one time, and auto means the duplex mode negotiating by each port.

100 BASE-FX only supports full duplex.

【Example】

！Configure ethernet 0/5 port to full duplex

Optiway(config-if-ethernet-2/5)#duplex full

    - flow-control

Use flow-control command to enable flow control on the Ethernet port. Use no flow-control command to disable flow control on the port.

flow-control

no flow-control

【Default】

Disable

【Command configuration mode】

Interface configuration mode

【Usage】

If the port is crowded, it needs controlling to avoid congestion and data loss. Use flow-control command to control the flow.

【Example】

！Enable flow control on Ethernet 2/5

Optiway(config-if-ethernet-2/5)#flow-control

！Disable flow control on Ethernet 2/5

Optiway(config-if-ethernet-2/5)#no flow-control

    - ingress acceptable-frame

Use ingress acceptable-frame command to configure ingress acceptable frame mode. Use no ingress acceptable-frame command to restore the default ingress acceptable frame.

ingress acceptable-frame { all | tagged }

no ingress acceptable-frame

【Default】

All types of frame is acceptable

【Command configuration mode】

Interface configuration mode

【Usage】

When ingress acceptable-frame enables, frame of other type are dropped. When ingress acceptable-frame disables, all types of frames are received.

【Example】

！Configure Ethernet 2/5 only to receive tagged frame

Optiway(config-if-ethernet-2/5)#ingress acceptable-frame tagged

！Restore default ingress acceptable-frame Ethernet 2/5

Optiway(config-if-ethernet-2/5)#no ingress accetable-frame

    - ingress filtering

Use ingress filtering command to enable interface ingress filtering. Use no ingress filtering command to disable interface ingress filtering.

ingress filtering

no ingress filtering

【Default】

Ingress filtering enables.

【Command configuration mode】

Interface configuration mode

【Usage】

When interface ingress filtering enables, the frame with the VLAN ID being different from the VLAN ID of the interface which the frame is received will be dropped; when interface ingress filtering disables, the frame will not be dropped.

【Example】

！Enable the ingress filtering of ethernet 2/5

Optiway(config-if-ethernet-2/5)#ingress filtering

！Disable the ingress filtering of ethernet 2/5

Optiway(config-if-ethernet-2/5)#no ingress filtering

    - priority

Use priority command to assign priority of the port. Use no priority command to restore default priority.

priority priority-value

no priority

【Parameter】

priority-value:Ranges from 2 to 7

【Default】

Default priority-value is 2

【Command configuration mode】

Interface configuration mode

【Usage】

The larger priority-value is, the higher the priority is.

【Example】

！Configure priority-value of Ethernet 2/3 to be 1

Optiway(config-if-ethernet-2/3)#priority 1

    - show description

Use show description command to display interface description.

show description interface [ interface-list ]

【Parameter】

interface-list:List of interfaces means many Ethernet ports

【Command configuration mode】

Any configuration mode

【Usage】

When displaying interface description, if interface-list is not specified, description of all interfaces is displayed. If interface is specified, the description of the specified interface is displayed.

【Example】

！Display description of Ethernet 2/3

Optiway(config)#show description interface ethernet 2/3

【Related command】

description

    - show interface

Use show interface command to display port configuration.

show interface [ interface-num ]

【Parameter】

interface-num:Means Ethernet port. Interface-num is in the form of interface-type + interface-number. Interface-type is Ethernet and interface-number is slot-num/port-num, in which slot-num is in the range of 2 to 2, and port-num is in the range of 1 to 24.

【Command configuration mode】

Any configuration mode

【Usage】

If port type and port number are not specified, the command displays information about all ports. If both port type and port number are specified, the command displays information about the specified port.

【Example】

！Display the configuration information of Ethernet 2/1

Optiway#show interface ethernet 2/1

    - show statistics interface

Use show statistics interface command to display the statistic information of specified port or all ports.

show statistics interface [ interface-num ]

【Parameter】

interface-num:Means Ethernet port. Interface-num is in the form of interface-type + interface-number. Interface-type is Ethernet and interface-number is slot-num/port-num, in which slot-num is in the range of 2 to 2, and port-num is in the range of 1 to 24.

【Command configuration mode】

Any mode

【Usage】

If port type and port number are not specified, the command displays statistic information about all ports. If both port type and port number are specified, the command displays statistic information about the specified port.

【Example】

！Display statustic information of Ethernet 2/1

Optiway#show statistics interface ethernet 2/1

    - show statistics higig

Use show statistics higig [unit unit higig-port higig-port] command in any mode to show port info.

Use show statistics dynamic higig to show dynamic statictics.

show statistics higig [ unit unit higig-port higig-port]

show statistics dynamic higig

【Parameter】

dynamic:dynamic statistics

unit:chip unit

higig-port:port number

【Command configuration mode】

Any mode

【Usage】

If no unit and higig port is specified, show all.

【Example】

！Show higig-statistics unit 2 higig-port1

Optiway(diag)%% show statistics higig unit 2 higig-port 1

    - clear higig-statistics

Use clear statistics [all | unit unit higig-port higig-port] command to clear info.

clear higig-statistics [all | unit unit higig-port higig-port]

【Parameter】

all: all ports

unit:chip unit

higig-port:port number

【Command configuration mode】

Global configuration mode

【Usage】

Use this command in global configuration mode, if no higig port is specified, clear all.

【Example】

！Clear higig-statistics unit 2 higig-port1

Optiway(diag)%% clear higig-statistics unit 2 higig-port 1

    - shutdown

Use shutdown command to disable an Ethernet port. Use no shutdown command to enable an Ethernet port.

shutdown

no shutdown

【Default】

Ethernet port enables

【Command configuration mode】

Interface configuration mode

【Usage】

Use no shutdown command to enable an Ethernet port after related parameter and protocol are configured. Disable a port and then enable it when there is a failure, which can recover the port.

【Example】

！Disable Ethernet 2/1, then enable it.

Optiway(config-if-ethernet-2/1)#shutdown

Optiway(config-if-ethernet-2/1)#no shutdown

    - port-control mode

Use this command to configure port-control mode. Use no command to restore to slave mode.

port-control mode master | slave

no port-control mode

【Default】

slave

【Command configuration mode】

Interface configuration mode

【Usage】

When two extended GE electric ports connect to each other and speed is in force, and the two sides are master and slave, the connection is successful.

【Example】

！Set e2/2 to be master and restore to be slave

Optiway(config-if-ethernet-2/2)# port-control mode master

Optiway(config-if-ethernet-2/2)# no port-control mode

    - show port-control mode

Use this command to show port-control mode.

show port-control mode

【Command configuration mode】

Any configuration mode

【Usage】

Only extended GE electric port needs this configuration.

【Example】

！Show port-control mode

Optiway# show port-control mode

    - speed

Use speed command to configure the port speed. Use no speed command to restore the port speed to the defaulting setting.

speed { 12 | 12auto | 122 | 122auto | auto }

no speed

【Parameter】

12:Means the port speed is 12Mbps

122:Means the port speed is 122Mbps

12auto: means the maximum port speed is 12Mbps,and duplex mode is auto-negotiation

122auto: means the maximum port speed is 122Mbps,and duplex mode is auto-negotiation

auto: means both port speed and duplex mode are auto-negotiation

【Default】

auto

【Command configuration mode】

Interface configuration mode

【Usage】

122 BASE TX supports the speed of 12Mbps and 122Mbps and the duplex mode of half, full duplex and auto-negotiation mode. 122 BASE FX supports the speed of 122Mbps and the duplex mode of full duplex.

【Example】

！Configure the speed of Ethernet 2/1 to 122Mbps

Optiway(config-if-ethernet-2/1)#speed 122

    - OLTport mode

Use OLTport mode command to configure port type. Use no OLTport mode command to restore default port type, that is, access port.

OLTport mode { access | trunk }

no OLTport mode

【Parameter】

access:Configure port to be non-trunk port.

trunk:Configure port to be trunk port.

【Default】

Default port mode is access port.

【Command configuration mode】

Interface configuration mode

【Usage】

Use OLTport mode command to configure a port to be trunk port or access port. If a port configures to be a trunk port, the vlan mode changes untagged into tagged, and if a port configures to be an access one, the vlan mode changes tagged into untagged. In addition, configure a port to be a trunk one, then create a vlan, this port will automatically be added to the vlan.

【Example】

！Configure Ethernet 2/1 to be trunk port

Optiway(config-if-ethernet-2/1)#OLTport mode trunk

    - OLTport trunk allowed vlan

Use OLTport trunk allowed vlan command to add trunk port to specified VLAN. Use no OLTport trunk allowed vlan command to remove trunk port from specified vlan.

OLTport trunk allowed vlan { vlan-list | all }

no OLTport trunk allowed vlan { vlan-list | all }

【Parameter】

vlan-list: vlan-listvlan-list can be discrete numbers, sequential numbers or both. Discrete numbers are separated by “,”, and sequential numbers use “-”, such as: 2, 5,8,12-22. Vlan-list in the following context expresses the same.

all:Add trunk ports to all VLAN.

【Command configuration mode】

Interface configuration mode

【Usage】

Use this command to add trunk port to specified VLAN. Trunk port can belong to more VLANs. If use OLTport trunk allowed vlan command in many times, VLAN allowed by the trunk port is the congregation of these vlan-list.

【Example】

！Add trunk port Ethernet2/1 to VLAN 3,4,72～152

Optiway(config-if-ethernet-2/1)#OLTport trunk allowed vlan 3,4,72-152

    - OLTport trunk default vlan

Use this command to configure port default vlan-id (pvid),with no command is to revert the default vlan-id.

OLTport default vlan vlan-id

no OLTport default vlan

【Parameter】

vlan-id range is 1～4094

【Default】

vlan-id default value is 1

【Command mode】

Interface configuration mode

【Example】

！Configure port 1 the default VLAN id is 100

OptiWay(config-if-ethernet-2/1)#OLTport default vlan 100

    - show statistics dynamic interface

Use show statistic dynamic interface command to display the statistic information of all interfaces.

show statistics dynamic interface

【Command configuration mode】

Any configuration mode

【Usage】

Statistic information refreshes automatically every 3 seconds.

【Example】

！Display statistic information of the port

Optiway#show statistics dynamic interface

    - show utilization interface

Use show utilization interface command to display the utilization information of all ports, including receiving and sending speed, bandwidth utilization rate, etc.

show utilization interface

【Command configuration mode】

Any configuration mode

【Usage】

Receiving and sending rate and bandwidth utilization rate refresh every 3 seconds.

【Example】

！Display utilization interface of the port

Optiway#show utilization interface

    - local-OLT

Use this command to enable local OLTing feature of Ethernet interface. Use the no command to disable it.

local-OLT

no local-OLT

【Default】

Disable

【Command configuration mode】

Interface configuration mode

【Usage】

Enabling local OLTing feature of Ethernet interface can transmit the packet from the port where it is in.

It is useful when a interface downlinking a WAP.

【Example】

！Enable local OLTing feature of e2/5

Optiway(config-if-ethernet-2/5)#local-OLT

！Disable local OLTing feature of e2/5

Optiway(config-if-ethernet-2/5)#no local-OLT

  - Interface Mirror Configuration Command

Interface Mirror configuration command includes:

- mirror destination-interface

- mirror source-interface

- show mirror

    - mirror destination-interface

Use mirror destination-interface command configure mirror destination interface. Use no mirror destination-interface command to remove mirror interface.

mirror destination-interface interface-num

no mirror destination-interface interface-num

【Parameter】

interface-num:Means Ethernet port. Interface-num is in the form of interface-type + interface-number. Interface-type is Ethernet and interface-number is slot-num/port-num, in which slot-num is in the range of 2 to 2, and port-num is in the range of 1 to 24.

【Command configuration mode】

Global configuration mode

【Example】

！Configure Ethernet 2/1 to be mirror destination-interface

Optiway(config)#mirror destination-interface ethernet 2/1

    - mirror source-interface

Use mirror source-interface command to configure mirror source-interface. Use no mirror source-interface command to remove mirror source-interface.

mirror source-interface { interface-list | cpu } { both | egress | ingress }

no mirror source-interface { interface-list | cpu }

【Parameter】

interface-list:List of interfaces provides in the form of interface-num [ to interface-num ], this can be repeated for 3 times.

cpu:Means CPU port

both:Means both egress and ingress can be mirrored

egress:Means egress mirror

ingress:Means ingress mirror

【Command configuration mode】

Global configuration mode

【Example】

！Configure Ethernet 2/1 to ethernet 2/12 to be mirror source-interface

Optiway(config)#mirror source-interface ethernet 2/1 to ethernet 2/12 both

    - show mirror

Use show mirror command to display system configuration of current mirror interface, including monitor port and mirrored port list.

show mirror

【Command configuration mode】

Any configuration mode

【Example】

！Display monitor port and mirrored port list

Optiway(config)#show mirror

    - no mirror all

This command is to delete all ports mirror and flow mirror.

no mirror all

【Command configuration mode】

Global configuration mode

【Example】

！Delete all ports mirror and flow mirror

TiNet(config)#no mirror all

  - Port CAR Configuration Command

Port CAR configuration command includes:

- port-car

- port-car-open-time

- port-car-rate

- show port-car

    - port-car

Use port-car command to enable port CAR of global system or port. Use no port-car command to disable port CAR of global system or port.

port-car

no port-car

【Default】

Port-car globally enables

【Command configuration mode】

Global configuration mode, interface configuration mode

【Example】

！Enable port-car globally

Optiway(config)#port-car

！Enable port-car of Ethernet 2/8

Optiway(config-if-ethernet-2/8)#port-car

    - port-car-open-time

Use port-car-open-time command to configure the reopen time of the port shutdown by port-car. Use no port-car-open-time command to restore the default port-car-open-time.

port-car-open-time port-car-open-time

no port-car-open-time

【Parameter】

port-car-open-time:The reopen time of the port shutdown by port-car. It ranges from 1 to 3622

【Default】

Default port-car-open-time is 482 seconds

【Command configuration mode】

Global configuration mode

【Example】

！Configure port-car-open-time to be 12 seconds

Optiway(config)#port-car-open-time 12

    - port-car-rate

Use port-car-rate command to configure the port-car-rate. Use no port-car-rate command to restore the default port-car-rate.

port-car-rate port-car-rate

no port-car-rate

【Parameter】

port-car-rate:Port-car-rate ranges from 1 to 2622

【Default】

Default port-car-rate is 322 packet/second

【Command configuration mode】

Global configuration mode

【Example】

！Configure port-car-rate to be 122 packet/second

Optiway(config)#port-car-rate 122

    - show port-car

Use show port-car command to display port-car information.

show port-car

【Command configuration mode】

Any configuration mode

【Example】

！Display port-car information

Optiway(config)#show port-car

  - Port LACP Configuration Command

Port LACP configuration command includes:

- channel-group

- channel-group mode

- channel-group load-balance

- higig-trunk load-balance

- lacp system-priority

- lacp port-priority

- show higig-trunk load-balance

- show utilization higig

- show lacp sys-id

- show lacp internal

- show lacp neighbor

- show statistics channel-group

- clear channel-group

- show statistics dynamic channel-group

- show utilization channel-group

    - channel-group

Use channel-group command to create channel group, but there is no member in the group. To remove the group, all the members of the group must be removed first. Use no channel-group command to remove the group.

channel-group channel-group-number

no channel-group channel-group-number

【Parameter】

channel-group-number:Range from 2 to 5

【Default】

Non

【Command configuration mode】

Global configuration mode

【Example】

！Create channel group 1

Optiway(config)#channel-group 1

    - channel-group mode

Use channel-group mode command to add port members to the group, and specify the mode.

channel-group channel-group-number mode {active | passive | on}

no channel-group channel-group-number

【Parameter】

channel-group-number:Range from 2 to 5

【Default】

Non

【Command configuration mode】

Interface /Interface group configuration mode

【Example】

！Add Ethernet 2/3 to channel-group 3 and specify the port to be active mode

Optiway(config-if-ethernet-2/3)#channel-group 3 mode active

！Add Ethernet 2/6 to ethernet 2/8 to channel-group 2 and specify the ports to be on mode

Optiway(config)#interface range ethernet 2/6 to ethernet 2/8

Optiway(config-if-range)#channel-group 2 mode on

    - channel-group load-balance

Use channel-group load-balance command to configure channel-group load-balance, that is, choose physical link program when message sending.

channel-group channel-group-number  load-balance

{dst-ip|dst-mac|src-dst-ip|src-dst-mac|src-ip|src-mac}

【Parameter】

channel-group-number:Range from 2 to 5

【Default】

Source MAC mode

【Command configuration mode】

Global configuration mode

【Example】

！Specify load-balance of channel-group 2 is destination mac

Optiway(config)#channel-group load-balance dst-mac

    - lacp system-priority

Use lacp system-priority command to configure lacp system priority. Use no lacp system-priority command to restore default priority.

The redundancy influence made by LACP system and port priority shows: LACP providing redundancy system needs guarantee the consistency of the choosing redundancy for conterminous OLTes, and user can configure redundancy link, which is realized by system and port priority. Choose redundancy in following steps:

1,Make sure which OLT is the standard of choice. For exchanging the message, two OLTes know each other’s LACP system priority and system mac. They compare local LACP system priority, the smaller one is the standard; if they have the same priority, compare the system MAC, the smaller is the standard.

2,Choose redundancy link with the port parameter of the standard OLT. Compare the port LACP priority first, the inferior is the redundant; if they have the same priority, the larger number of the port is redundant.

lacp system-priority priority

no lacp system-priority priority

【Parameter】

priority:Range from 1 to 65535

【Default】

default priority is 32768

【Command configuration mode】

Global configuration mode

【Example】

！Configure LACP system priority is 42222

Optiway(config)#lacp system-priority 42222

    - lacp port-priority

Use lacp port-priority command to configure lacp port-priority. When the port backup exists, the inferior one backups. Use no lacp port-priority command to restore default lacp port-priority.

lacp port-priority priority

【Parameter】

priority:Range from 1 to 65535

【Default】

Default priority is 128

【Command configuration mode】

Interface /Interface group configuration mode

【Example】

！Configure lacp port-priority of Ethernet 2/2 to be 12345

Optiway(config-if-ethernet-2/2)#lacp port-priority 12345

    - show lacp sys-id

Use show lacp sys-id command to display lacp system id, which is in the form of 16 characters of system priority and 32 characters of system MAC address.

show lacp sys-id

【Parameter】

Non

【Default】

Non

【Command configuration mode】

Any configuration mode

【Example】

！Display lacp system id

Optiway(config)#show lacp sys-id

    - show lacp internal

Use show lacp interval command to display the information of group members, if the there is no keywords, all groups are displayed.

show lacp internal  [channel-group-number ]

【Parameter】

channel-group-number:Range from 2 to 5

【Default】

Non

【Command configuration mode】

Any configuration mode

【Example】

！Such as:

Optiway#show lacp internal

    - show lacp neighbor

This command displays the channel adjacent port, if you do not enter a parameter , then display all channels from adjacent ports.

show lacp neighbor  [channel-group-number ]

【Parameter】

channel-group-number:Range from 2 to 7

【Default】

Non

【Command configuration mode】

Any configuration mode

【Example】

！Such as:

Optiway#show lacp neighbor

    - show statistics channel-group

Use this command to display statistic channel-group, it shows current all channel group if no parameter enters.

show statistics channel-group  [channel-group-number ]

【Parameter】

channel-group-number:channel index is 0～5

【Default】

None

【Command mode】

All modes

【Example】

！For example:

OptiWay#show statistics channel-group

    - clear channel-group

Use this command to delete statistic channel-group, it deletes current all channel group if no parameter enters.

clear channel-group  [channel-group-number ]

【Parameter】

channel-group-number:channel index is 0～5

【Default】

None

【Command mode】

All modes

【Example】

！For example:

OptiWay#clear channel-group

    - show statistics dynamic channel-group

Use this command to display dynamic channel-group.

show statistics dynamic channel-group

【Default】

None

【Command mode】

All modes

【Example】

！For example:

OptiWay#show statistics dynamic channel-group

    - show utilization channel-group

Use this command to display all channel utilization, including  receive and send rate, port bandwidth occupancy rate etc.

show utilization channel-group

【Parameter】

None

【Default】

None

【Command mode】

All modes

【Example】

！For example:

OptiWay#show utilization channel-group

  - Port Alarm Configuration Command

Port alarm configuration command includes:

- alarm all-packets

- alarm all-packets threshold

- show alarm all-packets

    - alarm all-packets

Use alarm all-packets command to enable global or port all-packets alarm.

Use no alarm all-packets command to disable global or port all-ports alarm.

alarm all-packets

no alarm all-packets

【Default】

Alarm all-packets enable

【Command configuration mode】

Global/interface configuration mode

【Example】

！ Enable global alarm all-packets

Optiway(config)#alarm all-packets

！Enable alarm all-packets of Ethernet 2/8

Optiway(config-if-ethernet-2/8)#alarm all-packets

    - alarm all-packets threshold

Use alarm all-packets threshold command to configure alarm all-packets exceed and normal threshold.

alarm all-packets threshold [ exceed exceed ] [ normal normal ]

no alarm all-packets

【Parameter】

exceed:Exceed threshold. 122BASE ranges from 2 to 12222

normal: normal threshold. 122BASE ranges from 2 to 12222

【Default】Default GE exceed threshold is 852,normal threshold is 622.

Default 12GE exceed threshold is 8522,normal threshold is 6222.

【Command configuration mode】

Interface configuration mode

【Usage】

Exceed > normal

【Example】

！Configure alarm all-packets exceed threshold to be 52,and normal threshold to be 32

Optiway(config)#alarm all-packets threshold exceed 522 normal 322

    - show alarm all-packets

Use show alarm all-packets command to display the information of global alarm all-packets.

show alarm all-packets

【Command configuration mode】

Any configuration mode

【Example】

！Display global alarm all-packets information

Optiway(config)#show alarm all-packets

Port alarm global status : enable

Port alarm exceed port

    - show alarm all-packets interface

Use show alarm all-packets interface command to display port alarm all-packets information.

show alarm all-packets interface [ interface-list ]

【Parameter】

interface-num:List of Ethernet ports to be added to or removed from a VLAN. This keyword needed to be provided in the form of interface-type + interface-number. Interface-type is Ethernet and interface-number is slot-num/port-num, in which slot-num is in the range of 2 to 2, and port-num is in the range of 1 to 24. Seriate interfaces with the same type can be linked by to keyword, but the port number to the right of the to keyword must be larger than the one to the left of the keyword, and this argument only can be repeated for up to 3 times.

【Command configuration mode】

Any configuration mode

【Usage】

Keyword “interface-list” is alternative. If there is no keyword, the alarm all-packets of all the interfaces are displayed, or the information of specified port is displayed.

【Example】

！Display the alarm all-packets interface information of Ethernet 2/1

Optiway(config)#show alarm all-packets interface ethernet 2/1

  - Interface shutdown-control Configuration Command

Interface shutdown-control Configuration Command includes:

- shutdown-control

- shutdown-control-open-time

- show shutdown-control

    - shutdown-control

Use shutdown-control command to configure interface shutdown-control.

shutdown-control [ broadcast | multicast | unicast ] target-rate

no shutdown-control [ broadcast | multicast | unicast ]

【Parameter】

broadcast:configure broadcast shutdown-control

multicast:configure multicast shutdown-control

unicast:configure unicast shutdown-control

target-rate:disable target rate

【Command configuration mode】

Global configuration mode

【Example】

！Enable shutdown-control of e2/8 for broadcast and speed rate is 122pps

Optiway(config-if-ethernet-2/8)#shutdown-control broadcast 122

    - shutdown-control-open-time

Use shutdown-control-open-time command to configure shutdown-control reopen time. Use no command to restore to default shutdown-control- open-time.

Shutdown-control-open-time  open-time

no shutdown-control-open-time

【Parameter】

open-time:shutdown-control reopen time  which is in the range of 1～3622

【Default】

The default shutdown-control open-time is 482 seconds.

【Command configuration mode】

Global configuration mode

【Example】

！Configure shutdown-control-open-time of CAR is 12 seconds.

Optiway(config)#shutdown-control-open-time 12

    - no shutdown-control-recover

Use this command to recover port shutdown-control as default configuration.

no shutdown-control-recover mode | automatic-open-time

【Command mode】

Global configuration mode

【Example】

！Recover port shutdown-control mode as default manual mode

OptiWay(config)# no shutdown-control-recover mode

    - show shutdown-control

Use show shutdown-control command to display interface shutdown-control information.

show shutdown-control

【Command configuration mode】

Any configuration mode

【Example】

！Display interface shutdown-control information

Optiway(config)#show shutdown-control

# VLAN Configuration Command

  - VLAN Configuration

VLAN(Virtual Local Area Network) configuration includes:

- description

- show vlan

- OLTport

- vlan

    - description

Use description command to assign a description string to the current VLAN. Use no description command to delete the description of the current VLAN.

description string

no description

【Parameter】

string:It is in the range of 1 to 32 characters to describe the current VLAN. The characters can be printable, excluding such wildcards as '/',':','*','?','\\','<','>','|','"'etc.

【Command configuration mode】

VLAN configuration mode

【Usage】

This command can assign a description to the current VLAN.

【Example】

！Specify the description string of the current VLAN as “market”

Optiway (config-if-vlan)#description market

    - show vlan

Use  show vlan command to display the information about the specified VLAN

show vlan [ vlan-id ]

【Parameter】

vlan-id:Specified the VLAN ID is in the range of 1 to 4294.

【Command configuration mode】

Any configuration mode

【Usage】

This command is used to display the information about the specified VLAN, including VLAN ID, VLAN description, and member ports.

If the VLAN with specified keyword exists, this command displays the information of the specified VLAN. If no keyword is specified, this command displays the list of all the existing VLANs.

【Example】

！Display the information of all the existing VLANs

Optiway(config)#show vlan

    - OLTport

Use OLTport command to add a port or multiple ports to a VLAN. Use no OLTport command to remove a port or multiple ports from a VLAN.

OLTport { interface-list | all }

no OLTport { interface-list | all }

【Parameter】

interface-list:List of Ethernet ports to be added to or removed from a VLAN. This keyword needed to be provided in the form of interface-type + interface-number. Interface-type is Ethernet and interface-number is slot-num/port-num, in which slot-num is in the range of 2 to 2, and port-num is in the range of 1 to 24. Seriate interfaces with the same type can be linked by to keyword, but the port number to the right of the to keyword must be larger than the one to the left of the keyword, and this argument only can be repeated for up to 3 times.

all:Means all the interfaces. When the keyword all is specified, all the interfaces in the system are added to a VLAN by using the OLTport command, and all the interfaces are removed from a VLAN by using the no OLTport command.

【View】

VLAN configuration view

【Usage】

In no OLTport command, all the interfaces would be removed from a VLAN when the interface-list is unspecified. When removing the interface from VLAN 1 (default VLAN), if the PVID of the interface is 1, the PVID must be changed into other VLAN ID, or the removing fails. When removing interface from other VLANs, if the PVID of the interface is the same as the VLAN ID, and the interface is also in VLAN 1, the removing succeeds, and the PVID of the interface default to 1, or the removing fails

【Example】

！Add Ethernet 1, 3, 4, 5, 8 to current VLAN

Optiway(config-if-vlan)#OLTport ethernet 2/1 ethernet 2/3 to ethernet 2/5 ethernet 2/8

！Remove Ethernet 3, 4, 5, 8 from current VLAN

Optiway(config-if-vlan)#no OLTport ethernet 2/3 to ethernet 2/5 ethernet 2/8

    - vlan

Use vlan command to enter VLAN mode. If the VLAN identified by the vlan-id argument does not exist, this command creates the VLAN and then enters VLAN mode. Use the no vlan commands to remove a VLAN.

vlan vlan-list

no vlan { vlan-list | all }

【Parameter】

vlan-list:The VLAN which you want to create and whose view you want to enter. Each id ranges from 1 to 4294.

all:Specifying all when removing VLAN, all created VLANs are removed except the default VLAN.

【Command configuration mode】

Global configuration mode

【Usage】

Use the vlan command to enter VLAN configuration view. If the vlan identified by the vlan-id keyword exists, enter VLAN configuration view. If not, this command creates the VLAN and then enters VLAN configuration view. Use the no vlan command to remove a VLAN. Caution: Default VLAN (VLAN 1) cannot be removed. If there is some port with the same default vlan-id as VLAN 1, the port’s VLAN will become VLAN 1 after using the no vlan command. If the VLAN to be removed exists in the multicast group, remove the related multicast group first.

【Example】

！Enter VLAN 1 configuration view

Optiway(config)#vlan 1

  - GVRP Configuration Command

GVRP command includes:

- gvrp

- show gvrp

- show gvrp interface

    - gvrp

Use the gvrp command to enable GVRP globally in global configuration mode or a port in Ethernet port configuration mode. Use no gvrp command to disable GVRP globally in global configuration mode or a port in Ethernet port configuration mode.

gvrp

no gvrp

【Default】

Disable GVRP globally

【Command configuration mode】

Globally configuration mode, Ethernet port configuration mode

【Usage】

You can enable GVRP only on trunk ports.

【Example】

！Enable GVRP globally

Optiway(config)#gvrp

！Enable GVRP on Ethernet port 8

Optiway(config-if-ethernet-2/8)#gvrp

    - show gvrp

Use show gvrp command to display the information about GVRP globally.

show gvrp

【Command configuration mode】

Any configuration mode

【Example】

！Display the information about GVRP globally

Optiway(config)#show gvrp

GVRP  state : enable

    - show gvrp interface

Use show gvrp interface command to display GVRP information on Ethernet port.

show gvrp interface [ interface-list ]

【Parameter】

interface-list:List of Ethernet ports to be added to or removed from a VLAN. This keyword needed to be provided in the form of interface-type + interface-number. Interface-type is Ethernet and interface-number is slot-num/port-num, in which slot-num is in the range of 2 to 2, and port-num is in the range of 1 to 24. Seriate interfaces with the same type can be linked by to keyword, but the port number to the right of the to keyword must be larger than the one to the left of the keyword, and this argument only can be repeated for up to 3 times.

【Command configuration mode】

Any configuration mode

【Usage】

Interface-list keyword is optional. If this keyword unspecified, the command displays GVRP information for all the Ethernet ports. If specified, the command displays GVRP information on specified Ethernet port.

【Example】

！Display GVRP information on Ethernet port 3, 25, 26

Optiway(config)#show gvrp interface ethernet 2/3 ethernet 2/5 ethernet 2/6

    - garp permit vlan

Use garp permit vlan command to add configured static vlan to GVRP module for other OLTes to learn.

garp permit vlan vlan-list

no garp permit vlan [ vlan-list]

【Parameter】

vlan-list:List of VLANs to be entered or to be created and entered. The single VLAN is in the range of 1 to 4294. The list is in the form of number, -, such as: 2, 5, 8, 12-22.

【Command configuration mode】

Global configuration mode

【Example】

!Add vlan 2, 3, 7 to GVRP

Optiway(config)#garp permit vlan 2-3,7

    - show garp permit vlan

Use show garp permit vlan command to display current static vlan permitted learning by GVRP

show garp permit vlan

【Command configuration mode】

Global configuration mode

【Example】

Display current static vlan permitted learning by GVRP

Optiway(config)#show garp permit vlan

  - QinQ command

QinQ command includes:

- dtag

- dtag inner-tpid

- dtag outer-tpid

- dtag mode

- show dtag

- dtag insert

- show dtag insert

- dtag swap

- show dtag swap

- dtag pass-through

    - dtag

Use this command to configure global QinQ.

dtag

no dtag

【Command configuration mode】

Global configuration mode

【Example】

！Enable QinQ

Optiway(config)dtag

    - dtag mode

Use dtag mode command to configure interface QinQ mode.

dtag mode { customer | service-provider }

no dtag mode

【Parameter】

customer: In this mode, the original tag head will be ignored and a new one will be added.

service-provider: In this mode, when the vlan protocol number of ingress packet is different from the configured parameter of the interface, a new tag head will be added.

【Command configuration mode】

Interface configuration mode

【Example】

Configure interface to be customer interface.

Optiway(config-if-ethernet-2/1)#dtag mode customer

    - show dag

Display the QinQ configurationof the OLT.

show dtag

【Command configuration mode】

Any configuration mode

【Example】

!Display the QinQ configuration

Optiway(config)#show dtag

    - dtag insert

Add vlan tag head in QinQ in interface configuration mode.

dtag insert  startvlanid  endvlanid  targetvlanid

no dtag insert  startvlanid  endvlanid

【Parameter】

startvlanid:the start vlan id which needs new tag head.

endvlanid:the end vlan id which needs new tag head.

targetvlanid:tag vlan added new tag head and it is transferred according to the new tag vlan.

【Command configuration mode】

Interface configuration mode

【Example】

Configure all vlans from vlan1 to vlan2 in e2/1 to add new tag head with the tag vlan to be vlan3

Optiway(config-if-ethernet-2/1)#dtag insert 1 2 3

    - show dag insert

Display vlan insert configuration.

show dtag insert

【Command configuration mode】

Any configuration mode

【Example】

Display current vlan insert configuration

Optiway(config)#show dtag insert

    - dtag swap

Configure OLTing vlan in interface mode.

dtag swap  startvlanid  endvlanid  targetvlanid

no dtag swap startvlanid  endvlanid

【Parameter】

startvlanid:start vlan needed to be replaced

endvlanid:end vlan needed to be replaced

targetvlanid:the vlan used to replace original vlan ID.

【Command configuration mode】

Interface configuration mode

【Example】

Configure tag packet from vlan1 to vlan 3 in e2/1 is replaced by vlan5

Optiway(config-if-ethernet-2/1)#dtag swap 1 3  5

    - show dag swap

Display current vlan swap configuration.

show dtag swap

【Command configuration mode】

Any configuration mode

【Example】

Display current vlan swap configuration

Optiway(config)#show dtag swap

    - dtag pass-through

Configure vlan transparent transmission in interface mode:

dtag pass-through  startvlanid  endvlanid

no dtag pass-through  startvlanid  endvlanid

【Parameter】

startvlanid:the start vlan needed to be transparent transmision

endvlanid:the end vlan needed to be transparent transmision

【Command configuration mode】

Interface configuration mode

【Example】

Configure tag packet transparent transmission from vlan 1 to vlan 3 in e2/1

Optiway(config-if-ethernet-2/1)#dtag pass-through 1 3

    - show dag pass-through

Display vlan pass-through configuration.

show dtag pass-through

【Command configuration mode】

Any configuration mode

【Example】

Display vlan pass-through configuration

Optiway(config)#show dtag pass-through

  - l2-tunnel Configuration

In VPN network, it needs encapsulating some protocol packets received in service-provider network edge according to a certain form. The internal device of SP network can recognize this encapsulation and guarantee the packet pass through SP network without any change. The encapsulation will be released on the other side of SP network for the normal session of the peer entity connected on the edge of SP network.

L2-tunnel command include:

- l2-tunnel

- show l2-tunnel interface

- l2-tunnel drop-threshold

- show l2-tunnel drop-threshold

    - l2-tunnel

Use this command to configure l2-tunnel status.

l2-tunnel [ cdp | pagp | lacp | stp | udld | vtp ]

no l2-tunnel [ cdp | pagp | lacp | stp | udld | vtp ]

【Parameter】

cdp:tunnel cisco cdp packet

pagp:tunnel cisco pagp packet

lacp: tunnel lacp packet

stp: tunnel stp packet

udld:tunnel cisco udld packet

vtp:tunnel cisco vtp packet

【Command configuration mode】

Interface configuration mode

【Example】

Configure l2-tunnel stp on e2/1.

Optiway(config-if-ethernet-2/1)#l2-tunnel stp

    - show l2-tunnel interface

Use this command to display l2-tunnel configuration.

show l2-tunnel interface interface-list

【Parameter】

interface-list :refer to ’interface range’

【Command configuration mode】

Any configuration mode

【Example】

Display l2-tunnel configuration

Optiway(config-if-ethernet-2/1)#show l2-tunnel interface

    - l2-tunnel drop-threshold

Use this command to configure l2-tunnel drop-threshold.

l2-tunnel drop-threshold[ cdp | pagp | lacp | stp | udld | vtp ] target-rate

no l2-tunnel  drop-threshold[ cdp | pagp | lacp | stp | udld | vtp ]

【Parameter】

cdp:tunnel cisco cdp packet

pagp:tunnel cisco pagp packet

lacp: tunnel lacp packet

stp: tunnel stp packet

udld:tunnel cisco udld packet

vtp:tunnel cisco vtp packet

target-rate:target rate of packet

【Command configuration mode】

Global configuration mode

【Example】

Configure the speed rate of cpu receiving stp packet to be 12pps

Optiway(config)#l2-tunnel drop-threshold stp 12

    - show l2-tunnel drop-threshold

Use this command to display l2-tunnel drop-threshold.

show l2-tunnel drop-threshold

【Command configuration mode】

Any configuration mode

【Example】

Display l2-tunnel drop-threshold configuration

Optiway(config)#show l2-tunnel drop-threshold

# ARP Configuration Command

  - ARP Configuration Command

ARP Configuration Command includes:

- arp

- no arp

    - no arp

Use no arp command to delete a specified arp item.

no arp { all | ip-address }

【Parameter】

ip-address:IP address of ARP maping item.

all:all arp item

【Command configuration mode】

Global configuration mode

【Usage】

This command can delete a arp item. If IP address is specified, delete the corresponded item; if choose all, delete all arp item. Above command cannot delete interface corresponded arp item.

【Example】

！Delete arp item corresponded to IP address 192.168.0.100

Optiway(config)#no arp 192.168.0.100

    - show arp

Use this command to show arp item.

show arp { all | ip-address }

【Parameter】

ip-address : IP address of ARP maping item.

all : all arp item

【Command configuration mode】

Global configuration mode

【Usage】

Use this command to show arp items

【Example】

！arp item corresponded to IP address 192.168.0.100

TiNet(config)#show arp 192.168.0.100

！show all arp items

TiNet(config)#show arp all

# DHCP Configuration Command

  - DHCP Configuration Command

  - dhcp-relay

  - dhcp-relay hide server-ip

  - dhcp-server

  - dhcp-snooping

  - dhcp-snooping trust

  - dhcp-snooping max-clients

  - dhcp option82

  - dhcp option82 strategy

  - dhcp option82 format

  - dhcp option82 circuit-id string

  - dhcp option82 remote-id string

  - ip-source-guard

  - ip-source-guard bind ip

  - show dhcp-relay

  - show dhcp-relay hide server-ip

  - show dhcp-server

  - show dhcp-server interface

  - show dhcp-snooping

  - show dhcp-snooping clients

    - dhcp-relay

Use Dhcp-relay command to enable DHCP relay. Use no dhcp-relay command to disable DHCP relay.

Dhcp-relay

no dhcp-relay

【Command configuration mode】

Global configuration mode

【Usage】

Enable DHCP relay before enabling DHCP server.

【Example】

！Enable DHCP relay

Optiway(config)#dhcp-relay

！Disable DHCP relay

- Optiway(config)#no dhcp-relay

    - dhcp-server

Use following command to configure DHCP server. Use its no command to delete configured DHCP server. Configure in global configuration mode:

dhcp-server dhcp-num ip ip-addres

no dhcp-server dhcp-num

Followings are specified DHCP server of layer 3 interface. Use its no command to cancel it. Configure it in interface configuration mode.

dhcp-server dhcp-num

no dhcp-server

【Parameter】

dhcp-num:DHCP server number which is in the range of 1～32

ip-addres:IP address of DHCP server

【Command configuration mode】

Global configuration mode, interface configuration mode

【Usage】

Use this command in global configuration mode to configure DHCP server. Specify DHCP server in layer 3 interface in interface configuration mode. If configuring IP address of DHCP server to be the IP address of some layer 3 interface, built-in DHCP server will be used.

【Example】

！Configure IP address of DHCP server 2 to be 192.168.2.122

Optiway(config)#dhcp-server 2 ip 192.168.2.122

！Delete DHCP server 2

Optiway(config)#no dhcp-server 2

！Specify VLAN interface 2 to use DHCP server 1

Optiway(config-if-vlanInterface-2)#dhcp-server 1

！Cancel specified DHCP server of VLAN interface 2

Optiway(config-if-vlanInterface-2)#no dhcp-server

    - dhcp-snooping

Use dhcp-snooping command to configure DHCP SNOOPING. Use no dhcp-snooping command to delete this configuration. Configure it in global configuration mode.

dhcp-snooping

no dhcp-snooping

【Command configuration mode】

Global configuration mode

【Example】

！Enable DHCP SNOOPING

Optiway(config)#dhcp-snooping

    - dhcp-snooping trust

Use dhcp-snooping trust command to configure DHCPSNOOPING interface to be trust interface. Use no dhcp-snooping trust command to restore it to be non-trust interface.

dhcp-snooping trust

no dhcp-snooping trust

【Command configuration mode】

Interface configuration mode

【Usage】

Interface is defaulted to be nontrust. Valid DHCP server must connect to trust interface.

【Example】

！Configure ethernet 2/1 to be trust interface.

Optiway(config-if-ethernet-2/1)#dhcp-snooping trust

    - dhcp-snooping max-clients

Followings are used to configure interface or the max DHCP client number permitted by VLAN.

dhcp-snooping max-clients num

no dhcp-snooping max-clients

【Parameter】

num:the max number which is in the range of 2 to 2248. it is defaulted to be 2248.

【Command configuration mode】

Interface configuration mode, VLAN configuration mode

【Example】

！Configure the max learning number of Ethernet 2/1 to be 33

Optiway(config-if-ethernet-2/1)#dhcp-snooping max-clients 33

！Configure the max learning number of VLAN 2 to be 33

Optiway(config-if-vlan)#dhcp-snooping max-clients 33

    - ip-source-guard

Use this command to enable interface IP source guard. Use no command to disable it.

ip-source-guard

no ip-source-guard

【Command configuration mode】

Interface configuration mode, VLAN configuration mode

【Example】

！Enable IP source guard of e2/1

Optiway(config-if-ethernet-2/1)#ip-source-guard

    - ip-source-guard bind ip

Use this command to configure IP source guard to static bind entry, use no to delete IP source guard static bind entry

ip-souce-guard bind ip ip-address [ mac mac [ interface ethernet interface-num ] ]

no ip-source-guard bind ip { ip-address | all }

【Command mode】

Global configuration mode

【Example】

！Configure IP source guard static bind entry

OptiWay(config-if-ethernet-2/1)#ip-source-guard bind ip 192.168.0.1 mac 0:0:0:0:0:1 interface ethernet 2/1

    - show dhcp-relay

Use show dhcp-relay command to display DHCP relay configuration.

show dhcp-relay

【Command configuration mode】

Any configuration mode

【Example】

！Display DHCP relay configuration

Optiway(config)#show dhcp-relay

DHCP relay is enabled!

    - show dhcp-snooping interface

Use show dhcp-snooping interface command to show DHCP SNOOPING configuration.

show dhcp-snooping interface [ interface-num ]

【Parameter】

interface-num:consist of port type + port num,port type is ethernet,port num is device/slot-num/port-num,device means stack device num,the range is  0～7,slot-num is slot num,the range is 0～1,port-num is the port num,the range is 1～48.

【Command configuration mode】

Any configuration mode

【Usage】

When displaying port information, if not appoints the port type and port num,it displays all ports information；if appoints to port type and port num, it displays select port information.

【Example】

！Display port 1 DHCP SNOOPING configuration

OptiWay(config)#show dhcp-snooping interface ethernet 2/1

    - show dhcp-snooping vlan

Use this command to display VLAN DHCP SNOOPING configuration.

show dhcp-snooping vlan [ vlan-id ]

【Command mode】

All command mode

【Usage】

When displaying vlan,it diplays configured vlan if without selected vlan-id,it diplays selected vlan if with vlan-id.

【Example】

！Display configured all vlan DHCP SNOOPING configuration

OptiWay(config)#show dhcp-snooping vlan

    - show dhcp-snooping clients

Use show dhcp-snooping clients command to display user information.

show dhcp-snooping clients

【Command configuration mode】

Any configuration mode

【Example】

！Display user information

Optiway(config)#show dhcp-snooping clients

# Multicast Protocol Configuration Command

  - Static Multicast Configuration Command

Static multicast configuration command includes:

- multicast mac-address

- multicast mac-address vlan interface

- show multicast

    - multicast mac-address

Use multicast mac-address command to create a multicast group. Use no multicast mac-address command to remove multicast group formed by specified mac address and related vlan-id.

multicast mac-address mac vlan vlan-id

no multicast [ mac-address mac vlan vlan-id ]

【Parameter】

mac:The mac address of multicast group displayed in the form of multicast address, such as: 21:22:5e:**:**:**

vlan-id:Range from 1 to 4294

【Command configuration mode】

Global configuration mode

【Usage】

To create multicast group, MAC address should be multicast group address, and vlan-id must be existed. If there is no parameter in any multicast mac-address command, all multicast group are removed.

【Example】

！Create a multicast group

Optiway(config)#multicast mac-address 21:22:5e:21:22:23 vlan 1

    - multicast mac-address vlan interface

Use multicast mac-address vlan interface command to add interface to existed multicast group. Use no multicast mac-address vlan interface command to remove interface.

multicast mac-address mac vlan vlan-id interface { all | interface-list }

no multicast mac-address mac vlan vlan-id interface { all | interface-list }

【Parameter】

mac:Means mac address of existed multicast which is in the form of multicast mac-address, such as: 21:22:5e:**:**:**

vlan-id:Range from 1 to 4294. Multicast group is assembled by vlan-id and mac-address.

interface-list:List of Ethernet ports to be added to or removed from a VLAN. This keyword needed to be provided in the form of interface-type + interface-number. Interface-type is Ethernet and interface-number is slot-num/port-num, in which slot-num is in the range of 2 to 2, and port-num is in the range of 1 to 24. Seriate interfaces with the same type can be linked by to keyword, but the port number to the right of the to keyword must be larger than the one to the left of the keyword, and this argument only can be repeated for up to 3 times.

all:means all interfaces in system in multicast mac-address vlan interface command, and means all the interfaces of the multicast group in the no multicast mac-address vlan interface command.

【Command configuration mode】

Global configuration mode

【Example】

！Remove ethernet 2/2 from existed multicast group.

Optiway(config)#no multicast mac-address 21:22:5e:21:22:23 vlan 1 interface ethernet 2/2

    - show multicast

Use show multicast command to display the information of the specified or all existed multicast group.

show multicast [ mac-address mac ]

【Parameter】

mac:MAC address existed in multicast group

【Command configuration mode】

Any configuration mode

【Usage】

If mac-address is not specified, information of the entire multicast group is displayed.

【Example】

！Display the information of multicast group with the MAC address to be 21:22:5e:21:22:23

Optiway(config)#show multicast mac-address 21:22:5e:21:22:23

  - IGMP snooping and GMRP Configuration Command

IGMP snooping and GMRP configuration command includes:

- gmrp

- igmp-snooping

- igmp-snooping host-aging-time

- igmp-snooping max-response-time

- igmp-snooping fast-leave

- igmp-snooping group-limit

- igmp-snooping permit/deny group

- igmp-snooping route-port forward

- igmp-snooping multicast vlan

- show gmrp

- show gmrp interface

- show igmp-snooping

    - gmrp

Use gmrp command to enable GMRP globally or for a port. Use no GMRP command to disable GMRP globally or for a port.

gmrp

no gmrp

【Default】

GMRP disables globally

【Command configuration mode】

Global configuration mode,Interface configuration mode

【Usage】

GMRP for a port must be enabling in trunk mode

【Example】

！Enable GMRP globally

Optiway(config)#gmrp

！Disable the GMRP of Ethernet 2/3

Optiway(config-if-ethernet-2/3)#no gmrp

    - igmp-snooping

Use igmp-snooping command to enable IGMP snooping. Use no IGMP-snooping command to disable IGMP snooping.

igmp-snooping

no igmp-snooping

【Default】

IGMP snooping disable

【Command configuration mode】

Global configuration mode

【Example】

！Enable IGMP snooping

Optiway (config)#igmp-snooping

    - igmp-snooping host-aging-time

Use igmp-snooping host-aging-time command to configure the host-aging-time of dynamic multicast group learnt by igmp-snooping. Use no igmp-snooping host-aging-time command to restore the default host-aging-time.

igmp-snooping host-aging-time seconds

no igmp-snooping host-aging-time

【Command configuration mode】

Global configuration mode

【Parameter】

seconds:range from 12 to 1222222 seconds

【Example】

！Configure host-aging-time of the dynamic multicast group learnt by igmp-snooping to be 12 seconds

Optiway(config)#igmp-snooping host-aging-time 12

    - igmp-snooping max-response-time

When receiving a leave message, igmp-snooping will wait for some time to see whether to remove interface of igmp-snooping multicast group. The time is the response time.

igmp-snooping max-reponse-time seconds

no igmp-snooping max-reponse-time

【Command configuration mode】

Global configuration mode

【Parameter】

seconds:Range from 1 to 122 seconds. The default time is 12 seconds

【Usage】

This command is effective when fast leave disables

【Example】

！Configure the max-response-time of igmp-snooping is 99 seconds

Optiway(config)#igmp-snooping max-response-time 99

    - igmp-snooping fast-leave

Use igmp-snooping fast-leave command to configure fast-leave of the interface. When fast-leave enables, if the fast-leave message is received, the interface leaves the aging group, or the time to leave is determined by the max-response-time.

igmp-snooping fast-leave

no igmp-snooping fast-leave

【Command configuration mode】

Interface configuration mode

【Default】

Fast-leave disables

【Example】

！Enable igmp-snooping fast-leave

Optiway(config-if-ethernet-2/1)#igmp-snooping fast-leave

    - igmp-snooping group-limit

Use igmp-snooping group-limit command to configure the number of the multicast group allowed learning.

igmp-snooping group-limit limit

no igmp-snooping group-limit

【Command configuration mode】

Interface configuration mode

【Parameter】

limit:Range from 2 to 128.  The default number is 128

【Example】

！Configure the igmp-snooping group-limit to be 99

Optiway(config-if-ethernet-2/1)#igmp-snooping group-limit 99

    - igmp-snooping permit/deny group

Use igmp-snooping permit/deny group command to configure the permit and deny group, and the learning regulations of the group which is not permit or deny group (We call it default group).

igmp-snooping permit/deny group [ all | group-address]

no igmp-snooping permit/deny group [group-address]

【Command configuration mode】

Interface configuration mode for permit/deny group

Global configuration mode for the learning regulations of default group

【Parameter】

group-address:Multicast MAC address is in the form of 21:22:5e:21:22:23

【Example】

！Configure the learning regulation of default group to allow all multicast group

Optiway(config)#igmp-snooping permit group all

！Configure Ethernet 2/3 not to learn multicast 21:22:5e:22:21:21

Optiway(config-if-ethernet-2/3)#igmp-snooping deny group 21:22:5e:22:21:21

    - igmp-snooping route-port forward

Multicast routers interface is the interface received IGMP inquiring message (It is also called mix router interface.).

Use igmp-snooping route-port forward command to configure whether to add router interface to IGMP snooping learning group.

igmp-snooping route-port forward

no igmp-snooping route-port forward

【Command configuration mode】

Global configuration mode

【Default】

Disable

【Example】

！Enable igmp-snooping route-port forward

Optiway(config)#igmp-snooping route-port forward

    - igmp-snooping multicast vlan

Use igmp-snooping multicast vlan command to specify a VLAN for a port to learn and transmit multicast message. IGMP message intercepted by IGMP Snooping will modify its VID to be specified VLAN to transmit. Descendent multicast message is transmitted in VLAN, and separated with unicast message VLAN.

igmp-snooping multicast vlan vlan-id

no igmp-snooping multicast vlan

【Command configuration mode】

Interface configuration mode

【Parameter】

vlan-id:Range from 1 to 4294

【Default】

No multicast VLAN configuration for port

【Example】

！Configure multicast vlan of Ethernet 2/1 to be vlan 2

Optiway(config-if-ethernet-2/1)#igmp-snooping multicast vlan 2

    - show gmrp

Use show gmrp command to display GMRP globally.

show gmrp

【Command configuration mode】

Any configuration mode

【Example】

！Display GMRP information globally

Optiway(config)#show gmrp

GMRP  state : enable

    - show gmrp interface

Use show gmrp interface command to display GMRP information of an interface.

show gmrp interface [ interface-list ]

【Parameter】

interface-list:List of Ethernet ports to be added to or removed from a VLAN. This keyword needed to be provided in the form of interface-type + interface-number. Interface-type is Ethernet and interface-number is slot-num/port-num, in which slot-num is in the range of 2 to 2, and port-num is in the range of 1 to 24. Seriate(sequential?) interfaces with the same type can be linked by to keyword, but the port number to the right of the to keyword must be larger than the one to the left of the keyword, and this argument only can be repeated for up to 3 times.

【Command configuration mode】

Any configuration mode

【Usage】

Key word “interface-list” is optional. If this keyword is lacking, all the information of the interfaces is displayed, or information of only specified interfaces is displayed.

【Example】

！Display information of gmrp interface Ethernet 2/1, ethetnet 2/2, Ethernet 2/3, Ethernet 2/1

Optiway(config)#show gmrp interface ethernet 2/1 to ethernet 2/3 ethernet 3/2

port  GMRP status

e2/1  enable

e2/2  enable

e2/3  enable

e3/2  enable

Total entries: 4

    - garp permit multicast mac-address

Use garp permit multicast mac-address command to add configured static multicast group to GMRP to be dynamic learned by other OLTes.

garp permit multicast [ mac-address mac vlan vlan-id ]

【Parameter】

mac:MAC address of existed multicast group in the form of multicast MAC address, such as: 21:22:5e:**:**:**

vlan-id:Range from 1 to 4294. Multicast group is combined by vlan-id and mac

【Command configuration mode】

Global configuration mode

【Example】

! Add multicast group 21:22:5e:22:21:21 vlan 1 to GMRP

Optiway(config)#garp permit multicast mac-address 21:22:5e:22:21:21 vlan 1

    - show garp permit multicast

Use show garp permit multicast command to display static multicast group permitted learning by GMRP.

show garp permit multicast

【Command configuration mode】

Any configuration mode

【Example】

! Display the static multicast permitted by GMRP

Optiway(config)#show garp permit multicast

    - show igmp-snooping

Use show igmp-snooping command to display the information of IGMP snooping

show igmp-snooping

【Command configuration mode】

Any configuration mode

【Example】

！Display IGMP snooping information

Optiway(config)#show igmp-snooping

    - igmp-snooping route-port vlan interface

The increased routing port specify forwarding port of leave or the report packet belong to a multicast group.

The routing information will not be agingigmp-snooping route-port vlan vlanID interface port-number

no igmp-snooping route-port vlan vlanID interface port-number

【Paramter】

Vlan ID：exist vlan id（1～4094）

interface-list：The select port list means multiple Ethernet ports, interface-list consists port type + port number, port type is ethernet port number represens slot-num/port-num, slot-num is the slot number,range 0, port-num is the slot port number, the range is from 1 to 8 consecutive similar port can be connected to the "to", but keeping up to the front of the port number must be less than tightwith the port number to "behind this form allows up to repeat three times, all ports in the system and can not be set to the port isolation downlink port

all：All ports. Only delete the port isolation downlink port, you can select all, delete all port isolation downstream ports

【Command】

All command modes

【Example】

Configure vlan 2 port 1 for the current group (decision) query source address routing port.

igmp-snooping route-port vlan 2 interface ethernet 0/1

    - no igmp-snooping router-port-age

Configure if aging by IGMP packets learning router ports, by default is aging.

no igmp-snooping router-port-age

igmp-snooping router-port-age

【Command】

All command modes

【Guide】

By default is aging, so use no igmp-snooping router-port-age to disable aging.

【Example】

Configure aging in the period time

no igmp-snooping router-port-age

    - igmp-snooping general-query source-ip

Configured query the message carried by the source IP is to point out the destination response IP.

The default is 0.0.0.0

igmp-snooping general-query source-ip ipaddress

no igmp-snooping general-query source-ip ipaddress

【Command】

All command modes

【Parameter】

ipaddress：address of ipv4 version

【Example】

！Configure source ip as 1.1.1.111 which query sending IGMP source ip.

TiNet(config)# igmp-snooping general-query source-ip 1.1.1.111

    - igmp-snooping query-max-respon

Configured max host response time of receiving the the general inquiries that is querier sends IGMP general query messages carried by the response of the value of the field, the default is 10 seconds.

igmp-snooping query-max-respon  second

no igmp-snooping query-max-respon

【Command】

All command modes

【Parameter】

Seconds：1－255 ，fill the maximum response time of IGMP general query message

【Example】

！Configure max response time as 15s

TiNet(config)# igmp-snooping query-max-respon 150

    - igmp-snooping querier-vlan

Configure the query sends IGMP general query sent to which vlan , the default vlan is 1

igmp-snooping querier-vlan vlanID

no igmp-snooping querier-vlan

【Parameter】

vlanID：exist vlan id（1～4094）

【Command】

All command modes

【Example】

！Configure querier send lan 10 to query message

TiNet(config)# igmp-snooping querier-vlan 10

    - igmp-snooping query-interval

Configure intervals of every two querier sends IGMP general query message  the default is 60s

igmp-snooping query-interval  seconds

no igmp-snooping query-interval

【Command】

All command modes

【Parameter】

Seconds：1－30000s

【Example】

！Intervals of every two querier sends IGMP general query message is 90s

TiNet(config)# igmp-snooping querier 90

    - igmp-snooping querier

Configure query whether to send IGMP general query messages, by default, do not send.

igmp-snooping querier

no igmp-snooping querier

【Command】

All command modes

【Example】

！open the query, it sends IGMP general query messages

TiNet(config)# igmp-snooping querier

    - igmp-snooping preview

This command is to open the IGMP snooping multicast Preview function, no form of this command is used to close

igmp-snooping preview

no igmp-snooping preview

【Default】

By default disable IGMP snooping preview

【Command】

Gloabl configuration mode

【Example】

！Enable IGMP snooping preview

TiNet (config)#igmp-snooping preview

    - igmp-snooping preview time-once time-interval time-reset  permit-times

This command is to configure the IGMP Snooping multicast preview control parameters, including single preview the preview interval reset Preview long and allows preview times, no form of this command to restore the default multicast preview control parameters value.

igmp-snooping preview { time-once time-once time-interval time-interval time-reset time-reset permit-times preview-times }

no igmp-snooping preview { time-once time-interval time-reset permit-times }

【Command】

Gloabl configuration mode

【Parameter】

time-once：单次预览时长，范围60-300s，缺省值180s

time-interval：perview interval，the range is 180-600s，default value is 300s

time-reset：perview reset time ，the range is 1800-7200s，default value is 3600s

preview-times：preview times, he range is 1-10，default value is 5

【Example】

！Configure IGMP Snooping preview time is 60s、interval is 180s、permit times is 8

TiNet(config)#igmp-snooping preview time-once 60 time-interval 180 permit-times 8

# ACL Configuration Command

  - ACL configuration command list

ACL command includes:

- absolute

- access-group

- access-list

- access-list extended

- access-list link

- access-list match-order

- access-list standard

- access-list user

- { permit | deny }

- periodic

- port-isolation

- show access-list config

- show access-list config statistic

- show access-list runtime all

- show access-list runtime statistic

- show port-isolation

- show time-range

- time-range

    - absolute

Use absolute command to create absolute time range. Use no absolute command to delete the configuration of absolute time range.

absolute [ start time date ] [ end time date ]

no absolute [ start time date ] [ end time date ]

【Parameter】

start time date:optional choice. Configure the start absolute time. The form of time is hh:mm:ss,using 24 hours. hh is in the range of 2～23,mm is in the range of 2－59, and ss is in the range of 2－59. The form of date is YYYY/MM/DD. day is in the range of 1～31,month is in the range of 1～12,year is 4 numbers. If the start time is not configured, it means there is no restriction to the start time but the end time.

end time date:optional choice. Configure the end absolute time. The form of time and date is the same as the start time and it must be larger than the start time. If the end time is not configured, it is the max time of system.

【Command configuration mode】

time-range configuration mode

【Usage】

Absolute time range can determine a large scale of effective time and restrict the time range of periodic time. Each time period can define 12 absolute time range. In the period of configuring absolute time and periodic time, only when the absolute time range is satisfied, periodic time range can be judged. When the staart time and end time are not specified, the specified time range is the earlist time the OLT can be recognized to the inferior time.

【Example】

！The following time range will be effective from 2:2 Jan 1st, 2222.

Optiway(config)#time-range tm1

Optiway(config-timerange-tm1)#absolute start 2:2 1-1-2222

Optiway(config-timerange-tm1)#exit

！The following time range will be effective from 22:22 December 12, 2222 to 22:21

Optiway(config)#time-range tm2

Optiway(config-timerange-tm2)#absolute end 22:22 12-12-2222

Optiway(config-timerange-tm2)#exit

！The following time range will be effective from 14:22 to 16:22 in each weekend from 22:22 December 31, 1999 to 22:22 December 12, 2222.（The configuration of periodic time range refers to periodic command.）

Optiway(config)# time-range testall

Optiway(config-timerange-testall)#absolute start 22:22 12-31-1999 end 22:22 12-12-2222

Optiway(config-timerange-testall)#periodic weekend 14:22 to 16:22 Optiway(config-timerange-testall)#exit

    - access-group

Use access-group command to activate accessing control list. Use no access-group command to cancel activate.

access-group { [ ip-group { access-list-number | access-list-name } [ subitem subitem ] ] [ link-group { access-list-number | access-list-name } [ subitem subitem ] ] } }

no access-group { all | [ ip-group { access-list-number | access-list-name } [ subitem subitem ] ] [ link-group { access-list-number | access-list-name } [ subitem subitem ] ] } }

【Parameter】

access-list-number:accessing control list number which is in the range of 1 to 399. access-list-name:the name of accessing list which is the character string and in the form of initial capitalized characters ([a-z, A-Z]), excluding space and quotation mark；subitem subitem:optional parameter, specifies the subitem in accessing list which is in the range of 2～127. If it is not specified, all subitems are activated.

Instruction:

Followings are the parameter of no command.

all:all the activated accessing list must be cancel.（including number and name ID）

【Usage】

This command supports activating accessing control list of layer 2 and layer 3 at the same time, but the action of each accessing control list should not be conflict, if there is conflict (such as one is permit, the other is deny), the activation fails. It can support at most 127 l2 and l3 ACL.

【Command configuration mode】

Global configuration mode

【Example】

！Activate accessing control list 1 and 222 at the same time.

Optiway(config)#access-group ip-group 1 link-group 222

    - access-list

Use access-list command to configure a ACL with number ID, which can be: standard ACL, extended ACL, Layer 2 ACL and user-defined ACL. Use no access-list command to delete all the subitems or one subitem in one ACL with number ID or name ID or all ACLs.

1. Define standard ACL with number ID.

access-list access-list-number1 { permit | deny } { source-addr source-wildcard | any } [ fragments ] [ time-range time-range-name ]

2. Define extended ACL with number ID.

access-list access-list-number2 { permit | deny } [ protocol ] [ established ]

{ source-addr source-wildcard | any } [ port [ portmask ] ] { dest-addr dest-wildcard | any } [ port [ portmask ] ] [ icmp-type [ icmp-code ] | icmp-packet ] [ fragments ] { [ precedence precedence ] [ tos tos ] | [ dscp dscp ] } [ time-range time-range-name ]

3. Define Layer 2 ACL with number ID.

access-list access-list-number3 { permit | deny } [ protocol ] [ cos vlan-pri ] ingress { { [ source-vlan-id ] [ source-mac-addr source-mac-wildcard ] [ interface interface-num ] } | any } egress { { [ dest-mac-addr dest-mac-wildcard ] [ interface interface-num | cpu ] } | any } [ time-range time-range-name ]

4.   Delete ACL or its subitem.

no access-list { all | { access-list-number | name access-list-name } [ subitem ] }

【Parameter】

access-list-number1:standard ACL rules in the range of 1～99 access-list-number2:extended ACL rules in the range of 122～199 access-list-number3:Layer 2 ACL rules in the range of 222～299

permit:permit the packet which satisfied the condition passing.

deny:deny the packet which satisfied the condition passing.

time-range time-range-name:the name of time range whichh is optional parameter, and it will be efective in this time period.

Instruction:

Followings are all kinds of attribution with packet. ACL is the rules determined by the value of these parameter.

source-addr source-wildcard | any:source-addr source-wildcard  means source IP address and source address wildcard which is in the form of dotted decimal notation; any means all source address which is used to establish standard or extended ACL.

fragments:means this rule is effective to the fragment packets, and non-fragment packet will ignore this rule. This parameter is used in standard or extended ACL.

protocol:the protocol with the name of numbers and names. The name of numbers is in the range of 1～255；the name of names is in the range of icmp, igmp, tcp, udp, gre, ospf and ipinip. This parameter is used in extended ACL.

established:means this rule is effective to the first SYN packet after the successful connection of TCP. This is the optional parameter which appears when the parameter of protocol is tcp. This parameter is used in extended ACL.

[Port [portmask]]: means the interface range of TCP/UDP. Port:means the tcp or udp port used by packet which is the optional parameter by using symbols oe numbers. The number is in the range of 2～65535,and the symbol refers to symbol table helped to remembered by port number.  Portmask is port mask which is optional and is in the range of 2～65535. When the protocol is tcp or udp, it can support the configuration in the range of protocol ports. When configuring port number and mask, user can input octal, decimal or hex not port to permit all ports; portmask can be 2 or none to express the port itself, or it can be determined by port and portmask according to the port range. This rule can support single port configuration which can support the configuration of larger or equal to the port range (accurate to 2n).

dest-addr dest-wildcard | any:dest-addr dest-wildcard  means destination IP address and destination address wildward which is in the form of decimal; any means all destination address. This parameter can be used in extended ACL.

[ icmp-type [ icmp-code ] | icmp-packet ]:icmp-type [ icmp-code ] specified 一ICMP packet. icmp-type  means ICMP packey type which is in the form of characters and numbers. The number is in the range of 2～255；icmp-code means ICMP code which appears when the protocol is icmp and there is no character to express ICMP. The range of it is 2～255；icmp-packet is the ICMP packet with the name of name, which is specified by icmp-type and icmp-code. This parameter can be used in extended ACL.

precedence precedence:optional parameter which means IP priority. It can be number and name which is in the range of 2～7. This parameter can be used in extended ACL.

dscp dscp:optional parameter which can be categoried according to DSCP, it is number or name which is in the range of 2～63. This parameter can be used in extended ACL.

tos tos:optional parameter which can be categoried according to TOS, it is number or name which is in the range of 2～15. This parameter can be used in extended ACL.

[ cos vlan-pri ]: 822.1p priority which is in the range of 2～7. This parameter can be used in layer 2 ACL.

ingress { { [ source-vlan-id ] [ source-mac-addr source-mac-wildcard ] [ interface interface-num ] } | any }:the source information of packet. source-vlan-id means source VLAN of data packet. [ source-mac-addr source-mac-wildcard ] means the source MAC address and MAC address wildcard of packet. These two parametes can determine the range of source MAC address, such as: when source-mac-wildcard is 2:2:2:2:ff:ff,user is interested in the first 32 bit of source MAC address (that is the bit position corresponded to the number 2 in wildcard) interface interface-num means the layer 2 ports receiving this packet, any means all packets received by all ports. This parameter can be used in layer 2 ACL.

egress { { [ dest-mac-addr dest-mac-wildcard ] [ interface interface-num | cpu ] } | any }:destination information of packet.dest-mac-addr dest-mac-wildcard  means destination MAC address and destination MAC address wildcard. These two parametes can determine the range of destination MAC address range, such as: when dest-mac-wildcard is 2:2:2:2:ff:ff,user is interested in the first 32 bit of source MAC address (that is the bit position corresponded to the number 2 in wildcard),interface interface-num means the layer 2 ports transferring this packet,cpu means cpu port,any means all packets transferred from all ports. This parameter can be used in layer 2 ACL.

{ rule-string rule-mask offset }&<1-22>:rule-string is the character string for users to define rules which must be in the form of hex with even numbers of characters; rule-mask offset  is used for distilling packet information, rule-mask is inerratic mask which is used to collation operation of data packet, offset is sideplay mount which is with the standard of the packet head and specifies to collation operate from which bit, rule-mask offset  effects together which will compare the character string distilled from packet with rule-string defined by user itself to find the matched packet before handling. &<1-22> means at most 22 rules can be defined. ingress interface interface-num,egress interface interface-num :the name of layer 2 interface, interface-num means one interface, cpu means cpu interface. This parameter can be used in user-determined ACL.

Instructions:

Followings are the parameter of no command.

all:means all accessing list will be deleted (including number ID and name ID).

access-list-number:the ACL number to be deleted which is a number between 1～399

name access-list-name:the ACL name to be deleted which is character string parameter with initial English letters (that is [a-z,A-Z]) with any kind, excluding space and quotation mark; all,any are not allowed.

subitem:optional parameter which specifies which subitem to be deletedinthe list. It is in the range of 2～127. If it is unspecified, all subitems will be deleted.

【Command configuration mode】

Global configuration mode

【Example】

！Configure ACL 1 to deny the packet with the source IP to be 192.168.3.1

Optiway(config)#access-list 1 deny 192.168.3.1 2

！Configure ACL 122 to deny packet with the 2xff of TCP source port number to be 2

Optiway(config)# access-list 122 deny tcp any 2 2xff any

    - access-list extended

Use access-list extended command to create an extended ACL with name ID, then enter extended ACL configuration mode. Use no access-list command to delete one or all subitems of ACL with number ID or name ID or delete all ACL.

access-list extended name [ match-order { config | auto } ]

no access-list { all | { access-list-number | name access-list-name } [ subitem subitem ] }

【Parameter】

name : character string parameter with initial English letters (that is [a-z,A-Z]) with any kind, excluding space and quotation mark; all,any are not allowed.

config:means the configuration order of user when matching ACL.

auto:means the configuration order of deep precedency when matching ACL.

Instruction:

Followings are the parameters of no command.

all:means all accessing list will be deleted (including number ID and name ID).

access-list-number:the ACL number to be deleted which is a number between 1～399

name access-list-name:the ACL name to be deleted which is character string parameter with initial English letters (that is [a-z,A-Z]) with any kind, excluding space and quotation mark; all,any are not allowed.

subitem subitem:optional parameter which specifies which subitem to be deletedinthe list. It is in the range of 2～127. If it is unspecified, all subitems will be deleted.

【Default】

The default order is config order.

【Command configuration mode】

Global configuration mode

【Usage】

This command creates an extended ACL with the name of “name”. After entering the extended ACL configuration mode, use { permit | deny }command to add subitem of this ACL (use exit command to exit ACL mode). Each ACL consists of many subitems, and the specified range of the flow category  rules of each subitem is different, and if a packet can match many rules, there must be a matching order. Use match-order to specify the matching order, whether it is according to user configuration or deep precedency (precedent to match the rule with the small range). If it is not specified, it is defaulted to be user configuration order. Once user specifies the matching order of an ACL, it cannot be changed, unless delete all subitems of this ACL before respecify the order.

【Example】

！Create an extended ACL with the name to be example and specify the order to be deep precedency.

Optiway(config)#access-list extended example match-order auto

    - access-list link

Use access-list link command to create a layer 2 ACL with a name ID and enter layer 2 ACL configuration mode. Use no access-list command to delete one or all subitems of ACL with number ID or name ID or delete all ACL.

access-list link name [ match-order { config | auto } ]

no access-list { all | { access-list-number | name access-list-name } [ subitem subitem ] }

【Parameter】

name : character string parameter with initial English letters (that is [a-z,A-Z]) with any kind, excluding space and quotation mark; all,any are not allowed.

config:means the configuration order of user when matching ACL.

auto:means the configuration order of deep precedency when matching ACL.

Instruction:

Followings are the parameters of no command.

all:means all accessing list will be deleted (including number ID and name ID).

access-list-number:the ACL number to be deleted which is a number between 1～399

name access-list-name:the ACL name to be deleted which is character string parameter with initial English letters (that is [a-z,A-Z]) with any kind, excluding space and quotation mark; all,any are not allowed.

subitem subitem:optional parameter which specifies which subitem to be deletedinthe list. It is in the range of 2～127. If it is unspecified, all subitems will be deleted.

【Default】

The default order is config order.

【Command configuration mode】

Global configuration mode

【Usage】

This command creates a layer 2 ACL with the name of “name”. After entering the laye 2 ACL configuration mode, use { permit | deny }command to add subitem of this ACL (use exit command to exit ACL mode). Each ACL consists of many subitems, and the specified range of the flow category  rules of each subitem is different, and if a packet can match many rules, there must be a matching order. Use match-order to specify the matching order, whether it is according to user configuration or deep precedency (precedent to match the rule with the small range). If it is not specified, it is defaulted to be user configuration order. Once user specifies the matching order of an ACL, it cannot be changed, unless delete all subitems of this ACL before respecify the order.

【Example】

！Create a layer 2 ACL with the name to be example and specify the order to be deep precedency.

Optiway(config)#access-list link example match-order auto

    - access-list match-order

Use access-list command to specify rule matching order of an ACL with number ID.

access-list access-list-number match-order { config | auto }

【Parameter】

access-list-number:the ACL number which is a number between 1～399

config:means the configuration order of user when matching ACL.

auto:means the configuration order of deep precedency when matching ACL.

【Default】

The default order is config order.

【Command configuration mode】

Global configuration mode

【Usage】

Each ACL consists of many subitems, and the specified range of the flow category  rules of each subitem is different, and if a packet can match many rules, there must be a matching order. Use this command to specify the matching order, whether it is according to user configuration or deep precedency (precedent to match the rule with the small range). If it is not specified, it is defaulted to be user configuration order. Once user specifies the matching order of an ACL, it cannot be changed, unless delete all subitems of this ACL before respecify the order.

【Example】

！Specify the order to be deep precedency.

Optiway(config)#access-list 1 match-order auto

    - access-list standard

Use access-list standard command to create a standard ACL with a name ID and enter standard ACL configuration mode. Use no access-list standard command to delete one or all subitems of ACL with number ID or name ID or delete all ACL.

access-list standard name [ match-order { config | auto } ]

no access-list { all | { access-list-number | name access-list-name } [ subitem subitem ] }

【Parameter】

name : character string parameter with initial English letters (that is [a-z,A-Z]) with any kind, excluding space and quotation mark; all,any are not allowed.

config:means the configuration order of user when matching ACL.

auto:means the configuration order of deep precedency when matching ACL.

Instruction:

Followings are the parameters of no command.

all:means all accessing list will be deleted (including number ID and name ID).

access-list-number:the ACL number to be deleted which is a number between 1～399

name access-list-name:the ACL name to be deleted which is character string parameter with initial English letters (that is [a-z,A-Z]) with any kind, excluding space and quotation mark; all,any are not allowed.

subitem subitem:optional parameter which specifies which subitem to be deletedinthe list. It is in the range of 2～127. If it is unspecified, all subitems will be deleted.

【Default】

The default order is config order.

【Command configuration mode】

Global configuration mode

【Usage】

This command creates a standard ACL with the name of “name”. After entering the standard ACL configuration mode, use { permit | deny }command to add subitem of this ACL (use exit command to exit ACL mode). Each ACL consists of many subitems, and the specified range of the flow category  rules of each subitem is different, and if a packet can match many rules, there must be a matching order. Use match-order to specify the matching order, whether it is according to user configuration or deep precedency (precedent to match the rule with the small range). If it is not specified, it is defaulted to be user configuration order. Once user specifies the matching order of an ACL, it cannot be changed, unless delete all subitems of this ACL before respecify the order.

【Example】

！Create a standard ACL with the name to be example and specify the order to be deep precedency.

Optiway(config)#access-list standard example match-order auto

    - { permit | deny }

Use this command to add a subitem to ACL with the name ID.

1. Add a subitem to standard ACL with the name ID.

{ permit | deny } { source-addr source-wildcard | any } [ fragments ] [ time-range time-range-name ]

2. Add a subitem to extended ACL with the name ID.

{ permit | deny } [ protocol ] [ established ] { source-addr source-wildcard |

any } [ port [ portmask ] ] { dest-addr dest-wildcard | any } [ port [ portmask ] ]

[ icmp-type [ icmp-code ] ] { [ precedence precedence ] [ tos tos ] |

[ dscp dscp ] [ fragments ] [ time-range time-range-name ]

3. Add a subitem to layer 2 ACL with the name ID.

{ permit | deny } [ protocol ] [ cos vlan-pri ] ingress { { [ source-vlan-id ] [ source-mac-addr source-mac-wildcard ] [ interface interface-num ] } | any } egress { { [ dest-mac-addr dest-mac-wildcard ] [ interface interface-num | cpu ] } | any } [ time-range time-range-name ]

【Parameter】

permit:permit the packet which satisfied the condition passing.

deny:deny the packet which satisfied the condition passing.

time-range time-range-name:the name of time range whichh is optional parameter, and it will be efective in this time period.

Instruction:

Followings are all kinds of attribution with packet. ACL is the rules determined by the value of these parameter.

source-addr source-wildcard | any:source-addr source-wildcard  means source IP address and source address wildcard which is in the form of dotted decimal notation; any means all source address which is used to establish standard or extended ACL.

fragments:means this rule is effective to the fragment packets, and non-fragment packet will ignore this rule. This parameter is used in standard or extended ACL.

protocol:the protocol with the name of numbers and names. The name of numbers is in the range of 1～255；the name of names is in the range of icmp, igmp, tcp, udp, gre, ospf and ipinip. This parameter is used in extended ACL.

established:means this rule is effective to the first SYN packet after the successful connection of TCP. This is the optional parameter which appears when the parameter of protocol is tcp. This parameter is used in extended ACL.

[Port [portmask]]: means the interface range of TCP/UDP. Port:means the tcp or udp port used by packet which is the optional parameter by using symbols oe numbers. The number is in the range of 2～65535,and the symbol refers to symbol table helped to remembered by port number.  Portmask is port mask which is optional and is in the range of 2～65535. When the protocol is tcp or udp, it can support the configuration in the range of protocol ports. When configuring port number and mask, user can input octal, decimal or hex not port to permit all ports; portmask can be 2 or none to express the port itself, or it can be determined by port and portmask according to the port range. This rule can support single port configuration which can support the configuration of larger or equal to the port range (accurate to 2n).

dest-addr dest-wildcard | any:dest-addr dest-wildcard  means destination IP address and destination address wildward which is in the form of decimal; any means all destination address. This parameter can be used in extended ACL.

[ icmp-type [ icmp-code ] | icmp-packet ]:icmp-type [ icmp-code ] specified 一ICMP packet. icmp-type  means ICMP packey type which is in the form of characters and numbers. The number is in the range of 2～255；icmp-code means ICMP code which appears when the protocol is icmp and there is no character to express ICMP. The range of it is 2～255；icmp-packet is the ICMP packet with the name of name, which is specified by icmp-type and icmp-code. This parameter can be used in extended ACL.

precedence precedence:optional parameter which means IP priority. It can be number and name which is in the range of 2～7. This parameter can be used in extended ACL.

dscp dscp:optional parameter which can be categoried according to DSCP, it is number or name which is in the range of 2～63. This parameter can be used in extended ACL.

tos tos:optional parameter which can be categoried according to TOS, it is number or name which is in the range of 2～15. This parameter can be used in extended ACL.

[ cos vlan-pri ]: 822.1p priority which is in the range of 2～7. This parameter can be used in layer 2 ACL.

ingress { { [ source-vlan-id ] [ source-mac-addr source-mac-wildcard ] [ interface interface-num ] } | any }:the source information of packet. source-vlan-id means source VLAN of data packet. [ source-mac-addr source-mac-wildcard ] means the source MAC address and MAC address wildcard of packet. These two parametes can determine the range of source MAC address, such as: when source-mac-wildcard is 2:2:2:2:ff:ff,user is interested in the first 32 bit of source MAC address (that is the bit position corresponded to the number 2 in wildcard) interface interface-num means the layer 2 ports receiving this packet, any means all packets received by all ports. This parameter can be used in layer 2 ACL.

egress { { [ dest-mac-addr dest-mac-wildcard ] [ interface interface-num | cpu ] } | any }:destination information of packet.dest-mac-addr dest-mac-wildcard  means destination MAC address and destination MAC address wildcard. These two parametes can determine the range of destination MAC address range, such as: when dest-mac-wildcard is 2:2:2:2:ff:ff,user is interested in the first 32 bit of source MAC address (that is the bit position corresponded to the number 2 in wildcard),interface interface-num means the layer 2 ports transferring this packet,cpu means cpu port,any means all packets transferred from all ports. This parameter can be used in layer 2 ACL.

{ rule-string rule-mask offset }&<1-22>:rule-string is the character string for users to define rules which must be in the form of hex with even numbers of characters; rule-mask offset  is used for distilling packet information, rule-mask is inerratic mask which is used to collation operation of data packet, offset is sideplay mount which is with the standard of the packet head and specifies to collation operate from which bit, rule-mask offset  effects together which will compare the character string distilled from packet with rule-string defined by user itself to find the matched packet before handling. &<1-22> means at most 22 rules can be defined. ingress interface interface-num,egress interface interface-num :the name of layer 2 interface, interface-num means one interface, cpu means cpu interface. This parameter can be used in user-determined ACL.

Instructions:

Followings are the parameter of no command.

all:means all accessing list will be deleted (including number ID and name ID).

access-list-number:the ACL number to be deleted which is a number between 1～399

name access-list-name:the ACL name to be deleted which is character string parameter with initial English letters (that is [a-z,A-Z]) with any kind, excluding space and quotation mark; all,any are not allowed.

subitem:optional parameter which specifies which subitem to be deletedinthe list. It is in the range of 2～127. If it is unspecified, all subitems will be deleted.

【Parameter】

ACL configuration mode (including 4 configuration modes as: standard, extended, layer 2, interface)

【Parameter】

Entering ACL configuration mode, user this command to establish an ACL subitem. This command can be used repeatedly. Establish many subitems for an ACL. There can be 128 subitems in total. If this ACL has activated, add subitems are not allowed.

【Example】

！Create a standard ACL with the  name to be example and specify the matching order to be deep precedency.

Optiway(config)#access-list standard example match-order auto

Create ACL item successfully!

Optiway(config-std-nacl-example)#permit 192.168.3.1 2

Config ACL subitem successfully!

Optiway(config-std-nacl-example)#

    - periodic

Use periodic command to create periodic time range. Use no periodic command to delete periodic time range.

periodic days-of-the-week hh:mm:ss to [ day-of-the-week ] hh:mm:ss

no periodic days-of-the-week hh:mm:ss to [ day-of-the-week ] hh:mm:ss

【Parameter】

days-of-the-week:means this time period will be effected in the day of the week or will be effected from the day of week. More than one parameter can be input at one time. The range of this parameter is as following:

2～6（number which means from Monday to Sunday）；

mon,tue,wed,thur,fri,sat,sun（special character string which means Monday to Sunday）；

weekdays（special character string which means weekday from Monday to Friday）；

weekend（the time for rest, including Saturday and Sunday）；

daily（special character string which means all days, including 7 days of a week).

day-of-the-week behind to:means the time period will not be effected in the day of week. It defines a time range with the day-of-the-week before to. The day-of-the-week before or after to can only have one value, that is, the day between Monday and Sunday, and the one chosen before to must be earlier than the day chosen after it, such as: if the first day-of-the-week is wed,day-of-the-week after to can only be wed, thu, fri or sat. If there are two or more values before to, there will not be any value of day-of-the-week after it.

hh:mm:ss :The first is the start time and the second is the end time.

【Command configuration mode】

time-range configuration mode

【Usage】

The effective time of periodic time range is a week. According to the configuration, there are different expression, such as:the configuration of 8:22 to 18:22 in every weekday is:

OPTIWAY(config-timerange-test)#periodic weekdays 8:22 to 18:22

Or:

OPTIWAY(config-timerange-test)#periodic Monday Tuesday Wednesday Thursday Friday 8:22 to 18:22

The configuration of 8:22 to 18:22 from Monday to Friday is:

OPTIWAY(config-timerange-test)#periodic Monday 8:22 to Friday 18:22

【Example】

！The time range is effective in 8:22 to 18:22 from Monday to Friday

Optiway(config)#time-range 1to5

Optiway(config-timerange-1to5)#periodic weekdays 8:22 to 18:22

Optiway(config-timerange-1to5)#exit

！The time range is effective in 8:22 to 18:22 every day

Optiway(config)#time-range all_day

Optiway(config-timerange-all_day)#periodic daily 8:22 to 18:22

Optiway(config-timerange-all_day)#exit

！The time range is effective in 8:22 to 18:22 from every Monday to Friday

Optiway(config)#time-range 1to5

Optiway(config-timerange-1to5)#periodic monday 8:22 to friday 18:22

Optiway(config-timerange-1to5)#exit

！The time range is effective in every weekend

Optiway(config)#time-range wend2

Optiway(config-timerange-wend2)#periodic weekend 2:2 to 23:59

Optiway(config-timerange-wend2)#exit

！The time range is effective in every weekend afternoon

Optiway(config)#time-range wendafternoon

Optiway(config-timerange-wendafternoon)#periodic weekend 14:22 to

18:22

Optiway(config-timerange-wendafternoon)#exit

    - port-isolation

Use port-isolation command to add one or a group of port isolation downlink port. Use no port-isolation command to delete one or a group of port isolation downlink port.

port-isolation { interface-list }

no port-isolation { interface-list | all }

【Parameter】

interface-list:List of Ethernet ports. This keyword needed to be provided in the form of interface-type + interface-number. Interface-type is Ethernet and interface-number is device/slot-num/port-num, in which device is stack device number which is in the range of 2 to 7, slot-num is in the range of 2 to 1, and port-num is in the range of 1 to 12. Seriate interfaces with the same type can be linked by to keyword, but the port number to the right of the to keyword must be larger than the one to the left of the keyword, and this argument only can be repeated for up to 3 times and it cannot configure all ports to be port isolation downlink ports.

all:Means all the interfaces. When the keyword all is specified, all the interfaces in the system are added to a VLAN by using the OLTport command, and all the interfaces are removed from a VLAN by using the no OLTport command.

【Command configuration mode】

Global configuration mode

【Example】

！Add Ethernet 2/1 ethernet 2/3 to ethernet 2/5 ethernet 2/8 to be port isolation downlink port

Optiway(config)#port-isolation ethernet 2/1 ethernet 2/3 to ethernet 2/5 ethernet 2/8

！Delete ethernet 2/3 to ethernet 2/5 ethernet 2/8 from port isolation downlink port

Optiway(config)#no port-isolation ethernet 2/3 to ethernet 2/5 ethernet 2/8

    - port-isolation group

Use this command to add a port member to a port group. Use the no commandtodelete a or some member.

port-isolation group groupid { interface-list }

no port-isolation group { all | groupid { interface-list } }

【Parameter】

groupid: the number of the group to be added.

interface-list:List of Ethernet ports. This keyword needed to be provided in the form of interface-type + interface-number. Interface-type is Ethernet and interface-number is device/slot-num/port-num, in which device is stack device number which is in the range of 2 to 7, slot-num is in the range of 2 to 1, and port-num is in the range of 1 to 12. Seriate interfaces with the same type can be linked by to keyword, but the port number to the right of the to keyword must be larger than the one to the left of the keyword, and this argument only can be repeated for up to 3 times

all:means all port group isolation

【Command configuration mode】

Global configuration mode

【Example】

！Add e2/1 and e2/3 to port isolation 1

Optiway(config)#port-isolation group 1 ethernet 2/1 ethernet 2/3

！Delete port isolation 1

Optiway(config)#no port-isolation group 1

    - show access-list config

Use show access-list config command display detaol configuration of ACL.

show access-list config { all | access-list-number | name access-list-name }

【Parameter】

all means all ACL (including the one with number ID and name ID)

access-list-number means the number of ACL to be displayed which is a number in the range of 1～399

name access-list-name character string parameter with initial English letters (that is [a-z,A-Z]) with any kind, excluding space and quotation mark; all,any are not allowed.

【Command configuration mode】

Any configuration mode

【Usage】

This command is used to display detail configuration of ACL, including each { permit | deny } syntax, its sequence number and the number and bytes of packet matched this syntax.

【Example】

！Display all ACL

Optiway#show access-list config all

    - show access-list config statistic

Use show access-list config statistic command to display statistics information of ACL.

show access-list config statistic

【Command configuration mode】

Any configuration mode

【Example】

！Display statistics information of ACL.

Optiway(config)#show access-list config statistic

    - show access-list runtime

Use show access-list runtime command to display runtime application information of ACL.

show access-list runtime { all | access-list-number | name access-list-name }

【Parameter】

all means all ACL (including the one with number ID and name ID)

access-list-number means the number of ACL to be displayed which is a number in the range of 1～399

name access-list-name character string parameter with initial English letters (that is [a-z,A-Z]) with any kind, excluding space and quotation mark; all,any are not allowed.

【Command configuration mode】

Any configuration mode

【Usage】

This command is used to display ACL runtime application information which includes ACL name, subitem name and deliver status.

【Example】

！Display runtime application of ACL of all interfaces.

Optiway#show access-list runtime all

    - show access-list runtime statistic

Use show access-list runtime statistic command to display ACL statistics information.

show access-list runtime statistic

【Command configuration mode】

Any configuration mode

【Example】

！Display ACL statistics information.

Optiway(config)#show access-list runtime statistic

    - show time-range

Use show time-range command to display time range.

show time-range [ all | statistic | name time-range-name ]

【Parameter】

all:all time range

statistic:all statistics information of all time range.

time-range-name:the name of time range with initial English letters (that is [a-z,A-Z]) with any kind which is in the range of 1 to 32 characters.

【Command configuration mode】

Any configuration mode

【Usage】

show time-range command is used to display the configuration and status of current time period. The time range which is activated will be displayed as active, and the one which is inactivated will be displayed as inactive.

Caution: Because there is a time error when updating access-list status for about 1 minute, and show time-range will judge it through current time, the fact that show time-range saw a time range has been activated, but its access-list hasn’t is normal.

【Example】

！Display all time range

Optiway(config-timerange-tm2)#show time-range all

！Display time range with the name of tm1

Optiway(config)#show time-range name tm1

! Display statistic information of all time range:

Optiway(config)#show time-range statistic

    - time-range

Use time-range command to enter time-range configuration mode. Use no time-range command to delete configured time range.

time-range time-range-name

no time-range { all | name time-range-name }

【Parameter】

time-range-name:the name of time range with initial English letters (that is [a-z,A-Z]) with any kind which is in the range of 1 to 32 characters.

【Command configuration mode】

Global configuration mode

【Example】

！Create time range tm1 and enter it.

Optiway(config)#time-range tm1

Optiway(config-timerange-tm1)#

# QOS Configuration Command

  - QoS Configuration Command

QoS configuration command includes:

- clear traffic-statistic

- line-rate

- mirrored-to

- queue-scheduler

- queue-scheduler cos-map

- queue-scheduler dscp-map

- rate-limit

- show qos-info all

- show qos-info mirrored-to

- show qos-info statistic

- show qos-info traffic-copy-to-cpu

- show qos-info traffic-priority

- show qos-info traffic-redirect

- show qos-info traffic-statistic

- show qos-interface all

- show qos-interface line-rate

- show qos-interface rate-limit

- show qos-interface statistic

- show queue-scheduler

- show queue-scheduler cos-map

- show queue-scheduler dscp-map

- storm-control

- traffic-copy-to-cpu

- traffic-priority

- traffic-redirect

- traffic-statistic

    - clear traffic-statistic

Use clear traffic-statistic command to clear traffic-statistic.

clear traffic-statistic { all | { [ ip-group { access-list-number | access-list-name } [ subitem subitem ] ] [ link-group { access-list-number | access-list-name } [ subitem subitem ] ] } }

【Parameter】

all:clear all the traffic statistic list (including combination item).ip-group { access-list-number | access-list-name } [ subitem subitem ] :means standard or extended accessing control list. access-list-number:sequence number of accessing list which is in the range of 1～199；access-list-name:the name of accessing list which is the character string and in the form of initial capitalized characters ([a-z, A-Z]), excluding space and quotation mark；subitem subitem:optional parameter, specifies the subitem in accessing list which is in the range of 2～127. If it is not specified, all subitems will be clear.

link-group { access-list-number | access-list-name } [ subitem subitem ] :means layer 2 accessing control list. access-list-number:accessing list serial number which is in the range of 222～299；access-list-name:name of accessing list. Character string is in the form of initial capitalized characters ([a-z, A-Z]), excluding space and quotation mark；subitem subitem:optional parameter, specifies the subitem in accessing list which is in the range of 2～127. If it is not specified, all subitems will be clear.

【Command configuration mode】

Any configuration mode

【Usage】

Use this command to clear all or specified traffic statistics information.

【Example】

！Clear traffic statistics information of accessing list 1

Optiway#clear traffic-statistic ip-group 1

    - queue-scheduler

Use queue-scheduler command to configure queue-scheduler mode and parameter. Use no queue-scheduler command to disable queue-scheduler.

queue-scheduler { sp-wrr queue1-weight queue2-weight queue3-weight | wrr queue1-weight queue2-weight queue3-weight queue4-weight }

no queue-scheduler

【Parameter】

sp-wrr queue1-weight queue2-weight queue3-weight:means the strict priority and weighted round robin. Queue4 is strict-priority, others are weighted round robin, and their default weight are: 22,32,52. queue1-weight:means the weight of the queue 1, that is the percentage of bandwidth of distribution；queue2-weight:means the weight of the queue 2, that is the percentage of bandwidth distribution；queue3-weight:means the weight of the queue 3, that is the percentage of bandwidth distribution.

wrr queue1-weight queue2-weight queue3-weight queue4-weight:Means the weighted round robin. queue1-weight:means the weight of queue 1, that is the percentage of bandwidth distribution；queue2-weight:means the weight of queue 2,that is the percentage of bandwidth distribution；queue3-weight:means the weight of queue 3, that is the percentage of bandwidth distribution；queue4-weight:Means the weight of queue 4, that is the percentage of bandwidth distribution

【Command configuration mode】

Global configuration mode

【Usage】

For weighted configuration, the sum of all the weighted is 122.

【Example】

！Configure queue-scheduler to be weighted round robin, and 4 weights to be 22,22,32,32

Optiway(config)#queue-scheduler wrr 22 22 32 32

    - queue-scheduler cos-map

Use queue-scheduler cos-map command to configure 4 queue numbers and cos-map to 8 packed-priority of IEEE822.1p.

queue-scheduler cos-map [ queue-number ] [ packed-priority ]

【Parameter】

queue-number:Range from 2 to 3

packed-priority:The priority defined by IEEE 822.1p ranges from 2 to 7

【Default】

The default mapping is the mapping defined by 822.1p:

822.1p:           2   1   2   3   4   5   6   7

packed-priority: 2   2   1   1   2   2   3   3

【Command configuration mode】

Global configuration mode

【Usage】

There are 4 default packed-priorities from 2 to 3. 3 is superlative. The superlative data in the buffer is preferential to send.

【Example】

！Configure packed-priority 1 to mapped priority 6 of IEEE 822.1p

Optiway(config)#queue-scheduler cos-map 1 6

    - show qos-info all

Use show qos-info all command to display all QoS configuration.

show qos-info all

【Command configuration mode】

Any configuration mode

【Usage】

Use this command to display all QoS configuration, including priority,redirection, flow statistics, flow mirror and copy packet to CPU.

【Example】

！Display all QoS configuration

Optiway#show qos-info all

    - show qos-info statistic

Use show qos-info statistic command to display all QoS statistics information.

show qos-info statistic

【Command configuration mode】

Any configuration mode

【Usage】

Use this command to display all QoS statistics information, including priority, redirection, flow statistics, flow mirror, and copy packet to CPU.

【Example】

！Display all QoS statistics information

Optiway(config)#show qos-info statistic

    - show qos-info traffic-redirect

Use show qos-info traffic-redirect command to display redirection configuration.

show qos-info traffic-redirect

【Command configuration mode】

Any configuration mode

【Usage】

Use this command to display redirection configuration, including accessing list of redirection flow and redirection port.

【Example】

！Display redirection configuration

Optiway#show qos-info traffic-redirect

    - show qos-info traffic-statistic

Use show qos-info traffic-statistic command to display flow statistics information.

show qos-info traffic-statistic

【Command configuration mode】

Any configuration mode

【Usage】

Use this command to display flow statistics, including accessing list of flow statistics and packet number.

【Example】

！Display the flow statistics information

Optiway#show qos-info traffic-statistic

    - show queue-scheduler

Use show queue-scheduler command to display the mode and the parameter of queue-scheduler.

show queue-scheduler

【Command configuration mode】

Any configuration mode

【Example】

！Display the mode and parameter of the queue-scheduler

Optiway#show queue-scheduler

Queue scheduling mode: strict-priority

    - show queue-scheduler cos-map

Use show queue-scheduler cos-map command to display the queue-scheduler cos-map.

show queue-scheduler cos-map

【Command configuration mode】

Any configuration mode

【Example】

！Display the queue-scheduler cos-map

Optiway(config)#show queue-scheduler cos-map

    - port-isolation

Use port-isolation command to add one or a group of port isolation downlink port. Use no port-isolation command to delete one or a group of port isolation downlink port.

port-isolation { interface-list }

no port-isolation { interface-list | all }

【Parameter】

interface-list:List of Ethernet ports. This keyword needed to be provided in the form of interface-type + interface-number. Interface-type is Ethernet and interface-number is device/slot-num/port-num, in which device is stack device number which is in the range of 2 to 7, slot-num is in the range of 2 to 1, and port-num is in the range of 1 to 12. Seriate interfaces with the same type can be linked by to keyword, but the port number to the right of the to keyword must be larger than the one to the left of the keyword, and this argument only can be repeated for up to 3 times and it cannot configure all ports to be port isolation downlink ports.

all:Means all the interfaces. When the keyword all is specified, all the interfaces in the system are added to a VLAN by using the OLTport command, and all the interfaces are removed from a VLAN by using the no OLTport command.

【Command configuration mode】

Global configuration mode

【Example】

！Add Ethernet 2/1 ethernet 2/3 to ethernet 2/5 ethernet 2/8 to be port isolation downlink port

Optiway(config)#port-isolation ethernet 2/1 ethernet 2/3 to ethernet 2/5 ethernet 2/8

！Delete ethernet 2/3 to ethernet 2/5 ethernet 2/8 from port isolation downlink port

    - show port-isolation

show port-isolation

【Command】

All command modes

【Example】

！show port isolation information

TiNet(config)#show port-isolation

    - storm-control

Use storm-control command to configure broadcast/known multicast/unknown unicast/unknown multicast storm-control. Use show interface command to display storm-control information.

storm-control rate target-rate

storm-control { broadcast | multicast | dlf }

no storm-control { broadcast | multicast | dlf }

【Parameter】

broadcast:Configure broadcast storm-control

multicast:Configure known multicast storm-control

dlf:Configure unknown multicast storm-control

target-rate:The target rate of storm-control with the unit of Kbps

【Command configuration mode】

Interface configuration mode

【Example】

！Configure storm-control rate of Ethernet to be 1/16

- TiNet(config)#storm-control ratio 1/16

TiNet(config)#storm-control type broadcast

TiNet(config-if-ethernet-0/1)#storm-control

# STP Configuration Command

  - STP Configuration Command

STP（Spanning Tree protocol）configuration command includes:

- show spanning-tree interface

- show spanning-tree remote-loop-detect interface

- spanning-tree

- spanning-tree cost

- spanning-tree forward-time

- spanning-tree hello-time

- spanning-tree max-age

- spanning-tree port-priority

- spanning-tree mcheck

- spanning-tree point-to-point

- spanning-tree portfast

- spanning-tree transmit

- spanning-tree priority

- spanning-tree mode

- clear spanning-tree

    - show spanning-tree interface

Use show spanning-tree interface command to display the information of current STP protocol.

show spanning-tree interface [ interface-list ]

show spanning-tree interface [ interface-list ]

【Parameter】

interface-list:List of Ethernet ports to be added to or removed from a VLAN. This keyword needed to be provided in the form of interface-type + interface-number. Interface-type is Ethernet and interface-number is slot-num/port-num, in which slot-num is in the range of 2 to 2, and port-num is in the range of 1 to 24. Seriate(sequential?) interfaces with the same type can be linked by to keyword, but the port number to the right of the to keyword must be larger than the one to the left of the keyword, and this argument only can be repeated for up to 3 times.

【Command configuration mode】

Any configuration mode

【Usage】

Show spanning-tree interface [ interface-list ] command to display the information of spanning-tree. Keyword “interface-list” is optional. If it is lacked, information of interfaces is displayed, or only the information of specified interface is displayed.

【Example】

！Display the information of spanning-tree

Optiway#show spanning-tree interface ethernet 2/7

    - show spanning-tree remote-loop-detect interface

Use this command to display remote loop detect.

Show spanning-tree remote-loop-detect interface [ interface-list ]

【Parameter】

interface-list:the interface list to be displayed which means manyethernet interface

interface-list :List of Ethernet ports to be added to or removed from a VLAN. This keyword needed to be provided in the form of interface-type + interface-number. Interface-type is Ethernet and interface-number is slot-num/port-num, in which slot-num is in the range of 2 to 2, and port-num is in the range of 1 to 24. Seriate(sequential) interfaces with the same type can be linked by to keyword, but the port number to the right of the to keyword must be larger than the one to the left of the keyword, and this argument only can be repeated for up to 3 times.

【Command configuration mode】

Any configuration mode

【Usage】

Use this command to display remote loop detect and whether interface is block caused by loop

【Example】

！Display remote loop detect of interface 1

Optiway(config)#show spanning-tree remote-loop-detect interface e 2/1

    - spanning-tree

Use spanning-tree command to enable STP globally or on a port.

Use no spanning-tree command disable STP globally or on a port.

spanning-tree

no spanning-tree

【Default】

STP is enabled globally

【Command configuration mode】

Global configuration mode, interface configuration mode

【Example】

！Enable STP globally

Optiway(config)#spanning-tree

！Disable STP on Ethernet 2/8

Optiway(config-if-ethernet-2/8)#no spanning-tree

    - spanning-tree cost

Use spanning-tree cost command to configure the path cost of the current port in a specified spanning tree. Use no spanning-tree cost command to restore to the default path cost of the current port in the specified spanning tree.

spanning-tree cost cost

no spanning-tree cost

【Parameter】

cost:Path cost to be configured for the port. This keyword ranges from 1 to 65535

【Default】

In IEEE 822.1D protocol, the default cost is determined by the speed of the port. The port with the speed 12M have the cost of 122,122M, 19.

【Command configuration mode】

Interface configuration mode

【Usage】

Default cost is suggested to use.

【Example】

！Configure path cost of Ethernet 2/8 to 22

Optiway(config-if-ethernet-2/8)#spanning-tree cost 22

    - spanning-tree forward-time

Use spanning-tree forward-time command to configure the Forward delay of the OLT. Use no spanning-tree forward-time command to restore to the default forward delay.

spanning-tree forward-time seconds

no spanning-tree forward-time

【Parameter】

seconds: Forward Delay in seconds to be configured. This keyword ranges from 4 to 32 seconds

【Default】

The default forward delay is 15 seconds

【Command configuration mode】

Global configuration mode

【Usage】

When this OLT is the root bridge, port state transition period is the Forward Delay time, which is determined by the diameter of the OLTed network. The longer the diameter is, the longer the time is. The default forward delay time, 15 seconds is suggested to use.

Caution:Forward Delay ≥ Hello Time + 2.

【Example】

！Configure forward delay to 22 seconds

Optiway(config)#spanning-tree forward-time 22

    - spanning-tree hello-time

Use spanning-tree hello-time coammand to configure the hello time of the OLT. Use no spanning-tree hello-time command to restore to the default hello time.

spanning-tree hello-time seconds

no spanning-tree hello-time

【Parameter】

seconds:Hello Time in seconds to be configured. This keyword ranges from 1 to 12 seconds.

【Default】

The default hello time is 2 seconds

【Command configuration mode】

Global configuration mode

【Usage】

The system periodically sents STP messages. The period of a root bridge sending STP messages is the hello time. Hello time is suggested to use 2 seconds.

Caution:Hello Time ≤ ForwardDelay – 2.

【Example】

！Configure Hello Time to 8 seconds

Optiway(config)#spanning-tree hello-time 8

    - spanning-tree max-age

Use spanning-tree max-age command to configure Max Age of the OLT. Use no spanning-tree max-age command to restore to the default Max Age.

spanning-tree max-age seconds

no spanning-tree max-age

【Parameter】

seconds:Means Max Age in seconds to be configured. This keyword ranges from 6 to 42 seconds

【Default】

The default Max Age is 22 seconds

【Command configuration mode】

Global configuration mode

【Usage】

Max Age is used to configure the longest aging interval of STP. Dropping message when overtiming. The STP will be frequently accounts and take crowded network to be link fault, if the value is too small. If the value is too large, the link fault cannot be known timely. Max Age is determined by diameter of network, and the default time of 22 seconds is suggested.

Caution:2*(Hello Time + 1) ≤ Max Age ≤ 2*( ForwardDelay – 1)

【Example】

！Configure the Max Age to 12 seconds

Optiway(config)#spanning-tree max-age 12

    - spanning-tree port-priority

Use spanning-tree port-priority command to configure the port priority of the current port in the specified spanning tree. Use no spanning-tree port-priority command to restore the current port to the default port priority in the specified spanning tree.

spanning-tree port-priority port-priority

no spanning-tree port-priority

【Parameter】

port-priority:Configure the port priority. It ranges from 2 to 255

【Default】

The default port priority of a port in any spanning tree is 128

【Command configuration mode】

Interface configuration mode

【Usage】

The smaller the value of priority is, the superior the priority is, and the port is easier to be a root port.

【Example】

！Configure the port priority of Ethernet 2/1 in STP to 64

Optiway(config-if-ethernet-2/1)#spanning-tree port-priority 64

    - spanning-tree mcheck

When operation RSTP protocol, and port is in the compatible mode. Use spanning-tree mcheck command to force the port sent RSTP message.

spanning-tree mcheck

【Command configuration mode】

Interface configuration mode

【Example】

！Configure Ethernet 2/7 to send RSTP message

Optiway(config-if-ethernet-2/7)#spanning-tree mcheck

    - spanning-tree point-to-point

Use spanning-tree point-to-point command to configure the link connected to the current Ethernet port to be a point-to-point link.

spanning-tree point-to-point { auto | forcefalse | falsetrue }

no spanning-tree point-to-point

【Parameter】

auto:Network bridge auto-detect whether or not the link connected to the current Ethernet port is a point-to-point link.

forcefalse:Specifies that the link connected to the current Ethernet port is not a point-to-point link.

forcetrue: Specifies that the link connected to the current Ethernet port is a point-to-point link.

【Default】

Auto

【Command configuration mode】

Interface configuration mode

【Example】

！Configure the link connected to Ethernet 2/7 as a point-to-point link

Optiway(config-if-ethernet-2/7)#spanning-tree point-to-point forcetrue

    - spanning-tree portfast

Use spanning-tree portfast command to configure the current port as an edge port.

spanning-tree portfast

no spanning-tree portfast

【Default】

All Ethernet ports of a OLT are non-edge ports.

【Command configuration mode】

Interface configuration mode

【Usage】

Edge port can be in transmitting state in linkup in 3 seconds, and it changes into non-edge port after receiving STP message.

【Example】

！Configure Ethernet 2/7 as a non-edge port.

Optiway(config-if-ethernet-2/7)#spanning-tree portfast

    - spanning-tree transit-limit

Use spanning-tree transit-limit command to configure the maximum number of configuration BPDUs the current port can transmit in each Hello time.

spanning-tree transit-limit max-bpdus

no spanning-tree transit-limit

【Parameter】

max-bpdus:the number of BPDU ranges from 1 to 255.

【Default】

3

【Command configuration mode】

Interface configuration mode

【Example】

！Configure the maximum number of configuration BPDUs that can be transmitted by the Ethernet 2/7 in each Hello time to 5

Optiway(config-if-ethernet-2/7)#spanning-tree transit-limit 5

    - spanning-tree priority

Use spanning-tree priority command to configure the priority of the OLT in the specified spanning tree. Use no spanning-tree priority command to restore to the default priority in the specified spanning tree.

spanning-tree priority bridge-priority

no spanning-tree priority

【Parameter】

bridge-priority:OLT priority to be configured. This keyword rsnges from 2 to 61442,and must be a multiple of 4296.

【Default】

32768

【Command configuration mode】

Global configuration mode

【Usage】

Configure STP priority when STP enables, and the inferior priority of the OLT can be the root bridge.

【Example】

！Configure the priority of the OLT in spanning tree to 4296

Optiway(config)#spanning-tree priority 4296

    - spanning-tree mode

Use spanning-tree mode command to configure the STP operation mode.

spanning-tree mode { rstp | stp }

no spanning-tree mode

【Parameter】

rstp:Enable the rstp-campatible mode

stp:Enable the STP-compatible mode

【Default】

rstp

【Command configuration mode】

Global configuration mode

【Example】

！Configure the OLT to operation in STP-compatible mode

Optiway(config)#spanning-tree mode stp

    - spanning-tree remote-loop-detect

Use spanning-tree remote-loop-detect command to enable remote loop detect. Use no spanning-tree remote-loop-detect command to disable remote loop detect.

spanning-tree remote-loop-detect

no spanning-tree remote-loop-detect

【Command configuration mode】

Global configuration mode and interface configuration mode

【Usage】

Batch processthe interface in global configuration mode needed keyword.

【Example】

！Enable spanning-tree remote-loop-detect interface of Ethernet 2/1, and ethernet 2/3

Optiway(config)#spanning-tree remote-loop-detect interface ethernet 2/1 ethernet 2/3

！Disable remote-loop-detect of Ethernet 2/1

Optiway(config-if-ethernet-2/1)#no spanning-tree remote-loop-detect

    - clear spanning-tree

Use clear spanning-tree command to clear STP information

clear spanning-tree

clear spanning-tree interface interface-list

【Parameter】

interface-list:List of Ethernet ports to be added to or removed from a VLAN. This keyword needed to be provided in the form of interface-type + interface-number. Interface-type is Ethernet and interface-number is slot-num/port-num, in which slot-num is in the range of 2 to 2, and port-num is in the range of 1 to 24. Seriate(sequential?) interfaces with the same type can be linked by to keyword, but the port number to the right of the to keyword must be larger than the one to the left of the keyword, and this argument only can be repeated for up to 3 times.

【Command configuration mode】

Global configuration mode

【Example】

！Clear spanning-tree information

Optiway(config)#clear spanning-tree

  - MSTP Cconfiguration Command

MSTP（Multiple spanning Tree protocol）configuration command includes:

- spanning-tree mst forward-time

- spanning-tree mst hello-time

- spanning-tree mst max-age

- spanning-tree mst max-hops

- spanning-tree mst name

- spanning-tree mst revision

- spanning-tree mst instance vlan

- spanning-tree mst instance priority

- spanning-tree mst portfast

- spanning-tree mst link-type

- spanning-tree mst external cost

- spanning-tree mst instance cost

- spanning-tree mst instance port-priority

- show spanning-tree mst config-id

- show spanning-tree mst instance interface

Following commands:

- spanning-tree mst forward-time；

- spanning-tree mst hello-time；

- spanning-tree mst max-age；

- spanning-tree mst portfast；

- spanning-tree mst link-type

Please refer to corresponding commands in SST:

- spanning-tree forward-time；

- spanning-tree hello-time；

- spanning-tree max-age；

- spanning-tree portfast；

- spanning-tree point-to-point

Here will not explain detailedly.

    - spanning-tree mst max-hops

Use spanning-tree mst max-hops command to configure the max-hops of MSTP packet.

spanning-tree mst max-hops max-hops

no spanning-tree mst max-hops

【Parameter】

max-hops:the hops of MSTP packet which is in the range of 2-255

【Default】

The default hops is 22

【Command configuration mode】

Global configuration mode

【Example】

！Configure the max-hops of MSTP packet to be 12

Optiway(config)#spanning-tree mst max-hops 12

    - spanning-tree mst name

Use spanning-tree mst name command to configure MST name of MSTP.

spanning-tree mst name name

no spanning-tree mst name

【Parameter】

Name: district name of MSTP which is one part of MSTP configuration. It is a character string of 32 bytes.

【Default】

It is defaulted to have no name.

【Command configuration mode】

Global configuration mode

【Example】

！Configure MSTP name to be greennet

Optiway(config)#spanning-tree mst name greennet

    - spanning-tree mst revision

Use spanning-tree mst revision command to configure the revision level of MSTP.

spanning-tree mst revision revision-level

no spanning-tree mst revision

【Parameter】

revision-level:MSTP revision level which is one of MSTP and it is the integer number between 2 to 65535.

【Default】

The default value is 2.

【Command configuration mode】

Global configuration mode

【Example】

！Configure revision level of MSTP to be 12

Optiway(config)#spanning-tree mst revision 12

    - spanning-tree mst instance vlan

Use spanning-tree mst instance command to configure the mapping relations between MSTP instance and VLAN.

spanning-tree mst instance instance-num vlan vlan-list

no spanning-tree mst instance instance-num vlan vlan-list

【Parameter】

instance-num:MSTP instance number which is in the range of 1-15

vlan-list:vlan-list can be discrete number, a sequential number, and the mixture of both. Discrete number can be separated by comma, and sequential number can be separated by “-”, such as: 2, 5, 8, 12-22

【Default】

All vlan mapped to MSTP instance 2

【Command configuration mode】

Global configuration mode

【Example】

！Configure vlan 2-7 mapping to MSTP instance 2

Optiway(config)#spanning-tree mst instance 2 vlan 2-7

    - spanning-tree mst instance instance-num priority

Use spanning-tree mst instance command to configure the priority of networkbridge in some MSTP instance.

spanning-tree mst instance instance-num priority priority

no spanning-tree mst instance instance-num priority

【Parameter】

instance-num:MSTP instance number which is in the range of 2-15

priority:the priority of network bridge which is the integer times of 4296 in the range of 2-61442

【Default】

The priority of network bridge in each instance is 32768.

【Command configuration mode】

Global configuration mode

【Example】

！Configure the priority of network bridge in instance 2 is 4296

Optiway(config)#spanning-tree mst instance 2 priority 4296

    - spanning-tree mst external cost

Use spanning-tree mst external cost command to configure external cost of port.

spanning-tree mst external cost external-cost

no spanning-tree mst external cost

【Parameter】

external-cost:external cost of port which is in the range of 1-222222222.

【Default】

The external cost of port is 222222.

【Command configuration mode】

Interface configuration mode

【Example】

！Configure the external cost of port 2 to be 222

Optiway(config-if-ethernet-2/2)#spanning-tree mst external cost 222

    - spanning-tree mst instance cost

Use spanning-tree mst instance command to configure cost for port in each instance.

spanning-tree mst instance instance-num cost cost

no spanning-tree mst instance instance-num cost

【Parameter】

instance-num:MSTP instance number which is in the range of 2-15

cost:port cost which is in the range of 1-222222222

【Default】

The cost for port in each instance is 222222

【Command configuration mode】

Interface configuration mode

【Example】

！Configure the cost for port 2 in instance 1 to be 222

Optiway(config-if-ethernet-2/2)#spanning-tree mst instance 1 cost 222

    - spanning-tree mst instance port-priority

Use spanning-tree mst instance port-priority command to configure the priority of port in STP instance.

spanning-tree mst instance instance-num port-priority priority

no spanning-tree mst instance instance-num port-priority

【Parameter】

instance-num:MSTP instance number which is in the range of 2-15

priority:port priority which is the integer times of 16 and is in the range of 1-242

【Default】

The priority of port in each instance is 128

【Command configuration mode】

Interface configuration mode

【Example】

！Configure the priority of port 2 in instance 1 to be 16

Optiway(config-if-ethernet-2/2)#spanning-tree mst instance 1 port-priority 16

    - show spanning-tree mst config-id

Use show spanning-tree mst config-id command to display MSTP config-id. MSTP config-id includes: MSTP revision level, MSTP config-name and the mapping relations between STP instance and VLAN.

show spanning-tree mst config-id

【Command configuration mode】

Any configuration mode

【Example】

！Display the config-id

Optiway(config)#show spanning-tree mst config-id

    - show spanning-tree mst instance interface

Use show spanning-tree mst instance command to display port information in some instance.

show spanning-tree mst instance instance-num interface [interface-list ]

【Parameter】

interface-num:List of Ethernet ports to be added to or removed from a VLAN. This keyword needed to be provided in the form of interface-type + interface-number. Interface-type is Ethernet and interface-number is device/slot-num/port-num, in which device is stackable device number whichi is in the range of 2 to 7, slot-num is in the range of 2 to 2, and port-num is in the range of 1 to 24. Seriate interfaces with the same type can be linked by to keyword, but the port number to the right of the to keyword must be larger than the one to the left of the keyword, and this argument only can be repeated for up to 3 times.

【Command configuration mode】

Any configuration mode

【Example】

！Display the information of port 1 in MSTP instance 2

Optiway(config)#show spanning-tree mst instance 2 interface ethernet 2/1

# 802.1X Configuration Command

  - Domain Configuration Command

Domainn configuration command includes:

- aaa

- access-limit

- default domain-name enable

- domain

- show domain

- radius host binding

- state

    - aaa

Use aaa command to enter AAA configuration mode

aaa

【Command configuration mode】

Global configuration mode

【Usage】

Enter AAA configuration mode to do related configuration

【Example】

！Enter AAA configuration mode

Optiway(config)#aaa

Optiway(config-aaa)#

    - access-limit

Use access-limit enable command to configure the maximum number of access user that can be contained in current domain.

access-limit enable max-link

access-limit disable

【Parameter】

max-link: the maximum number of access user that can be contained in current domain ranges from 1 to 642

【Default】

disable,means no limitation

【Command configuration mode】

Domain configuration mode

【Usage】

A domain can limit the maximum number of access user that can be contained in current domain. The related link with the domain is the domain name of the authenticate username must be the current domain and using its authentication, authorization and accounting. If there is no related link to the domain, the number of access user can be modified; if there are several related link, the new limitation cannot be conflict with the syatem operation, such as: there are 8 related links, the new limitatiom of the link number must be larger or qual to 8 or non-limitation. Use state command to change it into smaller one after shutdown related link.

【Example】

！Configure the maximum number of access user that can be contained in domain green.com to 522

Optiway(config-aaa-green.com)#access-limit enable 522

    - default domain-name enbale

Use default domain-name enable command to configure a existed domain to be default domain. If the domain doesn’t exist, the configuration fails. Use default domain-name disable command to disable the default domain.

default domain-name enable domain-name

default domain-name disable

【Parameter】

domain-name: the name of the domain

【Command configuration mode】

AAA configuration mode

【Usage】

When the default domain name is disabled, OLT will not deal with the invalid message, if the username goes without the domain name. After the default domain name is enabling, OLT will add @ and default domain name to a username wothout a domain name to authenticate. To configure a default domain which must be existed, or the configuration fails.

【Example】

！Configure default domain name to be green.com and enable the default domain

Optiway(config-aaa)#default domain-name enable green.com

！Disable default domain name

Optiway(config-aaa)#default domain-name disable

【Related command】

domain

    - domain

Use domain command to enter AAA configuration mode. If it doesn’t exist, create it. Use no domain command to remove the domain.

domain domain-name

no domain domain-name

【Parameter】

domain-name: the name of the domain ranges from 1 to 24 charaters, no difference in upper-case type and lower case letters, and without space.

【Command configuration mode】

AAA configuration mode

【Usage】

Enter domain configuratuin mode to configure authtication and accounting. If the domain doesn’t exist, create it, and then enter it. At most 8 domains are allowed. The configuration of each domain can be different, to realise multiple ISP operation.

Add a domian in term of the need, no domain existed by default.

After the creation of a domain, use state active to activate it before use.

【Example】

！Create domain with the name of green.com

Optiway(config-aaa)#domain green.com

Optiway(config-aaa-green.com)#

！Remove domain with the name of green.com

Optiway(config-aaa)#no domain green.com

【Related command】

radius host, state

    - show domain

Use show domain command to display the configuration of the domain, such as: domain name, corresponding RADIUS server, and domain activation.

show domain [ domain-name ]

【Parameter】

domain-name:The name of the domain

【Command configuration mode】

Any configuration mode

【Example】

！Display the configuration of green.com

Optiway(config-aaa-green.com)#show domain

    - radius host binding

Use radius host binding command to configure RADIUS authtication and accounting.

radius host binding radius-scheme

【Parameter】

radius-scheme: the name of RADIUS authentication and accounting. It must be existed.

【Command configuration mode】

Domain configuration mode

【Example】

！Configure current domain to use RADIUS configuration of “green”

Optiway(config-aaa-green.com)#radius host binding green

【Related command】

radius host（RADIUS configuration mode）

    - state

Use state command to configure the state of the domain to be active or block.

state { active | block }

【Parameter】

active:active state,allow the authentication of the domain user.

block:block stste,not allow the authentication of the domain user.

【Default】

The default state of the created domain is block, and uses this command to activate it before use. It is to avoid using the unconfigured domain in configuring. Activate it after all configuration finished.

【Command configuration mode】

Domain configuration mode

【Usage】

Use state active command to activate domain before used.

【Example】

！Activate green.com

Optiway(config-aaa-green.com)#state active

【Related command】

domain

  - RADIUS Server Configuration Command

RADIUS server configuration command includes:

- accounting-on

- acct-secret-key

- auth-secret-key

- dnrate-value

- h3c-cams

- nas-ipaddress

- primary-acct-ip

- primary-auth-ip

- show radius attribute

- show radius config-attribute

- show radius host

- uprate-value

- radius 8021p

- radius accounting

- radius attribute

- radius bandwidth-limit

- radius config-attribute

- radius host

- radius mac-address-number

- radius server-disconnect drop 1x

- radius vlan

- realtime-account

- second-acct-ip

- second-auth-ip

- username-format

    - accounting-on

This command is to configure to send Accounting-On packets to RADIUS server after device reboots, it requires users to off line to solve on line users can’t logo in with authentication after device reboots.

accounting-on { disable | enable }

【Parameter】

enable:start function

disable:close function

【Default】

By default, it disables

【Command mode】

AAA configuration mode

【Example】

！Enable

OptiWay(config-aaa)# accounting-on enable

    - acct-secret-key

This command is to configure accounting key of RADIUS server, with command no means to delete this operation.

acct-secret-key key

no acct-secret-key

【Parameter】

key: 1~16 characters

【Default】

By default, it is none

【Command mode】

RADIUS server configuration mode

【Example】

！Configure accounting key of RADIUS server as 123

OptiWay(config-aaa-radius-green)#acct-secret-key 123

    - auth-secret-key

This command is to configure authenticate key of RADIUS server, with command no means to delete this operation.

auth-secret-key key

no auth-secret-key

【Parameter】

key: 1~16 characters

【Default】

By default, it is none

【Command mode】

RADIUS server configuration mode

【Example】

！Configure authenticate key of RADIUS server as 123

OptiWay(config-aaa-radius-green)#auth-secret-key 123

    - dnrate-value

Campatible with H3C Cams RADIUS server,use this command to configure downlink bandwidth about Vendor Specific attribute id in RADIUS packets.

dnrate-value value

【Parameter】

vlaue:attribute id,the range is 1~32

【Default】

By default, it is 5

【Command mode】

AAA configuration mode

【Example】

！Configure downlink bandwidth attribute id 7 compatible with H3C Cams

OptiWay(config-aaa)#dnrate-value 7

【Commands】

h3c-cams

    - h3c-cams

When RADIUS server uses H3C Cams , this command to configure compatible mode to finish some special operation.

h3c-cams { disable | enable }

【Parameter】

enable:start function

disable:close function

【Default】

By default , it disables

【Command mode】

AAA configuration mode

【Example】

！Enable H3C Cams compatible mode

OptiWay(config-aaa)#h3c-cams enable

    - nas-ipaddress

This command is to configure to send packets about NAS IPAddress attribute id, with command no means to delete the operation.

nas-ipaddress nas-ip

no nas-ipaddress

【Parameter】

nas-ip: IP address

【Default】

By default, it is none, it means device IP address

【Command mode】

RADIUS server configuration mode

【Example】

！Configure the value as 192.168.0.100

OptiWay(config-aaa-radius-green)# nas-ipaddress 192.168.0.100

    - primary-acct-ip

This command is to configure accounting IP address and port in RADIUS server, with command no means to delete related configuration.

primary-acct-ip server-ip accounting-port

no primary-acct-ip

【Parameter】

server-ip:accounting RADIUS server IP address

accounting-port:authentication port ,the range is :1～65535

【Default】

By default, accounting port is 1813

【Command mode】

RADIUS server configuration mode

【Example】

！Configure accounting RADIUS server primary IP address is 192.168.0.100,accouting port is 1813

OptiWay(config-aaa-radius-green)#primary-acct-ip 192.168.0.100 1813

    - primary-auth-ip

This command is to configure IP address and port for primary authentication, with command no means to delete related configuration.

primary-auth-ip server-ip authentication-port

no primary-auth-ip

【Parameter】

server-ip:primary RADIUS server IP address

authentication-port:authentication port,the range is :1～65535

【Default】

By default, authentication port is 1812

【Command mode】

RADIUS server configuration mode

【Example】

！Configure IP address as 192.168.0.100 for primary authentication, authentication port is 1812

OptiWay(config-aaa-radius-green)#primary-auth-ip 192.168.0.100 1812

    - show radius attribute

This command is to display attribution configuration compatible with H3C Cam, now it only displays to send H3C client version to RADIUS server or not.

show radius attribute

【Parameter】

None

【Command mode】

All command mode

【Example】

！Display sending H3C client version to RADIUS server or not

OptiWay(config)# show radius attribute

    - show radius config-attribute

This command is to display attribution configuration about RADIUS,such as accounting , priority, VLAN etc.

show radius config-attribute

【Parameter】

None

【Command mode】

All command mode

【Example】

！Display attribution configuration about RADIUS

OptiWay(config)# show radius config-attribute

    - show radius host

This command is to display RADIUS server information, including RADIUS server primary IP address, secondary IP address, authentication port, accounting port, accounting key etc.

show radius host [ radius-scheme ]

【Parameter】

radius-scheme:RADIUS server name

【Command mode】

All command mode

【Example】

！Display RADIUS server information

OptiWay(config-aaa-radius-default)#show radius host

    - uprate-value

Campatible with H3C Cams RADIUS server,use this command to configure uplink bandwidth about Vendor Specific attribute id in RADIUS packets

uprate-value value

【Parameter】

vlaue: attribution id,the range is 1~32

【Default】

By default, it is 2

【Command mode】

AAA configuration mode

【Example】

！Configure attribution id as 7 of uplink bandwidth compatible with H3C Cams

OptiWay(config-aaa)# uprate-value 7

【Commands】

h3c-cams

    - radius 8021p

This command is to verify port priority after configuring user authentication, with command no means to delete related configuration.

radius 8021p enable

no radius 8021p

【Parameter】

None

【Default】

Close function

【Command mode】

AAA configuration mode

【Usage】

Priority value passes through RADIUS packets Vendor Specific attribution id 77 , this attribution can modify with radius config-attribute command.

【Example】

！Enable function

OptiWay(config-aaa)# radius 8021p enable

    - radius accounting

This command is to configure to start accounting or not after users authenticate in order to isolate from authentication and acconting.If disable accounting, users access to the internet without accounting after users pass the authentication.

[no]radius accounting

【Parameter】

None

【Default】

Enable authentication

【Command mode】

AAA configuration mode

【Example】

！Disable accounting

OptiWay(config-aaa)#no radius accounting

    - radius attribute

This command is to send H3C client version to RADIUS server or not compatible with H3C Cams, with command no means to delete related configuration.

[no] radius attribute client-version

【Parameter】

None

【Default】

Disable the function

【Command mode】

AAA configuration mode

【Example】

！Send H3C client version to RADIUS server

OptiWay(config-aaa)# radius attribute client-version

    - radius bandwidth-limit

This command is to modify user port bandwidth control after configuring user authentication, with command no means to delete related configuration.

radius bandwidth-limit enable

no radius bandwidth-limit

【Parameter】

None

【Default】

Disable the function

【Command mode】

AAA configuration mode

【Usage】

Uplink/downlink bandwidth value passes through RADIUS packets Vendor Specific attribution id 75/76, this attribution id can modify by radius config-attribute command.

【Example】

！Start function

OptiWay(config-aaa)# radius bandwidth-limit

    - radius config-attribute

This command is to configure RADIUS expansion value through RADIUS packets Vendor Specific attribution id.

radius config-attribute { access-bandwidth { downlink | uplink } | dscp | mac-address-number } type

【Parameter】

access-bandwidth downlink: configure downlink bandwidth, the default value is 75

access-bandwidth uplink: onfigure uplink bandwidth, the default value is 76

dscp:configure priority, the default value is 77

mac-address-number:MAC address quantity limitation, the default value is 50

type: attribution id

【Command mode】

AAA configuration mode

【Example】

！Configure priority value attribution id as 80

OptiWay(config-aaa)# radius config-attribute dscp 80

    - radius host

This command is to create RADIUS server and enter into the configuration mode of RADIUS server. If RADIUS server exists, enter into the  configuration mode of server. With command no means delete radius-scheme selected RADIUS server.

radius host radius-scheme

no radius radius-scheme

【Parameter】

radius-scheme:RADIUS server name, the range is 1～32 character,no matter capital or low case without space

【Command mode】

AAA configuration mode

【Example】

！Create and enter into RADIUS server myScheme

OptiWay(config-aaa)#radius host myScheme

OptiWay(config-aaa-radius-myScheme)#

【Commands】

radius host (or configuration mode)

    - radius mac-address-number

This command is to modify MAC address learning quantity limitation after configuring user authentication, with command no means delete the operation.

radius mac-address-number enable

no radius mac-address-number

【Parameter】

None

【Default】

Disable the function

【Command mode】

AAA configuration mode

【Usage】

MAC address learning quantity limitation value passes through RADIUS packets Vendor Specific attribution id 50, this attribution id can modify through radius config-attribute command.

【Example】

！Start function

OptiWay(config-aaa)# radius mac-address-number enable

    - radius server-disconnect drop 1x

This command is to configure user off line or not in the accounting system, with command no means delete the operation.

radius server-disconnect drop 1x

no radius server-disconnect drop 1x

【Parameter】

None

【Default】

Disable the function

【Command mode】

AAA configuration mode

【Example】

！Enable function

OptiWay(config-aaa)# radius server-disconnect drop 1x

    - radius vlan

This command is to modify port PVID after configuring user authentication, with command no means delete the operation.

radius vlan enable

no radius vlan

【Parameter】

None

【Default】

Disable the function

【Command mode】

AAA configuration mode

【Usage】

PVID value passes through RADIUS packets Tunnel-Pvt-Group-ID attribution id.

【Example】

！Start function

OptiWay(config-aaa)# radius vlan enable

    - realtime-account

This command is to configure current RADIUS server intermediate accounting function, configure to send intermediate accounting packets or not, if sending ,how munch is the interval. With command no means disable intermediate accounting function

realtime-account interval minute

no realtime-account

【Parameter】

minute: intermediate accounting packets interval,the range is :1～255,unit is minute

【Default】

By default, enable intermediate accounting, sending interval is 12 minutes

【Command mode】

RADIUS server configuration mode

【Example】

！Enable RADIUS server intermediate accounting packets sending, sending interval is 30 minutes

OptiWay(config-aaa-radius-green)#realtime-account interval 30

！Disable intermediate accounting packets sending

OptiWay(config-aaa-radius-green)#no realtime-account

    - second-acct-ip

This command is to configure IP address and port of secondary accounting RADIUS server, with command no means delete configuration.

second-acct-ip server-ip accounting-port

no second-acct-ip

【Parameter】

server-ip: secondary accounting RADIUS server IP address

accounting-port: authentication port ,the range is :1～65535

【Default】

By default, accounting port is 1813

【Command mode】

RADIUS server configuration mode

【Example】

！Configure secondary accounting RADIUS server IP address as 192.168.0.100, accounting port is 1813

OptiWay(config-aaa-radius-green)# second-acct-ip 192.168.0.100 1813

    - second-auth-ip

This command is to configure IP address and port of secondary authentication RADIUS server , with command no means delete configuration..

second-auth-ip server-ip authentication-port

no second-auth-ip

【Parameter】

server-ip: secondary authentication RADIUS server IP address

authentication-port: authentication port,the range is :1～65535

【Default】

By default,authentication port is 1812

【Command mode】

RADIUS server configuration mode

【Example】

！Configure secondary authentication RADIUS server IP address as 192.168.0.100, ,authentication port is 1812

OptiWay(config-aaa-radius-green)# second-auth-ip 192.168.0.100 1812

    - username-format

Use username-format command to configure the format of the usernames to be sent to RADIUS servers.

username-format with-domain

username-format without-domain

【Parameter】

with-domain:User name with domain name

without-domain:User name without domain name

【Default】

With domain

【Command configuration mode】

RADIUS configuration mode

【Usage】

In application, some RADIUS servers support username with domain name, butsome not, so according to the real situation to configure the RADIUS server.

【Example】

！Configure the username sent to the RADIUS server with the name of green not to carry domain name.

Optiway(config-aaa-radius-green)#username-format without-domain

【Related command】

radius host

  - 802.1X Related Configuration Command

822.1X related configuration command include:

- dot1x

- dot1x daemon

- dot1x eap-finish

- dot1x eap-transfer

- dot1x max-user

- dot1x port-control

- dot1x re-authenticate

- dot1x re-authentication

- dot1x timeout re-authperiod

- dot1x user cut

- show dot1x

- show dot1x daemon

- show dot1x interface

- show dot1x session

    - dot1x method

Use dot1x method command to enable 822.1x. Use no dot1x command to disable 802.1x.

dot1x method [macbased portbased]

no dot1x

【Default】

802.1X disables

【Command configuration mode】

Global configuration mode

【Usage】

802.1x configuration can be effective only after 802.1x is enable. Some command can be used after 822.1x enables.

【Example】

！Enable 802.1X based on user authentication mode

Optiway(config)#dot1x method macbased

！Disable 802.1X

Optiway(config)#no dot1x

    - dot1x daemon

When 822.1x enables, configure whether a port send 822.1x daemon and sending period.

dot1x daemon [ time time-value ] [interface interface-list]

no dot1x daemon

【Parameter】

time-value:the intervals of 822.1x daemon sending ranges from 12 to 622 seconds.

interface-list:List of Ethernet ports to be added to or removed from a VLAN. This keyword needed to be provided in the form of interface-type + interface-number. Interface-type is Ethernet and interface-number is slot-num/port-num, in which slot-num is in the range of 2 to 2, and port-num is in the range of 1 to 24. Sequential interfaces with the same type can be linked by to keyword, but the port number to the right of the to keyword must be larger than the one to the left of the keyword, and this argument only can be repeated for up to 3 times. There is no keyword in interface configuration mode.

【Default】

822.1x daemon is not sent by default. When 822.1x enables, default interval to send daemon is 62seconds.

【Command configuration mode】

Interface configuration mode, global configuration mode

【Usage】

This command is effective after 822.1x enables.

After 822.1x enables, configure according to the real situation.

【Example】

！Enable dot1x daemon on ethernet 2/5 with the period time of 22 seconds

Optiway(config-if-ethernet-2/5)#dot1x daemon time 22

！Configure dot1x daemon of ethernet 2/5 globally with the period time of 22 seconds

Optiway(config)# dot1x daemon time 22 interface ethernet 2/5

！Restore the default dot1x daemon configuration on ethernet 2/5

Optiway(config-if-fastethernet-5)#no dot1x daemon

！Restore the default dot1x daemon configuration of ethernet 2/5 globally

Optiway(config)#no dot1x daemon interface ethernet 2/5

    - dot1x eap-finish

After using dot1x eap-transfer command, 822.1 authentication message encapsulated by EAP frame from user is sent to RADIUS server after transfering to data frame encapsulated by other high level protocol.

After using dot1x eap-finish command,

dot1x eap-finish

【Default】

Use eap-finish way to transmit authentication message.

【Command configuration mode】

Global configuration mode

【Usage】

Choose dot1x eap-finish or dot1x eap-transfer command according to RADIUS server configuration. If authentication message transmitting way is different from RADIUS server authentication message receiving way, authentication fails.

【Example】

！Configure authentication message tramsitting to be eap-finish

Optiway(config)#dot1x eap-finish

【Related command】

dot1x eap-transfer

    - dot1x eap-transfer

After using dot1x eap-transfer command, 822.1 authentication message encapsulated by EAP frame from user is sent to RADIUS server without any changes.

dot1x eap-transfer

【Default】

Use eap-finish way to transmit authentication message.

【Command configuration mode】

Global configuration mode

【Usage】

Choose dot1x eap-finish or dot1x eap-transfer command according to RADIUS server configuration. If authentication message transmitting way is different from RADIUS server authentication message receiving way, authentication fails.

【Example】

！Configure authentication message tramsitting to be eap-transfer

Optiway(config)#dot1x eap-transfer

【Related command】

dot1x eap-finish

    - dot1x max-user

Use dot1x max-user command to configure the maximum number of supplicant systems an ethernet port can accommodate. Use no dot1x max-user command to configure the maximum number to be 1.

dot1x max-user host-num

no dot1x max-user

【Parameter】

host-num:The integer between 1 and 16

【Default】

The max-user of 122M ethernet port is 16

【Command configuration mode】

Interface configuration mode or global configuration mode

【Usage】

This command is effective after 822.1X authentication.

After 822.1X enables, max-user of a port is determined by the real situation. The max-user of 122M ethernet port is 16

【Example】

！Configure the max-user of ethernet 2/5 is 12 in interface configuration mode

Optiway(config-if-ethernet-2/5)#dot1x max-user 122

！Configure the max-user of ethernet 2/5 is 12 globally

Optiway(config)#dot1x max-user 122 interface ethernet 2/5

！Restore the default max-user of ethernet 2/5 in interface configuration mode

Optiway(config-if-fastethernet-5)#no dot1x max-user

！Restore the default max-user of ethernet 2/5 globally

Optiway(config)#no dot1x max-user interface ethernet 2/5

    - dot1x port-control

Use dot1x port-control command to configure port control mode. Use no dot1x port-control command to restore the default port control.

dot1x port-control { auto | forceauthorized | forceunauthorized }

no dot1x port-control

【Parameter】

auto:Means needing authentication. User of this type of interface can get the resource from the LAN after authentication.

forceauthorized:Means forcing authorization. User of this type of interface can get the resource from the LAN without authentication.

forceunauthorized:Means forcing unauthorization. User of this type of interface cannot get the resource from the LAN.

【Default】

Port control mode is auto by default.

【Command configuration mode】

Interface configuration mode or global configuration mode

【Usage】

This command is effective after 822.1X authentication.

After 822.1X enables, the port control mode of RADIUS server is configured to be forceauthorized, so that the information of authenticator can be delivered to RADIUS server for authentication.

The port for user can be configured to be auto. User of this type of interface can get the resource from the LAN after authentication.

【Example】

！Ethernet 2/5 is RADIUS server port. Configure port-control mode of ethernet 2/5 to be forceauthorized in interface configuration mode

Optiway(config-if-ethernet-2/5)#dot1x port-control forceauthorized

！Configure port-control mode of ethernet 2/5 to be forceauthorized globally.

Optiway(config)#dot1x port-control forceauthorized interface ethernet 2/5

【Related command】

dot1x

    - dot1x re-authenticate

Use dot1x re-authenticate command to re-authenticate current interface.

dot1x re-authenticate

【Command configuration mode】

Interface configuration mode or global configuration mode

【Usage】

This command is effective after 822.1X authentication.

822.1X re-authenticate only supports the message transmitting way of dot1x eap-transfer.

【Example】

！Re-authenticate ethernet 2/5 in interface configuration mode

Optiway(config-if-ethernet-2/5)#dot1x re-authenticate

！Re-authenticate ethernet 2/5 globally

Optiway(config)#dot1x re-authenticate interface ethernet 2/5

    - dot1x re-authentication

Use dot1x re-authentication command to enable 822.1x re-authentication. Use no dot1x re-authentication command to disable 822.1x re-authentication.

dot1x re-authentication

no dot1x re-authentication

【Default】

822.1X re-authentication disable

【Command configuration mode】

Interface configuration mode, global configuration mode

【Usage】

This command is effective after 822.1x authentication enables.

822.1X authentication only supports the message sending of dot1x eap-transfer.

【Example】

！Enable re-authentication of ethernet 2/5

Optiway(config-if-ethernet-2/5)#dot1x re-authentication

Optiway(config)#dot1x re-authentication interface ethernet 2/5

【Related command】

dot1x,dot1x eap-finish,dot1x eap-transfer

    - dot1x timeout re-authperiod

Use dot1x timeout re-authperiod command to configure 822.1x re-authperiod. Use no dot1x timeout re-authperiod command to restore the default 822.1x re-authperiod.

dot1x timeout re-authperiod seconds [ interface interface-num ]

no dot1x timeout re-authperiod [ interface interface-num ]

【Parameter】

seconds: 822.1X re-authperiod ranges from 1 to 65535 seconds

interface-num:Optional interface number

【Default】

The default 822.1X re-authperiod is 3622 seconds

【Command configuration mode】

Global configuration mode

【Usage】

This command is effective after 822.1X authentication enables.

When no port is specified, use dot1x timeout re-authperiod command to modify 822.1x re-authperiod of all ports,or specified port is modified.

【Example】

！Configure 822.1x re-authperiod of ethernet 2/3 to be 1822

Optiway(config)#dot1x timeout re-authperiod 1822 interface ethernet 2/3

！Restore all the re-authperiod to the default of 822.1x re-authperiod

Optiway(config)#no dot1x timeout re-authperiod

    - dot1x user cut

Use dot1x user cut command to remove specified online user.

dot1x user cut { { username username } | { mac-address mac-address [ vlan vlan-id ] } }

【Parameter】

username: the username to be removed

mac-address:Mac address of user to be removed

vlan-id:The vlan of user to be removed

【Command configuration mode】

Global configuration mode

【Example】

！Remove user with username of aaa@gnnet.com

Optiway(config)#dot1x user cut username aaa@gnnet.com

    - dot1x detect

Configure 802.1x user heartbead detection and interval time  .

dot1x detect [interval time-value] [ interface interface-num ]

no dot1x detect

【Parameter】

time-value:send interval time of 1x heartbeat packets, unit is second

【Default】

By default, close 1x heartbeat ,when opening, send the packets every 25s

【Command mode】

Port configuration mode or all global configuration mode

【Usage】

This command is effective after it only enables 802.1X authentication..

Enable 802.1X uthentication, operators control to send heartbead detection packets and interval time to 1x authentication user according to the actual situation.

【Example】

！Enable 1x heartbeat detection, interval time is 50s

OptiWay(config)# dot1x detect interval 50

    - dot1x quiet-period-value

Configure static function, when 802.1x users fail the authentication, it can’t continue to verify in some period .

dot1x quiet-period-value time-value

【Parameter】

time-value:default interval time ,unit is second, default 0 means disable

【Default】

By default, it disables.

【Command mode】

All global configuration mode

【Usage】

This command is effective after it only enables 802.1X authentication..

【Example】

！Enable static function, the configuration time is 50s

OptiWay(config)# dot1x quiet-period-value 50

    - show dot1x

Use show dot1x command to display 822.1x authentication information, such as: 822.1x authentication is enable or not, which authentication is used.

show dot1x

【Command configuration mode】

Any configuration mode

【Usage】

Use show command to display related information before configuration.

【Example】

！Display 822.1x authentication information

Optiway(config)#show dot1x

    - show dot1x daemon

Use show dot1x daemon command to display 822.1x daemon configuration.

show dot1x daemon [ interface interface-num ]

【Parameter】

interface-num:Optioned interface number

【Command configuration mode】

Any configuration mode

【Example】

！Display the 822.1x daemon of all the ports

Optiway(config)#show dot1x daemon

    - show dot1x interface

Use show dot1x interface command to display such configuration of interface as control mode, re-authenticate, re-authperiod, max-user, etc.

show dot1x interface [ interface-num ]

【Parameter】

interface-num:Optioned interface number

【Command configuration mode】

Any configuration mode

【Usage】

Use this command to display related information before configuration. Use show command to display the changes.

【Example】

！Display port-control, re-authentication, re-authperiod and max-user configuration of ethernet 2/5

Optiway(config)#show dot1x interface ethernet 2/5

    - show dot1x session

Use show dot1x session command to display 822.1x session, including online information: interface number, mac-address, username, etc.

show dot1x session [ { interface interface-num } | { mac-address mac } ]

【Parameter】

interface-num:The interface number

mac:The optioned mac-address

【Command configuration mode】

Any configuration mode

【Usage】

Use this command to display and detect the information of onlined user

【Example】

！Display all the onlined authentication users

Optiway(config)#show dot1x session

# SNTP Client Configuration Command

  - SNTP client configuration command list

SNTP client configuration command includes:

- show sntp client

- sntp client

- sntp client authenticate

- sntp client authentication-key

- sntp client broadcastdelay

- sntp client mode

- sntp client multicast ttl

- sntp client poll-interval

- sntp client retransmit

- sntp client retransmit-interval

- sntp client valid-server

- sntp server

- sntp trusted-key

    - show sntp client

Use the show sntp client command to display the information about SNTP client configuration and running.

show sntp client

【Command configuration mode 】

Any configuration mode

【Example】

！Display the information about SNTP client configuration and running

Optiway(config)#show sntp client

    - sntp client

Use sntp client command to enable SNTP client. Use no sntp client command to disable SNTP client.

sntp client

no sntp client

【Usage】

If SNTP client has been enabled, sntp client command fails.

【Command configuration mode】

Global configuration mode

【Example】

！Enable SNTP client

Optiway(config)#sntp client

    - sntp client authenticate

Use sntp client authenticate command to enable MD5 authentication of SNTP client. Use no SNTP client authenticate command to disable MD5 authentication of SNTP client.

sntp client authenticate

no sntp client authenticate

【Default】

SNTP client authenticate disables

【Command configuration mode】

Global configuration mode

【Example】

！Enable SNTP client authenticate

Optiway(config)#sntp client authenticate

    - sntp client authentication-key

Use sntp client authentication-key command to configure MD5 authentication-key. More than one authentication-key can be configured.

sntp client authentication-key number md5 value

no sntp client authentication-key number

【Parameter】

number:Authentication-key ID ranges from 1to 4294967295

value:Authentication-key of 16 characters at most, which can be numbers, letters, space and other symbols.

【Default】

No authentication-key

【Usage】

Use sntp client authentication-key command to configure MD5 authentication-key. If the configuration is successful, the authentication-key should be effective after sntp client authentication-key command configures it reliable or to be the key of unicast and anycast.

【Command configuration mode】

Global configuration mode

【Example】

！Configure SNTP client MD5 authentication-key, with the key ID being 12,and the key being abc

Optiway(config)#sntp client authentication-key 12 md5 abc

    - sntp client broadcastdelay

Use sntp client broadcastdelay command to configure the transmission delay of the SNTP client in broadcast or multicast. Use no sntp client broadcastdelay command to restore default transmission delay.

sntp client broadcastdelay milliseconds

no sntp client broadcastdelay

【Parameter】

milliseconds:This keyword ranges from 1 to 9999

【Default】

3 milliseconds

【Command configuration mode】

Global configuration mode

【Usage】

Transmission delay is necessary because client cannot time transmission delay and local time compensation in broadcast and multicast.

【Example】

！Configure broadcastdelay to be 1 second

Optiway(config)#sntp client broadcastdelay 1222

    - sntp client mode

Use sntp client mode command to configure the operation mode of SNTP client. Use no sntp client mode command to restore the default operation mode of SNTP client.

sntp client mode { unicast | broadcast | multicast | anycast [ key number ] }

no sntp client mode

【Parameter】

unicast:Unicast mode

broadcast:Broadcast mode

multicast:Multicast mode

anycast:Anycast mode

number: ID of anycast ranges from 2 to 4294967295,2 means unauthentication.

【Default】

Broadcast mode

【Usage】

Use sntp client mode command to configure the operation mode of SNTP client. Only when SNTP client enables, this command is effective.

【Command configuration mode】

Global configuration mode

【Example】

！Configure SNTP client to operate in anycast

Optiway(config)#sntp client mode anycast

    - sntp client multicast ttl

Use sntp client multicast ttl command to configure ttl-value of multicast message. Use no sntp client multicast ttl command to restore default ttl-value.

sntp client multicast ttl ttl-value

no sntp client multicast ttl

【Parameter】

ttl-value:Ttl in multicast message sending ranges from 1 to 255

【Default】

Default ttl-value is 255

【Command configuration mode】

Global configuration mode

【Usage】

This command should be effective by sending message through multicast address in anycast operation mode. In order to restrict the range of sending multicast message, TTL-value setting is suggested.

【Example】

！Configure TTTL-value of sending multicast message to be 5

Optiway(config)#sntp client multicast ttl 5

    - sntp client poll-interval

Use sntp client poll-interval command to configure poll-interval of SNTP client in unicast or anycas. Use no sntp client poll-interval command to restore default poll-interval.

sntp client poll-interval seconds

no sntp client poll-interval

【Parameter】

seconds:Resending interval ranges from 64 to 1224 seconds

【Default】

1222 seconds

【Command configuration mode】

Global configuration mode

【Usage】

SNTP client sends requirement message regularly to the server in unicast and anycast operation mode. System time will be revised after receiving the message.

【Example】

！Configure poll-interval to be 122 seconds

Optiway(config)#sntp client poll-interval 122

    - sntp client retransmit

Use sntp client retransmit command to configure retransmit times inunicast and anycast operation mode. Use no sntp client retransmit command to configure SNTP client not to retransmit requirement message.

sntp client retransmit times

no sntp client retransmit

【Parameter】

times:Times of retransmit ranges from 1 to 12

【Default】

non-retransmit（2）

【Command configuration mode】

Global configuration mode

【Usage】

In order to guarantee reliable transmission of SNTP client, overtime retransmission system is adopted. The requirement message will be resent if there’s no reply in a certain time until the retransmit times limits. This command is effective in unicast and anycast operation mode, because these modes need send requirement message and overtime retransmission.

【Example】

！Configure overtime retransmission to be twice

Optiway(config)#sntp client retransmit 2

    - sntp client retransmit-interval

Use sntp client retransmit-interval command to configure retransmit-interval of SNTP client in unicast and anycast operation mode.

sntp client retransmit-interval seconds

no sntp client retransmit-interval

【Parameter】

seconds:Retransmit-interval ranges from 1 to 32 seconds

【Default】

5 seconds

【Command configuration mode】

Global configuration mode

【Usage】

Overtime retransmit system is used to guarantee reliable transmission of the requirement message. When there is no reply in retransmit-interval, the requirement message will be resent.

【Example】

！Configure retransmit-interval to be 12 seconds.

Optiway(config)#sntp client retransmit-interval 12

    - sntp client valid-server

Use sntp client valid-server command to add a filtration list item of valid -server. Use no sntp client valid-server command to remove a filtration list item of valid-server.

sntp client valid-server ip-address wildcard

no sntp client valid-server ip-address wildcard

【Parameter】

ip-address:Means valid-server interface. Mainframe cannot be 2

wildcard:Similar to reverse the mask

【Command configuration mode】

Global configuration mode

【Usage】

In the mode of broadcast and multicast, SNTP client checks time by receiving protocol messages sent by all servers. And it cannot filtrate the servers when spiteful attack exists. To solve this problem, a series of valid servers can be listed to filtrate source address of the message.

【Example】

！Add a valid-server list

Optiway(config)#sntp client valid-server 12.1.2.2 2.2.255.255

    - sntp server

Use sntp server command to configure server ip-address in unicast mode. Use no sntp server command to remove server ip-address.

sntp server ip-address [ key number ]

no sntp server

【Parameter】

ip-address:Server ip-address.

number: To encrypt message when sending requirement to server. Use the key-number to decipher the message when the reply is received. The key-number ranges from 2 to 4294967295. 2 means unauthentication.

【Command configuration mode】

Global configuration mode

【Usage】

In unicast mode, server ip-address must be configured, or SNTP client cannot work smoothly.

【Example】

！Configure unicast server ip-address to be 192.168.2.122

Optiway(config)#sntp server 192.168.2.122

    - sntp client summer-time

This command is to configure SNTP client summer-time.

sntp client summer-time { dayly start-month start-day start-time end-month end-day end-time | weekly start-month start-week start-weekday start-time end-month end-week end-weekday end-time}

no sntp client summer-time

【Parameter】

start-month:start month, the range is 1～12

end-month: end month,the range is 1～12

start-day: start day, the range is 1～31

end-day:end day, the range is 1～31

start-time:start time, the range is 00:00:00～23:59:59

end-time:end time, the range is 00:00:00～23:59:59

start-week:start week, the range is 1～4

end-week:end week, the range is 1～4

start-weekday:start weekday, the range is sun～sat

end-weekday:end weekday, the range is sun～sat

【Default】

By default, no configure summer-time

【Usage】

Normally, end time of summer time is bigger than start time,it corresponds to the northern hemisphere's summer-time; start time of summer time is bigger than end time,it corresponds to the southern hemisphere's summer-time.Then the range of summer-time is from start time to year end plus from the beginning time to the end time.

【Command mode】

All global configuration mode

【Example】

！Configure summer time as 0 clock April 1st to 23:59:59 Oct 31st

OptiWay(config)#sntp client summer-time dayly 4 1 00:00:00 10 31 23:59:59

    - sntp trusted-key

Use sntp trusted-key command to configure a trusted-key.

sntp trusted-key number

no sntp trusted-key number

【Parameter】

number:Key ID ranges from 1 to 4294967295

【Default】

All key number is reliable

【Usage】

In broadcast and multicast, the authentication is valid only when key-number is configured. The authentication is invalid when receiving the message encrypt by untrusty-key.

【Command configuration mode】

Global configuration mode

【Example】

！Configure trusted-key to be 12

Optiway(config)#sntp trusted-key 12

# Syslog Configiration Command

  - Syslog Configuration Command

Syslog configuration command includes:

- show logging

- show logging buffered

- show logging flash

- show logging filter

- show debug

- logging

- logging sequence-numbers

- logging timestamps

- logging language

- logging monitor

- terminal monitor

- logging buffered

- clear logging buffered

- logging flash

- clear logging flash

- logging host

- logging facility

- logging source

- logging snmp-agent

- debug

- upload logging

    - show logging

Use show logging command to display Syslog configuration, state, and statistical information.

show logging

【Command configuration mode】

Any configuration mode

【Example】

！Display Syslog configuration, state, and statistical information.

Optiway(config)#show logging

    - show logging buffered

Use show logging buffered command to display buffered log.

show logging buffered [ level | level-list { level [ to level ] } &<1-8> ] [ module { xxx | … } * ]

【Parameter】

level:Level of information ranges from 2 to 7

xxx:Means the name of the module. … means other modules are omitted.

【Command configuration mode】

Any configuration mode

【Usage】

Use keyword “level-list” to display the specified level information in list. If the “level-list” is not specified, the information of the higher level (The smaller the level number is, the higher the level is.) and the equal level will be displayed.

【Example】

！Display the buffered log of level 7

Optiway(config)#show logging buffered level-list 7

    - show logging flash

Use show logging flash command to display flash log.

show logging flash [ level | level-list { level [ to level ] } &<1-8> ] [ module { xxx | … } * ]

【Parameter】

level:Level of information ranges from 2 to 7

xxx:Means the name of the module. … means other modules are omitted.

【Command configuration mode】

Any configuration mode

【Usage】

Use keyword “level-list” to display the specified level information in list. If the “level-list” is not specified, the information of the higher level (The smaller the level number is, the higher the level is.) and the equal level will be displayed.

【Example】

！Display the flash log of module vlan

Optiway(config)#show logging flash module vlan

    - show logging filter

Use show logging filter command to display filter log.

show logging filter { monitor monitor-no | buffered | flash | host ip-address | snmp-agent }

【Parameter】

monitor-no:Means terminal number. 2 means console, and 1 to 5 means Telnet terminal.

ip-address:ip address of log host（Syslog server）

【Command configuration mode】

Any configuration mode

【Example】

！Display buffered filter log

Optiway(config)#show logging filter buffered

    - show debug

Use show debug command to display the debug of the module.

show debug

【Command configuration mode】

Any configuration mode

【Example】

！Display the debug of module

Optiway(config)#show debug

    - logging

Use logging command to enable Syslog. Use no logging command to disable Syslog.

logging

no logging

【Default】

Syslog enables

【Command configuration mode】

Global configuration mode

【Example】

！Enable Syslog

Optiway(config)#logging

    - logging sequence-numbers

Use logging sequence-numbers command to configure global sequence number to be displayed in Syslog. Use no logging sequence-numbers command to configure global sequence number not to be displayed in Syslog.

logging sequence-numbers

no logging sequence-numbers

【Default】

Not display global sequence number

【Command configuration mode】

Global configuration mode

【Example】

！Configure global sequence number to be displayed in Syslog outputting information.

Optiway(config)#logging sequence-numbers

    - logging timestamps

Use logging timestamps command to configure the type of timestamps in Syslog. Use no logging timestamps command to restore the default type of timestamps.

logging timestamps { notime | uptime | datetime }

no logging timestamps

【Parameter】

notime:Timestamps are not displayed

uptime:Uptime is the timestamps

datetime:Datetime is the timestamps

【Default】

Uptime is the default timestamps

【Command configuration mode】

Global configuration mode

【Example】

！Configure datetime to be the timestamps

Optiway(config)#logging timestamps datetime

    - logging language

Use logging language command to configure the language in Syslog.

logging language { english | chinese }

【Parameter】

english:Use English to be logging language

chinese:Use Chinese to be logging language

【Default】

Use English to be logging language

【Command configuration mode】

Global configuration mode

【Example】

！Configure Chinese to be logging language

Optiway(config)#logging language chinese

    - logging monitor

Use logging monitor command to enable monitor logging and configure filter regulation. Use no logging monitor command to disable monitor logging and restore default filter regulation.

logging monitor { all | monitor-no }

no logging monitor { all | monitor-no }

logging monitor { all | monitor-no } { level | none | level-list { level [ to level ] } &<1-8> } [ module { xxx | … } * ]

no logging monitor { all | monitor-no } filter

【Parameter】

all:All terminals

monitor-no:Means terminal number. 2 means console, and 1 to 5 means Telnet terminal.

level:Level of information ranges from 2 to 7

none:Any level is not allowed

xxx:Means the name of the module. … means other modules are omitted.

【Default】

All monitor logging disable.

Filter regulations of all terminals are to allow all modules of all levels except level 6 to output information

【Command configuration mode】

Global configuration mode

【Usage】

Use keyword “level-list” to display the specified level information in list. If the “level-list” is not specified, the information of the higher level (The smaller the level number is, the higher the level is.) and the equal level will be displayed.

【Example】

！Enable monitor logging

Optiway(config)#logging monitor 2

！Configure filter regulations of all terminals to allow all module of levels 2 to 6 to output information

Optiway(config)#logging monitor 2 6

    - terminal monitor

Use terminal monitor command to enable current terminal information displaying. Use no terminal monitor command to disable current terminal information displaying.

terminal monitor

no terminal monitor

【Default】

Current terminal information displaying enables,all Telnetterminal information displaying disables.

【Command configuration mode】

Any configuration mode

【Usage】

This command has influence on current terminal and current log in.

【Example】

！Enable current terminal information displaying

Optiway(config)#terminal monitor

    - logging buffered

Use logging buffered command to enable buffered logging and configure filter regulations. Use no logging buffered command to disable buffered logging and restore to default filter regulations.

logging buffered

no logging buffered

logging buffered { level | none | level-list { level [ to level ] } &<1-8> } [ module { xxx | … } * ]

no logging buffered filter

【Parameter】

level:Level of information ranges from 2 to 7

none:Any level is not allowed.

xxx:Means the name of the module. … means other modules are omitted.

【Default】

All buffered logging enable.

Filter regulations of all terminals are to allow all modules of levels 2 to 6 to output information

【Command configuration mode】

Global configuration mode

【Usage】

Use keyword “level-list” to display the specified level information in list. If the “level-list” is not specified, the information of the higher level (The smaller the level number is, the higher the level is.) and the equal level will be displayed.

【Example】

！Disable buffered logging

Optiway(config)#no logging buffered

！Configure filter regulations of all terminals to allow all module of level 2,1,2 and 6 to output information

Optiway(config)#logging buffered level-list 2 to 2 6

    - clear logging buffered

Use clear logging buffered command to clear buffered logging.

clear logging buffered

【Command configuration mode】

Any configuration mode

【Example】

！Clear buffered logging

Optiway(config)#clear logging buffered

    - logging flash

Use logging flash command to enable flash logging and configure filter regulations. Use no logging flash command to disable flash logging and restore to default filter regulations.

logging flash

no logging flash

logging flash { level | none | level-list { level [ to level ] } &<1-8> } [ module { xxx | … } * ]

no logging flash filter

【Parameter】

level:Level of information ranges from 2 to 7

none:Any level is not allowed.

xxx:Means the name of the module. … means other modules are omitted.

【Default】

All flash logging enable.

Filter regulations of all terminals are to allow all modules of levels 2 to 6 to output information

【Command configuration mode】

Global configuration mode

【Usage】

Use keyword “level-list” to display the specified level information in list. If the “level-list” is not specified, the information of the higher level (The smaller the level number is, the higher the level is.) and the equal level will be displayed.

【Example】

！Disable flash logging

Optiway(config)#no logging flash

！Configure filter regulations of all terminals to allow all vlan module to output information

Optiway(config)#logging flash none

Optiway(config)#logging flash 7 module vlan

    - clear logging flash

Use clear logging flash command to clear flash logging.

clear logging flash

【Command configuration mode】

Any configuration mode

【Example】

！Clear flash logging

Optiway(config)#clear logging flash

    - logging host

Use logging host command to configure host ip address, and enable host logging, and configure filter regulation of Syslog server. Use no logging host command to remove host ip address, disable host logging, and configure default filter regulation.

logging ip-address

no logging ip-address

logging host { all | ip-address }

no logging host { all | ip-address }

logging host { all | ip-address } { level | none | level-list { level [ to level ] } &<1-8> } [ module { xxx | … } * ]

no logging host { all | ip-address } filter

【Parameter】

all:All logging host

ip-address:IP address of Syslog server

level:Level of information ranges from 2 to 7

none:Any level is not allowed.

xxx:Means the name of the module. … means other modules are omitted.

【Default】

All logging host enable.

Filter regulations of all terminals are to allow all modules of levels 2 to 6 to output information

【Command configuration mode】

Global configuration mode

【Usage】

At most 15 logging hosts are allowed to configure.

Use keyword “level-list” to display the specified level information in list. If the “level-list” is not specified, the information of the higher level (The smaller the level number is, the higher the level is.) and the equal level will be displayed.

【Example】

！Add a new logging host with the ip address of 1.1.1.1

Optiway(config)#logging 1.1.1.1

！Enable logging host 1.1.1.1

Optiway(config)#logging host 1.1.1.1

！Configure filter regulations of logging host 1.1.1.1 to allow all module of level 2 to 6 to output information

Optiway(config)#logging host 1.1.1.1 6

    - logging facility

Use logging facility command to configure logging facility used by logging host. Use no logging facility command to restore the default logging facility.

logging facility { xxx | … } *

no logging facility

【Parameter】

xxx:The name of logging facilities.… means other logging facilities are omitted.

【Default】

Default logging facility is localuse7

【Command configuration mode】

Global configuration mode

【Example】

！Configure logging facility to be localuse2

Optiway(config)#logging facility localuse2

    - logging source

Use logging source command to configure logging host to use fixed source ip address outputting. Use no logging source command to configure logging host not to use fixed source ip address outputting.

logging source ip-address

no logging source

【Parameter】

ip-address:Fixed source ip address

【Default】

Not to use fixed source ip address

【Command configuration mode】

Global configuration mode

【Usage】

The fixed source ip address must be the ip address of some port in facility to be configured, or configuration fails. If the fixed source ip address is not used, egress interface is used as the fixed source ip address.

【Example】

！Configure the fixed source ip address of logging host to be 1.1.1.2

Optiway(config)#logging source 1.1.1.2

    - logging snmp-agent

Use logging snmp-agent command to enable SNMP Agent logging and configure filter configuration. Use no logging snmp-agent command to disable SNMP Agent logging and restore to default filter configuration.

logging snmp-agent

no logging snmp-agent

logging snmp-agent { level | none | level-list { level [ to level ] } &<1-8> } [ module { xxx | … } * ]

no logging snmp-agent filter

【Parameter】

level: information level, 2～7

none: no any level

xxx: module mode,…ignore other module name

【Parameter】

level:Level of information ranges from 2 to 7

none:Any level is not allowed.

xxx:Means the name of the module. … means other modules are omitted.

【Default】

By default , information output power OLT disables

By default, filter rule is that permit all module output information with level  2～5

【Default】

All SNMP Agent logging enable.

Filter regulations of all terminals are to allow all modules of levels 2 to 5 to output information

【Command mode】

All global configuration mode

【Command configuration mode】

Global configuration mode

【Usage】

Use key word evel-list to configure selected level information output in permited level list；Without level-list configuration permit , it equals higher（the small value with higher level）and this level information output

Configure Trap host address if Syslog information with Trap packets sends to SNMP Workstation, it also configures Trap host address ,see SNMP configuration.

【Usage】

Use keyword “level-list” to display the specified level information in list. If the “level-list” is not specified, the information of the higher level (The smaller the level number is, the higher the level is.) and the equal level will be displayed.

Configure Trap host ip address for Syslog information to send to SNMP Workstation by Trap message. (Refer to SNMP configuration)

【Example】

！Enable SNMP Agent logging

Optiway(config)#logging snmp-agent

！Configure SNMP Agent outputting filtration rule to be permitting 2 to 3 levels of information

Optiway(config)#logging snmp-agent 3

【Related command】

snmp-server host

    - debug

Use debug command to enable debug of a module. Use no debug command to disable debug of a module.

debug { all | { xxx | … } * }

no debug { all | { xxx | … } * }

【Parameter】

all:All module

xxx:Means the name of the module. … means other modules are omitted.

【Default】

All debug disable.

【Command configuration mode】

Global configuration mode

【Example】

！Enable debug of module vlan

Optiway(config)#debug vlan

    - upload logging

Use upload logging command to upload Flash storage to ftp or tftp server.

upload logging tftp ip-address file-name

upload logging ftp ip-address file-name user-name password

【Parameter】

ip-address:IP address of server

file-name:The filename saved to server

user-name:Ftp username

password:Ftp password

【Command configuration mode】

Privileged mode

【Example】

！Upload Flash storage to tftp server 1.1.1.1,and saved file is aaa.txt

Optiway(config)#upload logging tftp 1.1.1.1 aaa.txt

# SSH Configuration Command

  - SSH configuration command list

SSH configuration command includes:

- show ssh

- show keyfile

- ssh

- crypto key generate rsa

- crypto key zeroize rsa

- crypto key refresh

- load keyfile

- upload keyfile

    - show ssh

Use show ssh command to display SSH configuration information, including version number, enabling/disabling SSH and SSH keyfile.

show ssh

【Command configuration mode】

Any configuration mode

【Example】

！Display SSH information

Optiway#show ssh

    - show keyfile

Use show keyfile command to display keyfile in Flash storage.

show keyfile { public | private }

【Command configuration mode】

Privileged configuration mode

【Example】

！Display SSH keyfile

Optiway#show keyfile public

    - ssh

Use this command to enable/disable SSH.

ssh

no ssh

【Default】

Disable

【Command configuration mode】

Global configuration mode

【Example】

！Enable SSH

Optiway(config)#ssh

    - crypto key generate rsa

Use crypto key generate rsa command to configure SSH to be generate rsa.

crypto key generate rsa

【Command configuration mode】

Privileged configuration mode

【Example】

！Configure SSH key to be generate rsa.

Optiway#crypto key generate rsa

    - crypto key zeroize rsa

Use crypto key zeroize rsa command to clear the keyfile in Flash storage.

crypto key zeroize rsa

【Command configuration mode】

Privileged configuration mode

【Example】

！Clear keyfile in Flash storage

Optiway#crypto key zeroize rsa

    - crypto key refresh

Use crypto key refresh command to load SSH key from Flash storage.

crypto key refresh

【Command configuration mode】

Privileged configuration mode

【Example】

！Load SSH key from Flash storage.

Optiway#crypto key refresh

    - load keyfile

Use load keyfile command to download keyfile to device from tftp or ftp server.

load keyfile { public | private } tftp server-ip filename

load keyfile { public | private } ftp server-ip filename username passwd

【Parameter】

server-ip:IP address of tftp or ftp server

filename:file name of keyfile.

username:ftp username

passwd:ftp password

【Command configuration mode】

Privileged configuration mode

【Example】

！Download keyfile pub.txt from tftp server 1.1.1.1 as public keyfile

Optiway#load keyfile public tftp 1.1.1.1 pub.txt

    - upload keyfile

Use upload keyfile command to upload keyfile to device from tftp or ftp server.

upload keyfile { public | private } tftp server-ip filename

upload keyfile { public | private } ftp server-ip filename username passwd

【Parameter】

server-ip:IP address of tftp or ftp server

filename:file name of keyfile.

username:ftp username

passwd:ftp password

【Command configuration mode】

Privileged configuration mode

【Example】

！Upload keyfile to tftp server 1.1.1.1 and saved as pub.txt

Optiway#upload keyfile public tftp 1.1.1.1 pub.txt

# OLT Manage and Maintenance Command

  - Configuration Files Management

Configuration files management includes:

- buildrun mode continue

- buildrun mode stop

- clear startup-config

- copy nm-interface-config startup-config

- copy running-config startup-config

- copy startup-config running-config

- show running-config

- show startup-config

    - buildrun mode continue

Use buildrun mode continue command to configure buildrun mode to be continune.

buildrun mode continue

【Command configuration mode】

Privileged mode

【Example】

！Configure buildrun mode to be continune

OPTIWAY#buildrun mode continue

    - buildrun mode stop

Use buildrun mode stop command to configure buildrun mode to be stop.

buildrun mode stop

【Command configuration mode】

Privileged mode

【Example】

！Configure buildrun mode to be stop.

OPTIWAY#buildrun mode stop

    - clear startup-config

Use clear startup-config command to clear saved configuration.

clear startup-config

【Command configuration mode】

Privileged mode

【Usage】

Use this command to clear saved configuration and reboot OLT. The OLT will restore to original configuration.

【Example】

！Restore the original configuration

OPTIWAY#clear startup-config

    - copy nm-interface-config startup-config

Use copy nm-interface-config startup-config command to save minmum manageable configuration of network administration.

copy nm-interface-config startup-config [ vlan-interface-id  [ ip-address mask gateway-address ] ]

【Parameter】

vlan-interface-id: VLAN interface number

ip-address:IP address

mask:netmask

gateway-address:gateway address

【Command configuration mode】

Privileged configuration mode

【Usage】

If no keyword is configured, vlan-interface-id is defaulted to be the id of VLAN interface 1,ip-address is IP address of VLAN interface 1,mask is netmask of VLAN interface 1,gateway-address is the gateway address of VLAN interface 1；

If only imputed vlan-interface-id, ip-address is defaulted to be IPaddress of imputed VLAN interface,mask is netmask of VLAN interface,gateway-address is defaulted route gateway；

If all keywords are imputed, it will be saved as inputting.

【Example】

！Save configuration of VLAN interface 1

OPTIWAY#copy nm-interface-config startup-config

！Save configuration of VLAN interface 2

OPTIWAY#copy nm-interface-config startup-config 2

！Save configuration of user-defined interface

OPTIWAY#copy nm-interface-config startup-config 2 192.168.2.122 255.255.255.2 192.168.2.1

    - copy running-config startup-config

Use copy running-config startup-config command to save current configuration.

copy running-config startup-config

【Command configuration mode】

Privileged mode

【Example】

！Save current configuration

OPTIWAY#copy running-config startup-config

    - copy startup-config running-config

Use copy startup-config running-config command to execute saved configuration, and executed configuration is the same as the saved one.

copy startup-config running-config

【Command configuration mode】

Privileged mode

【Example】

！Execute saved configuration

OPTIWAY#copy startup-config running-config

    - show running-config

Use show running-config command to display current configuration.

show running-config [ module-list ]

【Parameter】

module-list:Optional module. The module name can be changed with the version.

【Command configuration mode】

Any configuration mode

【Example】

！Display all configurations

OPTIWAY#show running-config

！Display configuration of GARP and OAM module

OPTIWAY#show running-config garp oam

    - show startup-config

Use show startup-config command to display saved configuration.

show startup-config [ module-list]

【Parameter】

module-list:Optional module. The module name can be changed with the version.

【Command configuration mode】

Any configuration mode

【Example】

！Display all saved configuration

OPTIWAY#show running-config

！Display saved configuration of GARP and OAM module

OPTIWAY#show running-config garp oam

  - Online Loading Upgrade Program

Online Loading Upgrade Program includes:

- load application ftp

- load application tftp

- load application xmodem

- load configuration ftp

- load configuration tftp

- load configuration xmodem

- load whole-bootrom ftp

- load whole-bootrom tftp

- load whole-bootrom xmodem

- upload alarm ftp

- upload alarm tftp

- upload configuration ftp

- upload configuration tftp

- upload logging ftp

- upload logging tftp

    - load application ftp

Use load application ftp command to load application program by FTP protocol.

load application ftp ftpserver-ip filename username userpassword

【Parameter】

ftpserver-ip:IP address of FTP server

filename:Filename to be loaded

username,userpassword:Username and password of FTP server

【Command configuration mode】

Privileged mode

【Usage】

Open FTP server and set username, password and file download path before use this command. Reboot the OLT after successful download and run new application program.

【Example】

！Download application program app.arj to 192.168.2.122 by FTP

OPTIWAY#load application ftp 192.168.2.122 app.arj username password

    - load application tftp

Use load application tftp command to load application program by TFTP protocol.

load application tftp tftpserver-ip filename

【Parameter】

tftpserver-ip:IP address of TFTP server

filename:Filename to be loaded

【Command configuration mode】

Privileged mode

【Usage】

Open TFTP server and set file download path before use this command. Reboot the OLT after successful download and run new application program.

【Example】

！Download application program app.arj to 192.168.2.122 by TFTP

OPTIWAY#load application tftp 192.168.2.122 app.arj

    - load application xmodem

Use load application xmodem command to load application program by Xmodem protocol.

load application xmodem

【Command configuration mode】

Privileged mode

【Usage】

Choose “send” -> “send file” in super terminal, and input full path and filename of the file in filename dialog box, and choose Xmodem protocol in “protocol” , then click 【send】.

Reboot the OLT after successful download and run new application program.

【Example】

！Download application program by Xmodem protocol

OPTIWAY#load application xmodem

    - load configuration ftp

Use load configuration ftp command to load configuration program by FTP protocol.

load configuration ftp ftpserver-ip filename username userpassword

【Parameter】

ftpserver-ip:IP address of FTP server

filename:Filename to be loaded

username,userpassword:Username and password of FTP server

【Command configuration mode】

Privileged mode

【Usage】

Open FTP server and set username, password and file download path before use this command. Reboot the OLT after successful download and run new configuration program.

【Example】

！Download configuration program abc to 192.168.2.122 by FTP

OPTIWAY#load configuration ftp 192.168.2.122 abc username password

    - load configuration tftp

Use load configuration tftp command to load configuration program by TFTP protocol.

load configuration tftp tftpserver-ip filename

【Parameter】

tftpserver-ip:IP address of TFTP server

filename:Filename to be loaded

【Command configuration mode】

Privileged mode

【Usage】

Open TFTP server and set file download path before use this command. Reboot the OLT after successful download and run new configuration program.

【Example】

！Download configuration program abc to 192.168.2.122 by TFTP

OPTIWAY#load configuration ftp 192.168.2.122 abc

    - load configuration xmodem

Use load configuration xmodem command to load configuration program by Xmodem protocol.

load configuration xmodem

【Command configuration mode】

Privileged mode

【Usage】

Choose “send” -> “send file” in super terminal, and input full path and filename of the file in filename dialog box, and choose Xmodem protocol in “protocol”, then click 【send】.

Reboot the OLT after successful download and run new application program.

【Example】

！Download configuration program by Xmodem protocol

OPTIWAY#load configuration xmodem

    - load whole-bootrom ftp

Use load whole-bootrom ftp command to load whole bootrom by FTP protocol.

load whole-bootrom ftp ftpserver-ip filename username userpassword

【Parameter】

ftpserver-ip:IP address of FTP server

filename:Filename to be loaded

username,userpassword:Username and password of FTP server

【Command configuration mode】

Privileged mode

【Usage】

Open FTP server and set username, password and file download path before use this command.

【Example】

！Download whole-bootrom abc to 192.168.2.122 by FTP

OPTIWAY#load whole-bootrom ftp 192.168.2.122 abc username password

    - load whole-bootrom tftp

Use load whole-bootrom tftp command to load whole bootrom by TFTP protocol.

load whole-bootrom tftp tftpserver-ip filename

【Parameter】

tftpserver-ip:IP address of TFTP server

filename:Filename to be loaded

【Command configuration mode】

Privileged mode

【Usage】

Open TFTP server and set file download path before using this command.

【Example】

！Download whole-bootrom abc to 192.168.2.122 by TFTP

OPTIWAY#load whole-bootrom tftp 192.168.2.122 abc username password

    - load whole-bootrom xmodem

Use load whole-bootrom xmodem command to load whole bootrom by xmodem protocol.

load whole-bootrom xmodem

【Command configuration mode】

Privileged mode

【Usage】

Choose “send” -> “send file” in super terminal, and input full path and filename of the file in filename dialog box, and choose Xmodem protocol in “protocol”, then click 【send】.

【Example】

！Download whole bootrom by Xmodem protocol

OPTIWAY#load whole-bootrom xmodem

    - upload alarm ftp

Use upload alarm ftp command to upload alarm by FTP protocol.

upload alarm ftp ftpserver-ip filename username userpassword

【Parameter】

ftpserver-ip:IP address of FTP server

filename:Filename to be uploaded which cannot be system keyword (such as in windows operating system, con cannot be filename.)

username,userpassword:Username and password of FTP server

【Command configuration mode】

Privileged mode

【Usage】

Open FTP server and set username, password and file upload path before use this command. Alaram information saved when uploading is successful.

【Example】

！Upload alarm to 192.168.2.122 by FTP and saved as abc

OPTIWAY#upload alarm ftp 192.168.2.122 abc username password

    - upload alarm tftp

Use upload alarm tftp command to upload alarm by TFTP protocol.

upload alarm tftp tftpserver-ip filename

【Parameter】

tftpserver-ip:IP address of TFTP server

filename:Filename to be uploaded which cannot be system keyword (such as in windows operating system, con cannot be filename.)

【Command configuration mode】

Privileged mode

【Usage】

Open TFTP server and set file upload path before using this command. Alaram information saved when uploading is successful.

【Example】

！Upload alarm to 192.168.2.122 by TFTP and saved as abc

    - upload configuration ftp

Use upload configuration ftp command to upload configuration program by FTP protocol.

upload configuration ftp ftpserver-ip filename username userpassword

【Parameter】

ftpserver-ip:IP address of FTP server

filename:Filename to be uploaded which cannot be system keyword (such as in windows operating system, con cannot be filename.)

username,userpassword:Username and password of FTP server

【Command configuration mode】

Privileged mode

【Usage】

Open FTP server and set username, password and file upload path before use this command. Configuration information saved when uploading is successful.

【Example】

！Upload configuration to 192.168.2.122 by FTP and saved as abc

OPTIWAY#upload configuration ftp 192.168.2.122 abc username password

    - upload configuration tftp

Use upload configuration tftp command to upload configuration program by TFTP protocol.

upload configuration tftp tftpserver-ip filename

【Parameter】

tftpserver-ip:IP address of TFTP server

filename:Filename to be uploaded which cannot be system keyword (such as in windows operating system, con cannot be filename.)

【Command configuration mode】

Privileged mode

【Usage】

Open TFTP server and set file upload path before using this command. Configuration information saved when uploading is successful.

【Example】

！Upload configuration to 192.168.2.122 by TFTP and saved as abc

OPTIWAY#upload configuration tftp 192.168.2.122 abc

    - upload logging ftp

Use upload logging ftp command to upload logging by FTP protocol.

upload logging ftp ftpserver-ip filename username userpassword

【Parameter】

ftpserver-ip:IP address of FTP server

filename:Filename to be uploaded which cannot be system keyword (such as in windows operating system, con cannot be filename.)

username,userpassword:Username and password of FTP server

【Command configuration mode】

Privileged mode

【Usage】

Open FTP server and set username, password and file upload path before use this command. Configuration information saved when uploading is successful.

【Example】

！Upload logging to 192.168.2.122 by FTP and saved as abc

OPTIWAY#upload logging ftp 192.168.2.122 abc username password

    - upload logging tftp

Use upload logging tftp command to upload logging by TFTP protocol.

upload logging tftp tftpserver-ip filename

【Parameter】

tftpserver-ip:IP address of TFTP server

filename:Filename to be uploaded which cannot be system keyword (such as in windows operating system, con cannot be filename.)

【Command configuration mode】

Privileged mode

【Usage】

Open TFTP server and set file upload path before using this command. Logging information saved when uploading is successful.

【Example】

！Upload logging to 192.168.2.122 by TFTP and saved as abc

OPTIWAY#upload logging tftp 192.168.2.122 abc

  - Reboot OLT

Reboot OLT command includes:

- reboot

    - reboot

Use reboot command to reboot OLT.

reboot

【Command configuration mode】

Privileged mode

【Example】

！Reboot OLT

OPTIWAY#reboot

  - Basic Configuration and Maintenance

Basic configuration and mainenance includes:

- broadcast-suppression

- clock set

- clock timezone

- discard-bpdu

- dlf-forward

- loopback

- mac-address-table

- mac-address-table aging-time

- mac-address-table learning

- ping

- show broadcast-suppression

- show clock

- show cpu

- show dhcp-server clients

- show discard-bpdu

- show dlf-forward

- show ip fdb

- show mac-address-table

- show mac-address-table aging-time

- show mac-address-table learning

- show memory

- show system

- show users

- show version

    - broadcast-suppression

Use broadcast-suppression command to configure the broadcast flow allowed by OLT. When broadcast flow is beyond the limit, it will be dropped to guarantee network to reduce broadcast flow to a resonable range. Use no broadcast-suppression command to disable broadcast storm suppression to configure the broadcast flow allowed by OLT to be the maximum of 222222 per second, which means no suppression on broadca

broadcast-suppression packet-num

no broadcast-suppression

【Default】

The default broadcast flow allowed by OLT is at most 5222 per second

【Command configuration mode】

Global configuration mode

【Usage】

To suppress broadcast storm, and avoid network congestion can use this command.

【Example】

！Allow at most 322 messages per second.

OPTIWAY(config)#broadcast-suppression 322

！Non broadcast suppression

OPTIWAY(config)#no broadcast-suppression

    - clock set

Use clock set command to configure system clock.

clock set

【Parameter】

HH:MM:SS:current time,HH ranges from 2 to 23,MM and SS range from 2 to 59

YYYY/MM/DD:Means current year, month, and date. YYYY ranges from 2222 to 2299,MM ranges from 1 to 12,and DD ranges from 1 to 31

【Default】

The default time is 2224/21/21 2:2:2

【Command configuration mode】

Privileged mode

【Usage】

Use this command to set current date and time when needing it.

【Example】

！Configure system clock to be 2221/21/21 2:2:2

OPTIWAY#clock set 2:2:2 2221/21/21

【Related command】

show clock

    - clock timezone

Use clock timezone command to configure clock timezone.

clock timezone name hour minute

no clock timezone

【Parameter】

name:Name of timezone ranges from 1 to 32 characters

hour:The hours offset ranges from –23 to 23

minute:The minutes offset ranges from 2 to 59

【Default】

Beijing time（CCT）,offset 8 hours

【Command configuration mode】

Global configuration mode

【Example】

！Configure the clock timezone to be Beijing time.

OPTIWAY#clock timezone CCT 8 2

【Related command】

show clock

    - discard-bpdu

Use Discard-bpdu command to enable dropping specified typed BPDU packet. Use no discard-bpdu command to disable this function.

Discard-bpdu

no discard-bpdu

【Default】

Transmit BPDU packet

【Usage】

If BPDU storm appears and interface CAR configuration cannot eliminate the conflict of BPDU storm to CPU, it can use this command to drop BPDU packet.

【Command configuration mode】

Global configuration mode

【Example】

！Enable dropping BPDU packtet

OPTIWAY(config)#discard-bpdu

    - discard-l2-tunnel

use this command to enable discard specified l2-tunnel packet.Use the no command to disable this function.

discard-l2-tunnel

no discard-l2-tunnel

【Default】

Transmit all l2-tunnel packet

【Usage】

If l2-tunnel storm appears, and cannot be eliminated through port-car, this command can be used.

【Command configuration mode】

Global configuration mode

【Example】

！Enable discard l2-tunnel

Optiway(config)#discard-l2-tunnel

    - dlf-forward

Use dlf-forward command to enable dlf forword. Use no dlf-forward command to disable dlf forward.

dlf-forward { multicast | unicast }

no dlf-forward { multicast | unicast }

【Parameter】

multicast:Multicast message

unicast:Unicast message

【Default】

Transmit unicast and multicast message.

【Usage】

To suppress broadcast storm, and avoid network congestion can use this command to control whether to transmit destination unknown message.

【Command configuration mode】

Global configuration mode, Interface configuration mode

【Example】

！Disable dlf forward for unicast

OPTIWAY(config)#no dlf-forward unicast

    - loopback

Use loopback command to loopback. External and internal can be chosed in global confuration or interface configurationmode.

loopback { external | internal }

【Parameter】

external:External loopback

internal:Internal loopback

【Command configuration mode】

Global configuration mode, interface configuration mode

【Example】

！Loopback on all interfaces

OPTIWAY(config)#loopback external

    - vct run

Use vct run command to port vct test. Vct test for all the ports in global configuration mode. Vct test for current port in interface configuration mode.

vct run

【Command configuration mode】

interface configuration mode

【Example】

！Enable VCT run on e2/1

Optiway(config-if-ethernet-2/1)#vct run

    - vct auto-run

This command is to enable global or port about VCT automatic detection, with command no is to disable global or port about VCT automatic detection.

vct auto-run

no vct auto-run

【Default】

By default, it disables global or port about VCT automatic detection

【Command mode】

All global configuration mode, ethernet configuration mode

【Example】

！Enable VCT automatic detection

OptiWay(config)#vct auto-run

！Enable interface 8 of VCT automatic detection

OptiWay(config-if-ethernet-0/8)#vct auto-run

    - show vct auto-run

This command is to display VCT automatic detection.

show vct auto-run

【Command mode】

All command mode

【Example】

！Display VCT automatic detection

OptiWay(config)#sh vct auto-run

VCT auto run global status : disable

VCT auto run enable port :

    - mac-address-table

Use mac-address-table command to add mac address table. Use no mac-address-table command to remove mac address table.

mac-address-table { dynamic | permanent | static } mac interface interface-num vlan vlan-id

mac-address-table blackhole mac vlan vlan-id

no mac-address-table [ blackhole | dynamic | permanent | static ] mac vlan vlan-id

no mac-address-table [ dynamic | permanent | static ] mac interface interface-num vlan vlan-id

no mac-address-table [ dynamic | permanent | static ] interface interface-num

no mac-address-table [ blackhole | dynamic | permanent | static ] vlan vlan-id

no mac-address-table

【Parameter】

mac:Unicast mac address

vlan-id:VLAN id

interface-num:Number of interface for message outputting

backhole:Blackhole address table which is not aging, and will not be lost after OLT rebooting. Message whose source or destination mac address is the same as this mac address will be dropped.

dynamic:Dynamic address table which can be aging.

permanent:Permanent address table which cannot be aging and will not be lost after OLT rebooting.

static:Static address table which is not aging and will be lost after OLT reboot.

All blackhole/static/dynamic/permanent address can add 522 totally.

【Command configuration mode】

Global configuration mode

【Example】

！Add mac address 22:21:22:23:24:25 to be permanent address table.

OPTIWAY(config)#mac-address-table permanent 22:21:22:23:24:25 interface ethernet 2/1 vlan 1

    - mac-address-table age-time

Use mac-address-table age-time command to configure MAC address aging time. Use no mac-address age-time command to restore it to default time.

mac-address-table age-time [ agetime | disable ]

no mac-address age-time

【Parameter】

agetime:Means MAC address aging time which ranges from 1 to 1248575 seconds

disable:Means MAC address not aging.

【Default】

Default MAC address aging time is 322 seconds

【Command configuration mode】

Global configuration mode

【Example】

！Configure MAC address aging time to be 622 seconds

OPTIWAY(config)#mac-address-table age-time 622

    - mac-address-table learning

Use mac-address-table learning command to enable MAC address learning. Use no mac-address-table learning command to disable MAC address learning. When disabling, the message from a port whose source address is not in this port, will not be transmitted.

mac-address-table learning

no mac-address-table learning

【Command configuration mode】

Global configuration mode

【Example】

！Enable MAC address learning.

OPTIWAY(config)#mac-address-table learning

    - mac-address-table max-mac-count

Use this command to configure the number of MAC address interface permits learning. Use no command to restore it to default number.

mac-address-table max-mac-count max-mac-count

no mac-address-table max-mac-count

【Parameter】

max-mac-count:the max number of MAC address that interface permits learning which is in the range of 2 – 4295.

【Default】

It is defaulted to be no restriction.

【Command configuration mode】

Interface configuration mode

【Example】

！Configure the max number of MAC address of interface 3 to be 8

Optiway(config-if-ethernet-2/3)#mac-address-table max-mac-count 8

    - ping

Use ping command to check the network connection.

ping [ -c count ] [ -s packetsize ] [ -t timeout ] host

【Parameter】

count:The number of message sending.

packetsize:The length of message sending, with the unit of second

timeout:the time of waiting for replying after message is sent,with the unit of second

host:Host ip address

【Command configuration mode】

Any configuration mode

【Usage】

Use this command to test whether the facility in the same net is connected or not.

【Example】

！The ip address of current OLT is 192.168.2.122. Test the connection of OLT with the ip address of 192.168.2.222

OPTIWAY#ping 192.168.2.222

    - show broadcast-suppression

Use show broadcast-suppression command to display the number of the broadcast flow allowed by OLT.

show broadcast-suppression

【Command configuration mode】

Any configuration mode

【Example】

！Display the max number of the broadcast flow allowed by OLT per second.

OPTIWAY(config)#show broadcast-suppression

    - show clock

Use show clock command to display system clock.

show clock

【Command configuration mode】

Any configuration mode

【Example】

！Display system clock

OPTIWAY#show clock

2221/21/21 22:22:22 CCT 8:22

【Related command】

clock set

    - show cpu-utilization

Use show cpu-utilization command to display cpu utilization. The smaller the value is,the busier the CPU is.

show cpu-utilization

【Command configuration mode】

Any configuration mode

【Example】

！Display CPU utilization

Optiway(config)#show cpu-utilization

    - show dhcp-server clients

show dhcp-server clients [ ip [mask] | mac | poolname ]

【Parameter】

ip:display information of specified IP address

mask:display information of specified IP address range

mac:display information of IP address according to MAC address

poolname:display information of IP address in specified IP address pool

【Command configuration mode】

Any configuration mode

【Example】

！Display distributed IP address information of internal DHCP server

OPTIWAY(config)#show dhcp-server clients

    - show discard-bpdu

Use this command to display the drop configuration of BPDU packet.

show discard-bpdu

【Command configuration mode】

Any configuration mode

【Example】

！Display drop configuration of BPDU packet

OPTIWAY(config)#show discard-bpdu

    - show dlf-forward

Use show dlf-forward command to display configuration of message transmitting to unknown destination.

show dlf-forward

【Command configuration mode】

Any configuration mode

【Example】

！Display onfiguration of message transmitting to unknown destination.

OPTIWAY(config)#show dlf-forward

    - show ip fdb

Use this command to display L3 table of all l3 interfaces or L3 table of specified IP.

show ip fdb [ ip ip-address [ mask ] ]

【Command configuration mode】

Any configuration mode

【Example】

！Display L3 table of all L3 interfaces

OPTIWAY(config)#show ip fdb

    - show mac-address-table

show mac-address-table

show mac-address-table { interface-num [ vlan vlan-id ] | cpu }

show mac-address-table mac [ vlan vlan-id ]

show mac-address-table max-mac-count interface [ethernet interface-num]

show mac-address-table { blackhole | dynamic | permanent | static } [ vlan vlan-id ]

show mac-address-table { blackhole | dynamic | permanent | static } interface interface-num [ vlan vlan-id ]

show mac-address-table vlan vlan-id

【Parameter】

mac:Unicast mac address

vlan-id:VLAN id

interface-num:Number of interface for message outputting

backhole:Blackhole address table which is not aging, and will not be lost after OLT rebooting. Message whose source or destination mac address is the same as this mac address will be dropped.

dynamic:Dynamic address table which can be aging.

permanent:Permanent address table which cannot be aging and will not be lost after OLT rebooting.

static:Static address table which is not aging and will be lost after OLT reboot.

CPU: system mac address

【Command configuration mode】

Any configuration mode

【Example】

！Display all MAC address table

OPTIWAY(config)#show mac-address-table

    - show mac-address-table age-time

Use show mac-address-table age-time command to display MAC address aging time.

show mac-address-table age-time

【Command configuration mode】

Any configuration mode

【Example】

！Display MAC address aging time.

OPTIWAY(config)#show mac-address-table aging-time

    - show mac-address-table learning

Use show mac-address-table learning command to display MAC address learning.

show mac-address-table learning

【Command configuration mode】

Any configuration mode

【Example】

！Display MAC address learning.

OPTIWAY(config)#show mac-address-table learning

    - show memory

Use show memory command to display memory usage.

show memory

【Command configuration mode】

Any configuration mode

【Example】

！Display memory usage

OPTIWAY(config)#show memory

    - show system

Use show system command to display system information.

show system

【Command configuration mode】

Any configuration mode

【Example】

！Display system information

OPTIWAY(config)#show system

    - show users

Use show users command to display the user information logged in.

show users

【Command configuration mode】

Any configuration mode

【Example】

！Display the user information logged in.

OPTIWAY (config)#show users

    - show version

Use show version command to display system version.

show version

【Command configuration mode】

Any configuration mode

【Usage】

The software information is different with different version.

【Example】

！Display system version

OPTIWAY# show version

    - login-access-list telnet-limit

Use this command to restrict the number of Telnet user (2-5) to enter privileged mode at the same time.

login-access-list telnet-limit limit-no

no login-access-list telnet-limit

【Command configuration mode】

Global configuration mode

【Parameter】

limit-no:the number of Telnet user to enter privileged mode (2～5)

【Default】

The max number is defaulted to be 5.

【Example】

！Configure only 1 Telnet user can enter privileged mode

OPTIWAY(config)# login-access-list telnet-limit 1

【Related command】

show users

    - tracert

Tracert is used for routing detecting and network examination.

tracert  [ -u | -c ] [ -p udpport | -f first_ttl | -h maximum_hops | -w time_out ] target_name

【Parameter】

-u  means sending udp packet,-c means sending echo packet of icmp. It is defaulted to be -c；

udpport:destination interface address for sending udp packet which is in the range of 1 to 65535 and defaulted to be 62929；

first_ttl:initial ttl of sending packet which is in the range of 1 to 255 and defaulted to be 1；

maximum_hops:the max ttl of sending packet which is in the range of 1 to 255 and defaulted to be 32；

time_out:the overtime of waiting for the response which is in the range of 12 to 62 with the unit of second and default to be 12 seconds；

target_name:destination host or router address

【Command configuration mode】

Any configuration mode

【Usage】

Tracert is used for routing detecting and network examination.

【Example】

！Tracert 192.168.2.222

OPTIWAY#tracert 192.168.2.222

    - cpu-car

Use this command to to configure cpu rate for receiving packet.

Cpu-car  target_rate

【Parameter】

target-rate: cpu rate for receiving packet , which is in the range of 1 to 1222pps and the default rate is 52.

【Command configuration mode】

Global configuration mode

【Example】

！Configure cpu rate for receiving packet to be 122pps

Optiway(config)#cpu-car 122

    - show cpu-car

Use this command to display cpu-car.

show cpu-car

【Command configuration mode】

Any configuration mode

【Example】

！Show cpu-car

Optiway#show cpu-car

    - show cpu- statistics

Use this command to display cpu received statistic information from interface.

show cpu-statistics

【Command configuration mode】

Any configuration mode

【Example】

！Display cpu received statistic information from interface.

Optiway#show cpu-statistics

    - clear cpu- statistics

Use this command to clear cpu received statistic information from interface.

clear cpu-statistics

【Command configuration mode】

Global configuration mode

【Example】

！Clear cpu received statistic information from interface

Optiway(config)#clear cpu-statistics

  - SNMP Configuration

SNMP configuration command includes:

- show snmp community

- show snmp contact

- show snmp host

- show snmp notify

- show snmp location

- show snmp engineID

- show snmp group

- show snmp user

- show snmp view

- snmp-server community

- snmp-server contact

- snmp-server host

- snmp-server location

- snmp-server name

- snmp-server enable traps

- snmp-server trap-source

- snmp-server engineID

- snmp-server view

- snmp-server group

- snmp-server user

- snmp-server security-name

    - show snmp community

Use show snmp community command to display information of all SNMP sever community list.

show snmp community

【Command configuration mode】

Any configuration mode

【Example】

！Display SNMP community information

OPTIWAY(config)#show snmp community

    - show snmp contact

Use show snmp contact command to display how to contact to administrator.

show snmp contact

【Command configuration mode】

Any configuration mode

【Usage】

Use this command when you need to contact to administrator

【Example】

！Display how to contact with administrator

OPTIWAY(config)#show snmp contact

    - show snmp host

Use show snmp host command to display Trap information of SNMP server

show snmp host

【Command configuration mode】

Any configuration mode

【Example】

！Display Trap information of snmp server

OPTIWAY(config)#show snmp host

    - show snmp notify

Use show snmp notify command to display all notify information.

show snmp notify

【Command configuration mode】

Any configuration mode

【Example】

！Display all notify information

OPTIWAY(config)#show snmp notify

    - show snmp location

Use show snmp location command to display system location.

show snmp location

【Command configuration mode】

Any configuration mode

【Usage】

Use this command when you need to know system location.

【Example】

！Display system location

OPTIWAY(config)#show snmp location

    - show snmp engineID

Use show snmp engineID command to display engine id configuration.

show snmp engineID [local | remote]

【Command configuration mode】

Any configuration mode

【Usage】

Choose “local” to display local engine, and choose “remote” to display remote engine.

【Example】

！Display local engine id

OPTIWAY(config)# show snmp engine id local

    - show snmp group

Use show snmp group command to display group configuration.

show snmp group

【Command configuration mode】

Any configuration mode

【Usage】

Use this command to display configured group.

【Example】

！Display configured group

OPTIWAY(config)# show snmp group

    - show snmp user

Use show snmp user command to display user configuration.

show snmp user

【Command configuration mode】

Any configuration mode

【Usage】

Use this command to display configured user.

【Example】

！Display configured user

OPTIWAY(config)# show snmp user

    - show snmp view

Use show snmp view command to display view configuration.

show snmp view

【Command configuration mode】

Any configuration mode

【Usage】

Use this command to display configured view.

【Example】

！Display configured view

OPTIWAY(config)# show snmp view

    - snmp-server community

Use snmp-server community command to configure or modify community name and other information in community list. Use no snmp-server community command to remove community name in the list.

snmp-server community community { ro | rw } { deny | permit } [ view view-name ]

no snmp-server community community

【Parameter】

community:The community name, a printable character string of 1 to 22 characters.

ro:Read only

rw:Can be read and write

deny:Cannot be activated

permit:Can be activated

view-name: view configured for community. A string of 1 to 32 printable characters, excluding space. The default configuration view is iso.

【Command configuration mode】

Global configuration mode

【Usage】

The community name in nosnmp-server community command should be existed.

【Example】

！Add community green,and configure privilege to be ro,and permit

OPTIWAY(config)#snmp-server community green ro permit

！Remove community green

OPTIWAY(config)#no snmp-server community green

    - snmp-server contact

Use snmp-server contact command to configure how to contact with administrator. Use no snmp-server contact command to restore default way of contacting to administrator.

snmp-server contact syscontact

no snmp-server contact

【Parameter】

syscontact:Contact way to administrator ranges from 1 to 255 printable characters.

【Default】

“GreenNet Shenzhen China (http://www.greennet.com.cn)”

【Command configuration mode】

Global configuration mode

【Usage】

Use quotation mark to quote space in charater string.

【Example】

！Configure administrator contact way to be support@greennet.com.cn.

OPTIWAY(config)#snmp-server contact support@greennet.com.cn

    - snmp-server host

Use snmp-server host command to send notify by SNMP server. Use no snmp-server host command to remove SNMP server sending notifies.

snmp-server host host-addr [version {1 | 2c | 3 [auth | noauth | priv]}] community-string  [udp-port port] [ notify-type [ notifytype-list ] ]

no snmp-server host ip-address community { 1 | 2c | 3 }

【Parameter】

community:Means community name corresponded by SNMP server sending notifylist.

1:Means SNMP version 1

2c:Means SNMP version 2c

3:Means SNMP version 3

ip-address:Means IP address in SNMP server notify sending list

port:Means objective host number

notifytype-list:Optional notify list. If it is unoptioned, default to choose all type. Only optionaed type will be sent to destination host.

【Command configuration mode】

Global configuration mode

【Usage】

Community cannot be vacant in snmp-server host version command. Community name in no snmp-server host command must be the same as that in snmp-server host.

【Example】

！Configure Trap in SNMP server, the IP address is configured to be 192.168.2.122,and SNMP version to be 2c,and community name to be user

OPTIWAY(config)#snmp-server host 192.168.2.122 version 2c user

    - snmp-server location

Use snmp-server location command configuration system location.

snmp-server location syslocation

【Parameter】

syslocation:The charater string of system location ranges from 1 to 255 printable characters.

【Command configuration mode】

Global configuration mode

【Usage】

Use quotation mark to quote space in charater string.

【Example】

！Configure system location to be sample sysLocation factory.

OPTIWAY(config)#snmp-server location “sample sysLocation factory”

    - snmp-server name

Use snmp-server name command to configure system name. Use no snmp-server name command to restore default syastem name.

snmp-server name sysname

no snmp-server name

【Parameter】

sysname:The charater string of system name ranges from 1 to 255 printable characters.

【Default】

The default system name is“OPTIWAY S2926V-O”

【Command configuration mode】

Global configuration mode

【Usage】

Use quotation mark to quote space in charater string.

【Example】

！Configure system name to be OPTIWAY S2926V-O

OPTIWAY(config)#snmp-server name "OPTIWAY S2926V-O"

    - snmp-server enable traps

Use snmp-server enable traps command to enable traps. Use no snmp-server enable traps command to disable traps.

snmp-server enable traps [ notificationtype-list ]

no snmp-server enable traps [ notificationtype-list ]

【Parameter】

notificationtype-list:Notificationtype list defined by system. To enable or disable specified notification type by choose one or serval type. If the keyword is vacant, all types of notification are enabled or disabled.

【Default】

Default sending way is trap,and snmp-server traps disables.

【Command configuration mode】

Global configuration mode

【Usage】

The notificationtype list can be optioned. If the keyword is vacant, all types will be optioned.

【Example】

！Enable notificationtype gbn

OPTIWAY(config)# snmp-server enable traps gbn

    - snmp-server trap-source

Use snmp-server trap-source command to configure vlan interface of trap sending source address. Use no snmp-server command to restore default trap sending source address.

snmp-server trap-source { vlan-interface vlan-id | supervlan-interface supervlan-id }

no snmp-server

【Parameter】

vlan-id is the vlan id of trap source-address. It ranges from 1 to 4294.

supervlan-id is the supervlan id of trap source-address. It ranges from 1 to 11.

【Default】

Trap source-address is defaulted to be output interface ip

【Command configuration mode】

Global configuration mode

【Usage】

System cannot be sure whether the vlan and supervlan of the input vlan-id or supervlan-id are existed or not and whether they have interface and the ip address of interfaces are also not sure.

【Example】

！Configure trap source-address to be the ip address of interface 1 of vlan

OPTIWAY(config)# snmp-server trap-source vlan-interface 1

    - snmp-server engineID

Use snmp-server engineID command to configure local engine-id or recognizable remote engine-id. Use no snmp-server engineID command to restore default local engine-id or remove remote engine-id.

snmp-server engineID  { local engineid-string | remote ip-address [udp-port port-number] engineid-string }

no snmp-server engineID { local | remote ip-address [udp-port port-number] }

【Parameter】

engineid-string is an engine id that can only be recognized in a network. This system only supports printable characters of engine id which excludes space.

Ip-address is remote engine ip address. Local ip address is not allowed to input.

Port-number is remote engine port number. Default port number is 162

【Default】

Default local engine id is 134642222222222222222222

【Command configuration mode】

Global configuration mode

【Usage】

Local engine cannot be removed, and at most 32 remote engines can be configured.

【Example】

！Configure local engine id to be 12345

OPTIWAY(config)# snmp-server engineid local 12345

！Configure remote engine that can be recognized locally. Configure remote engine ip to be 1.1.1.1,and port number to be 888,and id to be 1234

OPTIWAY(config)# snmp-server engineid remote 1.1.1.1 udp-port 888 1234

！Display local engine configuration

OPTIWAY(config)# show snmp engineid local

    - snmp-server view

Use snmp-server view command to configure view.

snmp-server view view-name oid-tree { included | excluded }

no snmp-server view view-name [ oid-tree ]

【Parameter】

View-name means the name of the view to be added. It ranges from 1 to 32,excluding space.

Oid-tree means the subtree of the view which corresponds to such a mib node as “1.3.6.1”；The substring of OID must be the integer between 2 and 2147483647.

【Default】

iso,internet and sysview are the default views.

【Command configuration mode】

Global configuration mode

【Usage】

At most 64 views can be configured, and the sum of the number of characters in view name string and the number of oid nodes should not be more than 62.

【Example】

！Add view “view1”,and configure it to have a subtree “1.3.6.1”

OPTIWAY(config)# snmp-server view view1 1.3.6.1 include

！Add a subtree “1.3.6.2” for existed view “view1”

OPTIWAY(config)# snmp-server view view1 1.3.6.2 include

！Remove existed view “view1”

OPTIWAY(config)# no snmp-server view view1

    - snmp-server group

Use snmp-server group command to configure group.

snmp-server group groupname { 1 | 2c | 3 [auth | noauth | priv] [context context-name]} [read readview] [ wrete writeview] [notify notifyview]

no snmp-server group groupname {1 | 2c | 3 [auth | noauth | priv] [context context-name]}

【Parameter】

groupname means group name, which ranges from 1 to 32 characters,excluding space.

Readview is a view name, which means the right to read in the view. If the keyword is vacant, it is default not to include readable view.

Writeview is a view name, which means the right to read and write in the view. If the keyword is vacant, it is default not to include readable and writable view.

Notifyview is a view name, which means the right to send notification in the view. If the keyword is vacant, it is default not to include notify sending view.

Context-name is facility context. If the keyword is vacant, it is default to be local facility.

【Default】

Folowing groups are default to exist: (1) security model is v3,the security level is differentiated group initial ; (2) security model is v3,the security level is differentiated encrypt group initial

【Command configuration mode】

Global configuration mode

【Usage】

At most 64 groups can be configured.

【Example】

！Add group “group1” to local facility,using security model 1, and configure read, write, and notify view to be internet

OPTIWAY(config)# snmp-server group group1 1 read internet write internet notify Internet

！Remove group “group1” from local facility

OPTIWAY(config)# no snmp-server group group1 1

！Display current group configuration.

OPTIWAY(config)# show snmp group

    - snmp-server user

Use snmp-server user command to configure user in snmp v3.

snmp-server user username groupname [ remote host [ udp-port port ] ] [ auth { md5 | sha } { authpassword { encrypt-authpassword authpassword | authpassword } |  authkey { encrypt-authkey authkey | authkey } } [ priv des { privpassword { encrypt-privpassword privpassword | privpassword } | privkey { encrypt-privkey privkey | privkey } } ]

no snmp-server user username [ remote host  [ udp-port port ] ]

【Parameter】

Username is the username to be configured. It ranges from 1 to 32 characters,excluding space.

Groupname is the groupname that user going to be added. It ranges from 1 to 32 characters,excluding space.

Host is remote engine ip address. If it is vacant, it is default to be local engine.

Port is the port number of remote engine. If it is vacant, it is default to be 162.

Authpassword is authentication password. Unencrypted password ranges from 1 to 32 characters. To avoid disclosing, this password should be encrypted. To configured encrypted password needs client-side which supports encryption to encrypt password, and use encrypted cryptograph to do the configuration. Cryptograph is different by different encryption. Input cryptograph in the form of hexadecimal system, such as “a22122b32123c45528f91232a4d47a5c”

Privpassword is encryption password. Unencrypted password ranges from 1 to 32 characters. To avoid disclosing, this password should be encrypted. To configured encrypted password needs client-side which supports encryption to encrypt password, and use encrypted cryptograph to do the configuration. Cryptograph is different by different encryption. Input cryptograph in the form of hexadecimal system, such as “a22122b32123c45528f91232a4d47a5c”

Authkey is authentication key. Unauthenticated key is in the range of 16 byte (using md5 key folding) or 22 byte (using SHA-1 key folding). Authenticated key is in the range of 16 byte (using md5 key folding) or 24 byte (using SHA-1 key folding).

Privkey is encrpted key. Unencypted key ranes from 16 byte, and encrypted key ranes from 16 byte.

【Default】

Following users are default to exist: (1)initialmd5（required md5 authentication）,(2) initialsha（required sha authentication）,(3) initialnone（non- authentication）

【Command configuration mode】

Global configuration mode

【Usage】

At most 64 groups can be configured.

【Example】

！Add user “user1” for local engine to group “grp1”,and configure this user not to use authentication and encryption.

OPTIWAY(config)# snmp-server user user1 grp1

！Add user “user2” for local engine to group “grp2”,and configure this user to use md5 authentication and non-encryption with the auth-password to be 1234

OPTIWAY(config)# snmp-server user user2 grp2 auth md5 auth-password 1234

！Add user “user3” for local engine to group “grp3”,and configure this user to use md5 authentication and des encryption with the auth-password to be 1234 and privpassword to be 4321

OPTIWAY(config)# snmp-server user user3 grp3 auth md5 auth-password 1234 priv des priv-password 4321

  - Manage IP Restriction Configuration

Manage IP restriction configuration includes:

- login-access-list

- show login-access-list

    - login-access-list

Use login-access-list command to user’s IP address allowed by web, snmp, and telnet manage system. Use no login-access-list command to remove login-access-list configuration.

login-access-list { snmp | telnet | web } ip-address

no login-access-list { snmp | telnet | web } ip-address wildcard

【Parameter】

ip-address:IP address,2.2.2.2 means any ip address is allowed to manage system except 127.*.*.*

wildcard means mask wildcard which is in the form of mask in reverse. 2 means mask this bit, and 1 ,eams does not mask this bit. When mask in reserve is 2.2.2.2, it means host address, and 255.255.255.255 means all host.

【Command configuration mode】

Global cofiguration mode

【Usage】

Remove ip address 2.2.2.2 so that the configuration can be successful.

【Example】

！Configure ip address allowed by telnet management system to be 192.168.2.122

OPTIWAY(config)#login-access-list telnet 192.168.2.122 2.2.2.2

OPTIWAY(config)#no login-access-list telnet 2.2.2.2 255.255.255.255

    - show login-access-list

Use show login-access-list command to display all ip address allowed by web, snmp, telnet management system.

show login-access-list

【Command configuration mode】

Any configuration mode

【Example】

！Display all ip address allowed by web, snmp, telnet management system

OPTIWAY(config)#show login-access-list

  - Telnet Client

Telnet client command includes:

- telnet

- show telnet client

- stop telnet client

    - telnet

Use telnet command to enable Telnet client.

telnet ip-addr [ port-num ] [ /localecho ]

【Parameter】

ip-addr:IP address of Telnet server.

port-num:Telnet server port which is in the range of 1-65535

/localecho:Enable local echo options.

【Default】

port-num is defaulted to be 23. By default, disable local echo options.

【Command configuration mode】

Privileged mode

【Example】

！Log in a OLT of 12.9.2.34 from telnet client

Optiway#telnet 12.9.2.34

    - show telnet client

Use show telnet client command to display the operation information of all Telnet client.

show telnet client

【Command configuration mode】

Any configuration mode

【Example】

！Display the operation information of all Telnet client.

Optiway#show telnet client

    - stop telnet client

Use stop telnet client command to force to stop Telnet client.

stop telnet client { all | term-id }

【Parameter】

all:Stop all Telnet client.

term-id:The terminal number of Telnet client which is in the range of 2-5,2 means console ,1-5 means Telnet terminal 1-5.

【Command configuration mode】

Privileged mode

【Usage】

This command can only be used by “admin”. User can log in devices by console or telnet and at most 6 users can log in at the same time: a console user and 5 telnet users. The connection of each logged in user and devices is called terminal. Each terminal can enable one telnet client, so at most 6 telnet clients can be run at the same time.

【Example】

！Stop Telnet client in telnet terminal 2

Optiway#stop telnet client 2

  - CPU Alarm Configuration Command

CPU alarm configuration command includes:

- alarm cpu

- alarm cpu threshold

- show alarm cpu

    - alarm cpu

Use alarm cpu command to enable CPU alarm. Use no alarm cpu command to disable CPU alarm.

alarm cpu

no alarm cpu

【Default】

Enable CPU alarm

【Command configuration mode】

Global configuration mode

【Example】

！Enable CPU alarm

Optiway(config)#alarm cpu

    - alarm cpu threshold

Use alarm cpu threshold command to configure CPU busy or unbusy threshold.

alarm cpu threshold [ busy busy ] [ unbusy unbusy ]

no alarm cpu

【Parameter】

busy:CPU busy threshold ranges from 2 to 122

unbusy: CPU unbusy threshold ranges from 2 to 122

【Default】

Default CPU busy threshold is 92,and CPU unbusy threshold is 62

【Command configuration mode】

Global configuration mode

【Usage】

busy > unbusy

【Example】

！Configure CPU busy threshold to be 52,and CPU unbusy threshold to be 32

Optiway(config)#alarm cpu threshold busy 52 unbusy 32

    - show alarm cpu

Use show alarm cpu command to display cpu alarm information.

show alarm cpu

【Command configuration mode】

Any configuration mode

【Example】

！Display CPU alarm information

Optiway(config)#show alarm cpu

  - Anti-DOS Attack

  - anti-dos ip fragment

  - anti-dos ip ttl

  - show anti-dos

    - anti-dos ip fragment

Use anti-dos ip fragment command to configure maximum ip fragment message

anti-dos ip fragment maxnum

【Parameter】

maximum:maximum number

【Default】

822

【Command configuration mode】

Global configuration mode

【Example】

！Configure maximum ip fragment message to be 32

Optiway(config)#anti-dos ip fragment 32

    - show anti-dos

Use Show anti-dos command to display anti-dos information.

Show anti-dos

【Command configuration mode】

Any configuration mode

【Example】

！Display related information

OptiWay(config)#show anti-dos

# LLDP Configuration Command

  - LLDP Configuration Command

LLDP（Link Layer Discovery Protocol）configuration command includes:

- lldp

- lldp hello-time

- lldp hold-time

- lldp { rx | tx | rxtx }

- show lldp interface [ <interface-list> ]

    - lldp

Use lldp command to enable LLDP globally. Use no lldp command to disable LLDP globally.

lldp

no lldp

【Default】

Global LLDP disables

【Command configuration mode】

Global configuration mode

【Example】

！Enable global LLDP

Optiway(config)#lldp

    - lldp hello-time

Use lldp hello-time command to configure LLDP hello-time. Use no lldp hello -time command to restore to default LLDP hello-time.

lldp hello-time <5-32768>

no lldp hello -time

【Default】

Default LLDP hello-time is 32 seconds

【Command configuration mode】

Global configuration mode

【Example】

！Configure LLDP hello-time to be 22 seconds

Optiway(config)#lldp hello-time 22

    - lldp hold-time

Use lldp hold-time command to configure LLDP hold-time. Use no lldp hold-time command to restore LLDP hold-time.

lldp hold-time <2-12>

no lldp hold-time

【Default】

Default LLDP hold-time is 4

【Command configuration mode】

Global configuration mode

【Example】

！Configure LLDP hold-time to be 2

Optiway(config)#lldp hold-time 2

    - lldp { rx | tx | rxtx }

Use lldp command to configure LLDP message receving and sending mode. Use  no lldp command to disable LLDP message receving and sending mode.

lldp { rx | tx | rxtx }

no lldp

【Default】

The default LLDP message receving and sending mode to be rxtx

【Command configuration mode】

Interface configuration mode

【Example】

！Configure e 2/1 only to send LLDP message

Optiway(config-if-ethernet-2/1)#lldp tx

    - show lldp interface [ <interface-list> ]

Use show lldp interface command to display LLDP information globally or on a port.

show lldp interface [ <interface-list> ]

【Command configuration mode】

Any configuration mode

【Example】

！Display LLDP information of e 2/1

Optiway(config)#show lldp interface ethernet 2/1

# Flex links Configuration Command

  - Flex links Configuration Command

    - OLTport backup

Use this command to configure Flex links backup interface.

OLTport backup {interface interface-num | channel-group channel-group-number }

no OLTport backup {interface interface-num | channel-group channel-group-number }

【Default】

It is defaulted not to configure Flex links backup interface.

【Command configuration mode】

Interface configuration mode

【Example】

！Configure flex links backup interface of e2/1 to be e2/2

Optiway(config-if-ethernet-2/1)#OLTport backup interface Ethernet 2/2

    - channel-group channel-group-number backup

Use this command to configure Flex links backup interface of channel group.

channel-group channel-group –number backup {interface interface-num | channel-group channel-group-number }

no channel-group channel-group –number backup {interface interface-num | channel-group channel-group-number }

【Default】

It is defaulted not to configure Flex links backup interface of channel-group.

【Command configuration mode】

Global configuration mode

【Example】

！Configure flex links backup interface of channel-group 1 to be e2/2

Optiway(config)#channel-group 1 backup interface Ethernet 2/2

    - OLTport backup preemption mode

Use this command to configure Flex links preemption mode of master interface.

OLTport backup {interface interface-num | channel-group channel-group-number } preemption mode {Forced | Bandwidth | Off}

no OLTport backup {interface interface-num | channel-group channel-group-number } preemption mode {Forced | Bandwidth | Off}

【Default】

The default Flex links preemption mode of master interface is Off.

【Command configuration mode】

Interface configuration mode

【Example】

！Configure flex links preemption mode of e2/1 to be Forced

Optiway(config-if-ethernet-2/1)#OLTport backup interface Ethernet 2/2 preemption mode Forced

    - channel group backup preemption mode

Use this command to configure Flex links preemption mode of channel-group.

channel group channel-group-number backup {interface interface-num | channel-group channel-group-number } preemption mode {Forced | Bandwidth | Off}

no channel group channel-group-number backup {interface interface-num | channel-group channel-group-number } preemption mode {Forced | Bandwidth | Off}

【Default】

The default Flex links preemption mode of channel-group is Off.

【Command configuration mode】

Global configuration mode

【Example】

！Configure flex links preemption mode of channel-group 1 to be Forced

Optiway(config)#channel-group 1 backup interface Ethernet 2/2 preemption mode Forced

    - OLTport backup preemption delay

Use this command to configure Flex links preemption delay.

OLTport backup {interface interface-num | channel-group channel-group-number } preemption delay delay-time

no OLTport backup {interface interface-num | channel-group channel-group-number } preemption delay delay-time

【Default】

The default Flex links preemption delay is 45 seconds.

【Command configuration mode】

Interface configuration mode

【Example】

！Configure flex links preemption delay of e2/1 to be 62 seconds

Optiway(config-if-ethernet-2/1)#OLTport backup interface Ethernet 2/2 preemption delay 62

    - channel-group backup preemption delay

Use this command to configure Flex links preemption delay of channel-group.

channel-group channel-group-number  backup {interface interface-num | channel-group channel-group-number } preemption delay delay-time

no channel-group channel-group-number  backup {interface interface-num | channel-group channel-group-number } preemption delay delay-time

【Default】

The default Flex links preemption delay of channel-group is 45 seconds.

【Command configuration mode】

Global configuration mode

【Example】

！Configure flex links preemption delay of channel-group 1 to be 62 seconds

Optiway(config)#channel-group 1 backup interface Ethernet 2/2 preemption delay 62

    - show swithport interface backup

This command is to display all Flex links interfaces

show OLTport interface backup

【Command mode】

All modes

【Example】

！Display all Flex links interfaces

OptiWay(confi)# show OLTport interface backup

    - mac-address-table move update transmit

This command is to enable MacMoveUpdate transmit

mac-address-table move update transmit

【Command mode】

All global configuration mode

【Example】

！Enable MacMoveUpdate transmit

OptiWay(config)#mac-address-table move update transmit

    - mac-address-table move update receive

This command is to enable MacMoveUpdate receive

mac-address-table move update receive

【Command mode】

All global configuration mode

【Example】

！Enable MacMoveUpdate receive

OptiWay(config)#mac-address-table move update receive

    - show swithport interface backup

Use this command to display Flex links interfaces.

show OLTport interface backup

【Command configuration mode】

Any configuration mode

【Example】

！Display all Flex links information

Optiway(confi)# show OLTport interface backup

# CFM Configuration Command

CFM（Connectivity Fault Management）Configuration Command includes:

- cfm domain

- cfm mep

- cfm mip

- cfm rmep

- cfm cc interval

- cfm cc enable level vlan

- cfm ping

- cfm traceroute

- show cfm domain

- show cfm maintenance-points local

- show cfm maintenance-points remote

- show cfm cc database

- show cfm errors

    - cfm md

Use this command to configure cfm maintance domain. Use no command to delete cfm domain.

cfm md md-id（index of maintenance domain）

cfm md format string name name level level-id

no cfm md md-id

【Default】

It is defaulted not to configure cfm domain.

【Command configuration mode】

Global configuration mode

【Parameter】

domain-name:CFM domain name

level-id: the integrity from 2-7

【Example】

！Configure cfm domain customer level 7

OptiWay(config)#cfm md 1

OptiWay(config-cfm-md-1)#cfm md format string name customer level 7

    - cfm mep

Use this command to configure cfm mep. Use no command to delete cfm mep.

cfm mep level level-id direction {up | down } mpid mep-id vlan vlan-id

no cfm mep level level-id vlan vlan-id

【Default】

It is defaulted not to configure cfm mep.

【Command configuration mode】

Interface configuration mode

【Parameter】

level-id: the integrity from 2-7

up: direction of MEP

down: direction of MEP

mep-id: MEP id

vlan-id: VLAN of MEP

【Example】

！Configure cfm mep level 7 direction up mpid 7112 vlan 112

Optiway(config-if-ethernet-2/1)#cfm mep level 7 direction up mpid 7112 vlan 112

    - cfm mip

Use this command to configure cfm mip. Use no command to delete cfm mip.

cfm mip level level-id

no cfm mep level level-id

【Default】

It is defaulted not to configure cfm mip.

【Command configuration mode】

Interface configuration mode

【Parameter】

level-id: the integrity from 2-7

【Example】

！Configure cfm mip level 7

Optiway(config-if-ethernet-2/1)#cfm mip level 7

    - cfm rmep

Use this command to configure cfm rmep. Use no command to delete cfm rmep.

cfm rmep level level-id mpid mep-id vlan vlan-id

no cfm rmep level level-id mpid mep-id vlan vlan-id

【Default】

It is defaulted not to configure cfm rmep.

【Command configuration mode】

Global configuration mode

【Parameter】

level-id: the integrity from 2-7

mep-id: MEP id

vlan-id: VLAN of MEP

【Example】

！Configure cfm rmep level 7 mpid 7112 vlan 112

Optiway(config)#cfm rmep level 7 mpid 7112 vlan 112

    - cfm cc interval

Use this command to configure cfm cc interval. Use no command to restore default cfm cc interval

cfm cc interval { 1 |12 | 62 }

no cfm cc interval

【Default】

The default cfm cc interval is 12s

【Command configuration mode】

Global configuration mode

【Parameter】

1: sending interval is 1 second

12: sending interval is 12 seconds

62: sending interval is 62 seconds

【Example】

！Configure cfm cc interval to be 1s

Optiway(config)#cfm cc interval 1

    - cfm loopback

Use this command to check network connection and the arrival of destination mac address.

cfm loopback mep mep-id dst-mac mac-address

【Command configuration mode】

Maintanance domain configuration mode

【Parameter】

Mep-id:mep identifer

mac-address:Destination mac address.

    - cfm linktrack

Use this command for link tracert and checking network connection.

cfm linktrack mep mep-id dst-mac mac-address

【Command configuration mode】

Global configuration mode

【Parameter】

Mep-id:mep identifer

mac-address:Destination mac address.

    - show cfm md

Use this command to display cfm domain.

show cfm domain

【Command configuration mode】

Any configuration mode

【Example】

！Display cfm domain

Optiway(config)#show c

Optiway(config)#show cfm md

    - show cfm mp local

Use this command to display cfm maintenance-points local.

show cfm mp local

【Command configuration mode】

Any configuration mode

【Example】

！Display cfm maintenance-points local

Optiway(config)# show cfm mp local

    - show cfm mp remote

Use this command to display cfm maintenance-points remote.

show cfm mp remote

【Command configuration mode】

Any configuration mode

【Example】

！Display cfm maintenance-points remote

Optiway(config)# show cfm mp remote

    - show cfm cc database

Use this command to display cfm cc database.

show cfm cc database

【Command configuration mode】

Any configuration mode

【Example】

！Display cfm cc database

Optiway(config)# show cfm cc database

    - show cfm errors

Use this command to display cfm errors.

show cfm errors

【Command configuration mode】

Any configuration mode

【Example】

！Display cfm errors

Optiway(config)# show cfm errors

# PPPoE PlusConfiguration Command

  - PPPoE PlusConfiguration Command

  - Pppoeplus

  - pppoeplus type

  - show pppoeplus

    - pppoeplus

Use this command to enable PPPoE Plus. Use no command to disable PPPoE Plus.

pppoeplus

no pppoeplus

【Command configuration mode】

Any configuration mode

【Example】

！Enable PPPoE Plus

Optiway(config)#pppoeplus

！Disble PPPoE Plus

- Optiway(config)#no pppoeplus

    - pppoeplus type

Use this command to configure PPPoE Plus type.

Pppoeplus type { standard | huawei }

【Command configuration mode】

Global configuration mode

【Example】

！Configure PPPoE Plus type being huawei BRAS

- Optiway(config)#pppoeplus type huawei

    - show pppoeplus

Use this command to display pppoe plus.

show pppoeplus

【Command configuration mode】

Any configuration mode

【Example】

！Display pppoe plus

Optiway(config)#show pppoeplus

# ERRP Configuration Command

  - ERRP Configuration Command

ERRP（Ethernet Redundant Ring Protocol）Configuration Command includes:

- errp

- errp hello-timer

- errp fail-timer

- errp domain

- ring role primary-port secondary-port level

- ring role common-port edge-port

- ring { enable | disable }

- show errp

- ring query-solicit

    - errp

Use this command to enable global ERRP. Use no command to disable it.

errp

no errp

【Default】

Disable

【Command configuration mode】

Global configuration mode

【Example】

！Enable ERRP

Optiway(config)#errp

    - errp hello-timer

Use this command to configure hello-time of ERRP. Use no command to restore default hello-time.

errp hello-timer <1-12>

no errp hello –timer

【Default】

default hello-time is 1 second

【Command configuration mode】

Global configuration mode

【Example】

！Configure ERRP hello-time to be 2 seconds

Optiway(config)#errp hello-timer 2

    - errp fail-timer

Use this command to configure ERRP fail-time. Use no command to restore default fail-time.

errp fail-timer <1-12>

no errp fail-timer

【Default】

Default fail-time of ERRP is 3

【Command configuration mode】

Global configuration mode

【Example】

！Configure ERRP fail-time to be 2

Optiway(config)#errp fail-timer 2

    - errp domain

Use this command to enter ERRP domain configuration mode.

errp domain domain-id

【Command configuration mode】

Global configuration mode

【Parameter】

domain-id:ERRP domain id which is in the range of <2-15>

【Example】

！Configure ERRP domain 2

Optiway(config)#errp domain 2

    - control-vlan

Use this command to configure control VLAN of ERRP domain. Use no command to delete it.

control-vlan vlan-id

no control-vlan

【Command configuration mode】

ERRP domain configuration mode

【Parameter】

vlan-id:control vlan id of ERRP domain which is the integrty in the range of 1-4293.

【Usage】

Control VLAN is relative to data VLAN. Data VLAN is for transmitting date packet and control VLAN is only for transmitting ERRP protocol packet. Every ERRP domain owns two control VLANs,that are master control VLAN and sub-control VLAN. Protocol packet of master ring is transmitted in master control-VLAN and protocol packet of sub-ring is transmitted in sub-control VLAN. When configuring, specify master control. When configuring, specify master control VLAN,and sub-control VLAN is the one whose VLAN ID is 1 bigger than that of the master control VlAN.

Port only accessing to Ethernet ring (ERRP port) of each OLT belong to control VLAN. ERRP port of master ring belong to both master control VLAN and sub-control VLAN. ERRP port of sub-ring belongs to sub-control VLAN only. There can be ERRP port and non- ERRP port in data VLAN. Master ring is taken as a logical nod of sub-ring. The protocol packet of sub-ring is transparent transmitted through master ring and handled as data packet in master ring. The protocol packet of master ring can only be transmitted in master ring.

Add all ERRP port to corresponded master and sub-control VLAN before or after handed down ERRP configuration and configure master and sub-control VLAN being tag vlan.

【Example】

！Configure control VLAN of ERRP domain 2 being 25

Optiway(config-errp-2)#control-vlan 25

！Delete control VLAN of ERRP domain 2. if there is activated ring, the control VLAN will not allow to be deleted.

Optiway(config-errp-2)#no control-vlan

    - ring role primary-port secondary-port level

Use this command to configure a ring. Network bridge in this ring is master role or transmission role.

ring ring-id role { master | transit } primary-port interface-pri secondary-port interface-sec level level-value

no ring ring-id

【Command configuration mode】

ERRP domain configuration mode

【Parameter】

ring-id:ring id which is in the range of <2-15>

master: Network bridge in this ring is master role

transit:Network bridge in this ring is transit role

interface-pri: primary port id such as ethernet 2/1

interface-sec:secondary port id such as ethernet 2/1

level-value:ring level. 2 means primary ring and 1 means secondary

【Example】

！Configure primary ring 2 with role mode being master, primary port being 1 and secondary port being 2

Optiway(config-errp)#ring 1 role master primary-port ethernet 2/1 secondary-port ethernet 2/2 level 2

    - ring role common-port edge-port

Use this command to configure a ring. Network bridge in this ring is edge role or assistant edge.

ring ring-id role { edge | assistant-edge } common-port interface-common edge-port interface-edge

no ring ring-id

【Command configuration mode】

ERRP domain configuration mode

【Parameter】

ring-id:ring id which is in the range of <2-15>

edge: Network bridge in this ring is edge role

assistant-edge:Network bridge in this ring is assistant-edge role

interface-common: common port id such as ethernet 2/1

interface-edge:interface port id such as ethernet 2/1

【Example】

！Configure primary ring 2 with role mode being master, primary port being 1 and secondary port being 2

Optiway(config-errp)#ring 1 role edge common-port ethernet 2/1 edge-port ethernet 2/2

    - ring { enable | disable }

Use this command to activate / inactivate a ring.

ring ring-id { enable | disable }

【Command configuration mode】

ERRP domain configuration mode

【Parameter】

ring-id:ring id which is in the range of <2-15>

enable: activate a ring

disable:inactivate a ring

【Example】

！Activate ring 2

Optiway(config-errp)#ring 2 enable

    - show errp

Use this command to display ERRP.

show errp [ domain domain-id [ ring ring-id ] ]

【Command configuration mode】

Any configuration mode

【Parameter】

domain-id:errp domain which is in the range of <2-15>

ring-id:ring id which is in the range of <2-15>

【Example】

！Display errp domain 2 ring 2

Optiway(config)#show errp domain 2 ring 2

    - ring query-solicit

use this command to enable Query Solicitation of ERRP ring. Use no command to disable it.

ring ring-id query-solicit

no ring ring-id query-solicit

【Command configuration mode】

ERRP domain configuration mode

【Parameter】

ring-id:ring id which is in the range of 2-15.

【Example】

！Enable Query Solicitation of ERRP domain 2 ring 2

Optiway(config-errp-domain-2)#ring 2 query-solicit

# EFM Configuration Command

- 20.1 EFM Configuration Command

EFM（ethernet of first mile ）command includes:

- efm

- efm mode

- efm pdu-timeout

- efm link-timeout

- efm remote-response-timeout

- efm link-monitor para

- efm remote-failure

- efm link-monitor

- efm variable-retrieval

- efm remote-loopback

- efm remote-loopback { start | stop }

- efm remote-loopback { ignore | process }

- show efm status interface

- show efm summary

- show efm discovery interface

- show / clear efm statistics interface

- show efm remote-mib

    - efm

This command is to enable EFM or disable EFM.

efm

no efm

【Default】

By default disable EFM.

【Command mode】

Port configration mode

【Example】

！Enable EFM

TiNet(config-if-ethernet-0/1)#efm

    - efm mode

This command is to configure EFM work mode as passive/ active.

efm mode { passive | active }

【Default】

By default is active mode.

【Command mode】

Port configration mode

【Parameter】

passive:

active:

【Example】

！Configure EFM work mode as passive

TiNet(config-if-ethernet-0/1)#efm mode passive

    - efm pdu-timeout

This command is to configure discover period of EFM, command with no is recover discover period of EFM to be default.

efm pdu-timeout time

no efm pdu-timeout

【Default】

By default is 1s

【Command mode】

Port configration mode

【Parameter】

time:discover period of EFM ，unit is s，range is 1s～60s，this value s less than 1/3 timeout.

【Example】

！Configure EFM discover period is 5s

TiNet(config-if-ethernet-0/1)#efm pdu-timeout 5

    - efm link-timeout

This command is to configure EFM discover timeout, command with no is recover discover period of EFM to be default.

efm link-timeout time

no efm link-timeout

【Default】

By default is 5s

【Command mode】

Port configration mode

【Parameter】

time:EFM discove timeout，unit is s，range is 3s～300s，this value is less than 3 times of EFM discover period.

【Example】

！Configure EFM discover timeout is 15s

TiNet(config-if-ethernet-0/1)#efm link-timeout 15

    - efm remote-response-timeout

This command is configure timeout of remote OAMPDU request packet, command with no is recover the default.

efm remote-response-timeout time

no efm remote-response-timeout

【Default】

By default is 2s

【Command mode】

Port configration mode

【Parameter】

time:response time，unit is s，range is 1s～10s

【Example】

！configure response time as 5s

TiNet(config-if-ethernet-0/1)#efm remote-response-timeout 5

    - efm link-monitor para

This command is to configure window/threshold value of EFM link monitor, command with no is recover the default.

efm link-monitor errored-symbol-period window high win-value1 low win-value2

efm link-monitor errored-symbol-period threshold high th-value1 low th-value2

【Parameter】

window:means qty of receiving symbol （length is 8 bytes），range is :1～0xffffffffffffffff，default is 10000，win-value1 and win-value2 means high 4 bytes and low 4 bytes

threshold:means qty of receiving wrong symbol（length is 8 bytes），range is :1～0xffffffffffffffff，by default is 1，th-value1and th-value2 high 4 bytes and low 4 bytes.

efm link-monitor errored-frame window win-value

efm link-monitor errored-frame threshold th-value

【Parameter】

win-value:means receiving time，range is :10(100ms)～600(100ms)，default is 10(100ms)

th-value:means receiving wrong packets ，range is :1～0xffffffff， default is 1

efm link-monitor errored-frame-period window win-value

efm link-monitor errored-frame-period threshold th-value

【Parameter】

win-value:means receiving time，range is :1～0xffffffff，default is 10000

th-value:means receiving wrong packets，range is :1～0xffffffff，By default is 1

efm link-monitor errored-frame-seconds window win-value

efm link-monitor errored-frame-seconds threshold th-value

【Command mode】

Port configration mode

【Example】

！Configure window of errored-symbol-period as 50000

TiNet(config-if-ethernet-0/1)#efm link-monitor errored-symbol-period window high 0 low 50000

    - efm remote-failure

This command is to enable remote failure ,command with no is recover the default.

efm remote-failure { link-fault | dying-gasp | critical-event }

no efm remote-failure { link-fault | dying-gasp | critical-event }

【Default】

By default, enable remote failure

【Command mode】

Port configration mode

【Parameter】

link-fault: means failures from local receiving direction.

dying-gasp: temepartely not define testing method.

critical-event: temepartely not define testing method.

【Example】

！Disable link-fault

TiNet(config-if-ethernet-0/1)#no efm remote-failure link-fault

    - efm link-monitor

This command is to enable link monitor, command with no is recover the default.

efm link-monitor { errored-symbol-period | errored-frame | errored-frame-period | errored-frame-seconds }

no efm link-monitor { errored-symbol-period | errored-frame | errored-frame-period | errored-frame-seconds }

【Default】

By default, enable link monitor.

【Command mode】

Port configration mode

【Parameter】

errored-symbol-period: qty exceed the threshold of recent wrong symbol

errored-frame: wrong packets exceed the threshold of certain time.

errored-frame-period: error seconds in N period

errored-frame-seconds: error seconds in M seconds (one second at least one error frame) exceeds the threshold

【Example】

！Enable errored-frame

TiNet(config-if-ethernet-0/1)#efm link-monitor errored-frame

    - efm variable-retrieval

This command is to enable remote MIB variable，command with no is recover the default.

efm variable-retrieval

no efm variable-retrieval

【Default】

By default, enable remote MIB variable retrieval.

【Command mode】

Port configration mode

【Example】

！Disable remote MIB variable retrieval.

TiNet(config-if-ethernet-0/1)#no efm variable-retrieval

    - efm remote-loopback

This command is to enable remote loopback, command with no is recover the default.

efm remote-loopback

no efm remote-loopback

【Default】

By default, disable remote loopback.

【Command mode】

Port configration mode

【Example】

！Disable remote loopback.

TiNet(config-if-ethernet-0/1)#no efm remote-loopback

    - efm remote-loopback { start | stop }

This command is to start/stop remote loopack.

efm remote-loopback { start | stop }

【Command mode】

Port configration mode

【Parameter】

start: enable remote loopback.

stop: disable remote loopback

【Example】

！Enable remote loopback

TiNet(config-if-ethernet-0/1)#efm remote-loopback start

    - efm remote-loopback { ignore | process }

This command is to configure the port whether deal with the OAMPDU from remote request.

efm remote-loopback { ignore | process }

【Default】

By default, it can’t deal with OAMPDU from remote request.

【Command mode】

Port configration mode

【Parameter】

ignore:

process:

【Example】

！Configure OAMPDU from remote request.

TiNet(config-if-ethernet-0/1)#efm remote-loopback process

    - show efm status interface

This command is to display EFM running status.

show efm status interface [ interface-name ]

【Command mode】

All modes

【Parameter】

interface-name: check EFM running status.

【Example】

！Show all ports running status.

TiNet(config)#show efm status interface

    - show efm summary

This command is to show EFM summary.

show efm summary

【Command mode】

Any mode

【Example】

！Show efm summary.

TiNet(config)#show efm summary

    - show efm discovery interface

This command is to show EFM discovery information.

show efm discovery interface [ interface-name ]

【Command mode】

Any mode

【Parameter】

interface-name: check EFM discovery information

【Example】

！Show all ports EFM discovery information

TiNet(config)#show efm discovery interface

    - show / clear efm statistics interface

This command is to show/clear EFM statistics.

show efm statistics interface [ interface-name ]

clear efm statistics interface [ interface-name ]

【Command mode】

In global configuration mode

【Parameter】

interface-name: show/clear EFM statistics

【Example】

！Show all ports of EFM statistics

TiNet(config)#show efm statistics interface

    - show efm remote-mib

This command is to display remote MIB variable.

show efm port port-id-list remote-mib { phyadminstate | autonegadminstate }

show efm remote-mib { fecability | fecmode }

【Command mode】

Port configration mode

【Parameter】

port-id-list: be checked ports list

phyadminstate: port enable status

autonegadminstate: port negotiate enable status.

fecability:FEC ability

fecmode:FEC mode

【Example】

！Show port1 enable status

TiNet(config-if-ethernet-0/1)#show efm port 1 remote-mib  phyadminstate

# OLT Slot Management Configuration Command

  - OLT Slot Management Configuration Command

- set slot slot-id type { pon-14 | ge-24| 10ge }

- no slot slot-id type

- show slot type

- show pon {slot slot-id | mac mac-address}

    - set slot

This command is used to configure insert card type in the slot, command with no means delete the insert card type in the slot.

set slot slot-id type { pon-14| ge-24| 10ge }

no slot slot-id type

【Command mode】

All global configuration mode

【Example】

！Configure slot 2 as ge-24 type

OptiWay(config)#set slot 2 type ge-24

    - show slot

This command is used to display slot information

show slot type

【Command mode】

Any configuration mode

【Example】

！Display slot information

OptiWay(config)#show slot type

    - show pon

This command is used to display on line business slot and ONU information

show pon { slot slot-id | mac mac-address }

【Parameter 】

slot-id:slot

mac-address:ONU mac address

【Command mode】

Any configuration mode

【Example】

！display on line business slot and ONU information

OptiWay(config)#show pon

Slot    3

ONU   Mac Address       LLID     Config Description

3/3/1 00:0a:5a:12:4a:db     60820000 OK

Total onu entries: 1 .

# PON Configuration Command

  - PON Configuration Command

- onu-authenticate

- white-list

- black-list

- loid-list

- hybrid-list

- onu-p2p

- encrypt

- mac-address-table

- class

- enable-pon-vlan-isolation

- show onu-mac-auth

- show white-list

- show black-list

- show loid-list

- show hybrid-list

- show onu-p2p

- show dba

- show mac-address-table

- show class

- show enable-pon-vlan-isolation

    - onu-authenticate

This command is used to configure anthentication based on MAC address,ONU authentication based on logic idenfier and hybrid authentication.

onu-authenticate slot slotid mode { { mac-auth white-list | black-list } |

loid-auth | hybrid-auth | disable }

onu-authenticate mode { { mac-auth white-list | black-list } | loid-auth |

hybrid-auth | disable }

【Parameter】

slot:slot

【Command mode】

All global configuration mode and PON port configuration mode

【Example】

！Enable slot 5 ONU MAC authentication white list

OptiWay(config)#onu-authenticate 5 mode mac-auth white-list

！Enable PON hybrid authentication

OptiWay(config-if-pon-3/3)#onu-authenticate mode hybrid-auth

    - white-list

This command is used to add/delete white list

white-list add onu-id H:H:H:H:H:H

white-list del { all | onu-id }

【Command mode】

PON port configuration mode

【Example】

！Add white list

OptiWay(config-if-pon-3/1)#white-list add 1 00:11:22:33:44:55

！Delete white list

OptiWay(config-if-pon-3/1)#white-list del 1

    - black-list

This command is used to add/delete black list

black-list add H:H:H:H:H:H

black-list del { all | mac-id | H:H:H:H:H:H }

【Command mode】

PONport configuration mode

【Example】

！Add black list

OptiWay(config-if-pon-3/1)#black-list add 00:11:22:33:44:55

！Delete black list

OptiWay(config-if-pon-3/1)# black -list del 1

    - loid-list

This command is used to add, delete logic id authentication entry

loid-list add loid loid-id password pwd-id

loid-list del { all | index index-id | loid loid-id }

【Command mode】

PONport configuration mode

【Example】

！Add logic id authentication entry

OptiWay(config-if-pon-3/1)# loid-list add loid bbbbb password 12345

！Delete logic id authentication entry

OptiWay(config-if-pon-3/1)# loid-list del 1

    - hybrid-list

This command is used to add, delete hybird authentication entry

hybrid-list add { loid loid-id password pwd-id | mac H:H:H:H:H:H }

hybrid-list del { all | index index-id | loid loid-id }

【Command mode】

PONport configuration mode

【Example】

！Add hybird authentication entry

OptiWay(config-if-pon-3/1)# hybrid-list add loid aaaabbbbcccc password 1234

OptiWay(config-if-pon-3/1)# hybrid-list add mac 0:0:0:0:12:34

！Delete hybird authentication entry

OptiWay(config-if-pon-3/1)# hybrid-list del 1

    - onu-p2p

This command is used to enable ONU intersession,with command no to disable ONU intersession.

onu-p2p

no onu-p2p

【Command mode】

PONport configuration mode

【Example】

！Enable ONU intersession

OptiWay(config)# onu-p2p

！Disable ONU intersession

OptiWay(config)# no onu-p2p

    - encryp

This command is used to enable downlink encryption ,with command no to disable downlink encryption

encryp churn slot rekey-time

encryp 32_aes slot rekey-time

encryp 48_aes slot rekey-time

no encryp slot

【Parameter 】

slot:slot

rekey-time: key refresh time

【Command mode】

All global configuration mode

【Example】

！Enable downlink 3 layer jitter encryption

OptiWay(config)#encryp churn 5 1000000

！Disable downlink 3 layer jitter encryption

OptiWay(config)#no encryp 5

    - dba

This command is for dba related parameter.

dba algorithm slot { cbr | nonworkconserv | workconserv }

dba mode slot { hw | sw | hw-tuning | sw-tuning }

dba params slot pon-num cycletime discovfreq discovtime

【Parameter 】

slot:slot

pon-num:PON port

cycletime:cycle time

discovfreq:discover frequency

discovtime:discover time

【Command mode】

All global configuration mode

【Example】

！Configure DBA algorithm as cbr

OptiWay(config)# dba algorithm 2 cbr

- ！Configure DBA mode as hardware dispatch

OptiWay(config)# dba mode 2 hw

！Configure PON port optical parameter

OptiWay(config)# dba params 2 1 125000 255 65500

    - mac-address-table

This command is used to configure MAC address learning quantity limitation of ONU LLID

mac-address-table onu-index { llid_index | all } max-mac-count number

【Parameter 】

llid_index: ONU LLID index

number:MAC address learning quantity limitation,the range is 1－1000

【Command mode】

PON port configuration mode

【Example】

！Configure MAC address learning quantity limitation as 20 of the first LLID

OptiWay(config-if-pon-3/1)#mac-address-table onu-index 1

max-mac-count 20

！Configure MAC address learning quantity limitation as 20 of all LLID

OptiWay(config-if-pon-3/1)#mac-address-table onu-index all

max-mac-count 20

    - classif

This command is used to configure flow classification rule

classif ruleid { pon | onu onu_id | nni } permit packet ip

{   [ ip-da ip ] | [ destination-port value ] |

[ dot1p-priority value ] | [ dscp value ] |

[ ip-protocol { id | icmp | igmp | tcp | udp | esp-ah | pim } ] |

[ ip-sa ip ] | [ source-port value ] | [ vlan vlanStart_id vlanEnd_id ] } *

[ ethernet-priority-action value ]

[ vlan-action { push vlanid | translation vlanid | pop } ]

classif ruleid { pon | onu onu_id | nni } permit packet arp

{   [ ip-da ip ] | [ destination-port value ] |

[ dot1p-priority value ] | [ dscp value ] |

[ ip-sa ip ] | [ source-port value ] | [ vlan vlanStart_id vlanEnd_id ] } *

[ ethernet-priority-action value ]

[ vlan-action { push vlanid | translation vlanid | pop } ]

classif ruleid { pon | onu onu_id | nni } permit packet eth

{  [ destination-mac H:H:H:H:H:H ] |

[ dot1p-priority value ] | [ ethernet-type value ] |

[ ip-protocol { id | icmp | igmp | tcp | udp | esp-ah | pim } ] |

[ vlan vlanStart_id vlanEnd_id ] } *

[ ethernet-priority-action value ]

[ vlan-action { push vlanid | translation vlanid | pop } ]

classif ruleid { pon | onu onu_id | nni } permit packet any

{  [ destination-mac H:H:H:H:H:H | dot1p-priority value ] |

[ ip-protocol { id | icmp | igmp | tcp | udp | esp-ah | pim } ] |

[ vlan vlanStart_id vlanEnd_id ] } *

[ ethernet-priority-action value ]

[ vlan-action { push vlanid | translation vlanid | pop } ]

classif ruleid { pon | onu onu_id | nni } deny packet ip

{   [ ip-da ip ] | [ destination-port value ] |

[ dot1p-priority value ] | [ dscp value ] |

[ ip-protocol { id | icmp | igmp | tcp | udp | esp-ah | pim } ] |

[ ip-sa ip ] | [ source-port value ] | [ vlan vlanStart_id vlanEnd_id ] } *

classif ruleid { pon | onu onu_id | nni } deny packet arp

{   [ ip-da ip ] | [ destination-port value ] |

[ dot1p-priority value ] | [ dscp value ] |

[ ip-protocol { id | icmp | igmp | tcp | udp | esp-ah | pim } ] |

[ ip-sa ip ] | [ source-port value ] | [ vlan vlanStart_id vlanEnd_id ] } *

classif ruleid { pon | onu onu_id | nni } deny packet eth

{  [ destination-mac H:H:H:H:H:H ] |

[ dot1p-priority value ] | [ ethernet-type value ] |

[ ip-protocol { id | icmp | igmp | tcp | udp | esp-ah | pim } ] |

[ vlan vlanStart_id vlanEnd_id ] } *

classif ruleid { pon | onu onu_id | nni } deny packet any

{  [ destination-mac H:H:H:H:H:H | dot1p-priority value ] |

[ ip-protocol { id | icmp | igmp | tcp | udp | esp-ah | pim } ] |

[ vlan vlanStart_id vlanEnd_id ] } *

classif slot slotid { pon | nni } permit packet ip

{   [ ip-da ip ] | [ destination-port value ] |

[ dot1p-priority value ] | [ dscp value ] |

[ ip-protocol { id | icmp | igmp | tcp | udp | esp-ah | pim } ] |

[ ip-sa ip ] | [ source-port value ] | [ vlan vlanStart_id vlanEnd_id ] } *

[ ethernet-priority-action value ]

[ vlan-action { push vlanid | translation vlanid | pop } ]

classif slot slotid { pon | nni } permit packet arp

{   [ ip-da ip ] | [ destination-port value ] |

[ dot1p-priority value ] | [ dscp value ] |

[ ip-protocol { id | icmp | igmp | tcp | udp | esp-ah | pim } ] |

[ ip-sa ip ] | [ source-port value ] | [ vlan vlanStart_id vlanEnd_id ] } *

[ ethernet-priority-action value ]

[ vlan-action { push vlanid | translation vlanid | pop } ]

classif slot slotid { pon | nni } permit packet eth

{ [ destination-mac H:H:H:H:H:H ] |

[ dot1p-priority value ] | [ ethernet-type value ] |

[ ip-protocol { id | icmp | igmp | tcp | udp | esp-ah | pim } ] |

[ vlan vlanStart_id vlanEnd_id ] } *

[ ethernet-priority-action value ]

[ vlan-action { push vlanid | translation vlanid | pop } ]

classif slot slotid { pon | nni } permit packet any

{ [ destination-mac H:H:H:H:H:H | dot1p-priority value ] |

[ ip-protocol { id | icmp | igmp | tcp | udp | esp-ah | pim } ] |

[ vlan vlanStart_id vlanEnd_id ] } *

[ ethernet-priority-action value ]

[ vlan-action { push vlanid | translation vlanid | pop } ]

classif slot slotid { pon | nni } deny packet ip

{   [ ip-da ip ] | [ destination-port value ] |

[ dot1p-priority value ] | [ dscp value ] |

[ ip-protocol { id | icmp | igmp | tcp | udp | esp-ah | pim } ] |

[ ip-sa ip ] | [ source-port value ] | [ vlan vlanStart_id vlanEnd_id ] } *

classif slot slotid { pon | nni } deny packet arp

{   [ ip-da ip ] | [ destination-port value ] |

[ dot1p-priority value ] | [ dscp value ] |

[ ip-protocol { id | icmp | igmp | tcp | udp | esp-ah | pim } ] |

[ ip-sa ip ] | [ source-port value ] | [ vlan vlanStart_id vlanEnd_id ] } *

classif slot slotid { pon | nni } deny packet eth

{  [ destination-mac H:H:H:H:H:H ] |

[ dot1p-priority value ] | [ ethernet-type value ] |

[ ip-protocol { id | icmp | igmp | tcp | udp | esp-ah | pim } ] |

[ vlan vlanStart_id vlanEnd_id ] } *

classif slot slotid { pon | nni } deny packet any

{  [ destination-mac H:H:H:H:H:H | dot1p-priority value ] |

[ ip-protocol { id | icmp | igmp | tcp | udp | esp-ah | pim } ] |

[ vlan vlanStart_id vlanEnd_id ] } *

【Parameter 】

ruleid:flow classification rule index,the range is 1－99

onu_id:ONU index,the range is 1－64

【Command mode】

PONport configuration mode and all global configuration mode

【Example】

！Configure interface 1 onu 1 flow classification rule in business card pon3 :

Select data packets as IP, source IP address is 192.168.0.32 ,vlan tag packets of vlanid=100 ,that packets mark outside vlan,the value is vlan-id=200

OptiWay(config-if-pon-3/1)#class 1 onu 1 permit packet ip source-ip 192.168.0.32 vlan 100 vlan-action push 200

    - no classif

This command is to delete PON flow classification rule.

no classif [ ruleid ]

no classif [ slot slotid ]

【Command mode】

PON port configuration mode and all global configuration mode

【Example】

！Delete interface 1 onu 1 flow classification rule in business card pon3 OptiWay(config-if-pon-3/1)#no class 1

    - enable-pon-vlan-isolation

This command is to enable or disable VLAN isolation mechanism

enable-pon-vlan-isolation vlan vlan0-id vlan1-id vlan2-id vlan3-id
no enable-pon-vlan-isolation

【Parameter 】

Vlan0-id:isolate VLAN id corresponding to PON 1
Vlan1-id:isolate VLAN id corresponding to PON 2
Vlan2-id:isolate VLAN id corresponding to PON 3
Vlan3-id:isolate VLAN id corresponding to PON 4

【Command mode】

All global configuration mode

【Example】

！Enable VLAN isolation mechanism and configure isolate VLAN id as 100 200 300 400
OptiWay(config)#enable-pon-vlan-isolation 100 200 300 400
! Disable VLAN isolation mechanism
OptiWay(config)#no enable-pon-vlan-isolation

    - show onu-mac-auth

This command is to display ONU MAC authentication.

show onu-mac-auth mode

show onu-mac-auth mode [ slot slotid [ pon ponid ] ]

【Command mode】

PON port configuration mode and global mode

【Example】

！Display ONU MAC authentication

OptiWay(config)# show onu-mac-auth mode slot 2 pon 1

- slot : 2

- pon 2/1 mac-auth mode:  black-list

    - show white-list

This command is to display white list.

show white-list

【Command mode】

PONport configuration mode

【Example】

！Display white list

OptiWay(config-if-pon-3/1)#show white-list

WHITE LIST:

ONU      INDEX      MAC ADDRESS      ACTIVE      NAME

2/4/1      152      02:02:02:02:02:02    y

    - show black-list

This command is to display black list.

show black-list

【Command mode】

PON port configuration mode

【Example】

！Display black list

OptiWay(config-if-pon-5/4)#show black-list

BLACK LIST:

PORT         INDEX      MAC ADDRESS

pon-2/4      1          01:01:01:01:01:01

    - show loid-list

This command is to display logic identifier authentication entry .

show loid-list

【Command mode】

PONport configuration mode

【Example】

！Display logic identifier authentication entry

OptiWay(config-if-pon-2/4)#show loid-list

LOID LIST:

Index Loid  Password

1     bbbbb 12345

Total loid entries: 1 .

    - show hybrid-list

This command is to display hybrid authentication entry.

show hybrid-list

【Command mode】

PON port configuration mode

【Example】

！Display hybrid authentication entry

OptiWay(config-if-pon-2/4)# show hybrid-list

Hybrid LIST:

Index Loid/Mac Address Password

1     aaaabbbbcccc      1234

2     00:00:00:00:12:34

Total hybrid entries: 2 .

    - show onu-p2p

This command is to display ONU intersession configuration.

show onu-p2p

【Command mode】

PON configuration mode

【Example】

！Display ONU intersession configuration.

OptiWay(config-if-pon-3/1)#show onu-p2p

- onu-p2p : disable

    - show dba

This command is to display dba configuration.

show dba slot

【Command mode】

Global configuration mode

【Example】

！Display dba configuration.

OptiWay(config)#show dba 2

- DBA mode: hardware DBA with dynamic cycletime tuning

- DBA algorithm: NONWORKCONSERV

- DBA params:

- pon 1: cycletime = 125000 discovfreq = 64 discovtime = 14000

- pon 2: cycletime = 125000 discovfreq = 64 discovtime = 14000

- pon 3: cycletime = 125000 discovfreq = 64 discovtime = 14000

- pon 4: cycletime = 125000 discovfreq = 64 discovtime = 14000

    - show mac-address-table

This command is to display MAC address quality limitation based on ONU LLID

show mac-address-table onu-index { llid_index | all } max-mac-count

【Parameter 】

llid_index: ONU LLID index

【Command mode】

PONport configuration mode

【Example】

！Display MAC address quality limitation based on the first ONU LLID

OptiWay(config-if-pon-3/3)#show mac-address-table onu-index 1

max-mac-count

onu-index     llid            limit OLT      Max mac address number

01           0x61040000      enable            20

    - show classif

This command is to display OLT detect flow classification configuration

show classif [ ruleid ]

show classif [ slot slotid ]

【Parameter 】

ruleid:flow classification rule id index ,the range is 1－99

slotid:OLT business card slot id,2-5

【Command mode】

PONport configuration modeand All global configuration mode

【Example】

！Display interface 1 slotid 1 onu 3 flow classification rule in pon3

OptiWay(config-if-pon-3/1)#show classif 1

！Display pon 3 flow classification rule

OptiWay(config)#show classif slot 3

    - show enable-pon-vlan-isolation

This command is to display PON VLAN isolation information

show enable-pon-vlan-isolation

【Command mode】

All global configuration mode

【Example】

！Display PON VLAN isolation information

OptiWay(config)#show enable-pon-vlan-isolation

PON isolation by vlan is enable: 4010 4011 4012 4013.

# ONU Management Configuration Commands

  - ONU Management Configuration Commands

- onu-description

- show onu-description

- onu-binding

- show onu-status

- onu-reboot

- onu-bandwidth

- onu-encrypt

- onu-loopback

- onu-flow-control

- onu-shutdown

- onu-speed auto

- onu-vlan-mode

- onu-classification

- onu-mac-address-table

- onu-queue-scheduler

- onu-dtag

- onu-ip address static

- no onu-ip address

- show onu-bandwidth

- show onu-encrypt

- show onu-loopback oam

- show onu-interface

- show onu-sn

- show onu-firmware

- show onu-pon-chip

- show onu-capabilities

- show onu-vlan-mode

- show onu-mac-address-table

- show onu-queue-scheduler

- show onu-dtag

- show onu-ip address onu

    - onu-description

This command is to configure corresponding descriptor to distinguish each ONU.

onu-description onu-name

【Command mode】

ONU configuration mode

【Example】

！Configure onu3/4/1 description as greennet

OptiWay(onu-3/4/1)# onu-description greennet

    - show onu-description

This command is to display ONU configured descriptor.

show onu-description

【Command mode】

ONU configuration mode

【Example】

！Display onu3/4/1 descriptor

OptiWay(onu-3/4/1)#show onu-description

    - onu-binding

This command is to bind ONU type or MAC corresponding to related ONU index；Meanwhile unbind to binding ONU

onu-binding mac H:H:H:H:H:H type onu-type onu onu-id

onu-binding mac H:H:H:H:H:H onu onu-id

onu-binding type onu-type onu onu-id

no onu-binding onu onu-id

onu-binding mac H:H:H:H:H:H type onu-type

onu-binding mac H:H:H:H:H:H

onu-binding type onu-type

no onu-binding

no onu-binding mac

no onu-binding type

【Command mode】

Global configuration mode and ONU configuration mode

【Example】

！Bind onu3/4/1 type and MAC

OptiWay(config)# onu-binding mac 0:0:0:0:0:11 type 2040 onu 3/4/1

！Unbind onu3/4/1 binding

OptiWay(config)# no onu-binding

！Bind onu3/4/1 type

OptiWay(onu-3/4/1)#onu-binding type 2040

！Bind onu3/4/1 MAC

OptiWay(onu-3/4/1)#onu-binding mac 0:0:0:0:0:11

！Unbind onu3/4/1 binding type

OptiWay(onu-3/4/1)#no onu-binding type

！Unbind onu3/4/1 binding MAC

OptiWay(onu-3/4/1)#no onu-binding mac

    - show onu-status

This command is to receive ONUregistration,including MAC of ONU,RTT,on line time,ONU type, software version, current online status.

show onu-status

【Command mode】

ONU configuration mode

【Example】

！Configure onu3/4/1 description as greennet

OptiWay(onu-3/4/1)#show onu-status

ONU   Mac Address     RTT(TQ) RegisterTime Type   Software    State

3/4/1 00:0a:5a:12:46:59 54 00/021/02 00:40:08  2160 B01D001P005SP1 Up

    - onu-reboot

This command is to reboot on line ONU.

onu-reboot

【Command mode】

ONU configuration mode

【Example】

！Reboot ONU

OptiWay(onu-3/3/1)#onu-reboot

    - onu-bandwidth

This command is to configure the whole ONU uplink and downlink bandwidth,traffic shaping and strategy.

onu-bandwidth upstream fir fir-number cir cir-number pir pir-number

[ burst burst-number ] [ priority priority-number ] [delay delay-number ]

[ jitter jitter-number ]

onu-bandwidth downstream fir fir-number cir cir-number pir pir-number

[ burst burst-number ] [ priority priority-number ] [delay delay-number ]

[ jitter jitter-number ]

onu-bandwidth shaper { cir | pir }

onu-bandwidth police upstream { cir { dorp | mark } | pir { dorp | mark }

| mixed }

onu-bandwidth police downstream { cir { dorp | mark } | pir { dorp

| mark } | mixed }

no onu-bandwidth upstream

no onu-bandwidth downstream

no onu-bandwidth shaper

no onu-bandwidth police upstream

no onu-bandwidth police downstream

【Parameter】

fir-number: fixed bandwidth : 0-980000 Kbps

cir-number: permitted bandwidth : 0-1000000 Kbps

pir-number: maximum bandwidth: 512-1000000 Kbps

burst-number: burst size : 32-640 KB

priority-number: priority /weight: 1-32

delay-number: maximum delay : us

jitter-number: maximum jitter : us

【Command mode】

ONU configuration mode

【Example】

！Configure ONU uplink bandwidth

OptiWay(onu-3/3/1)#onu-bandwidth upstream fir 0 cir 600000 pir 980000

！Configure ONU traffic shaping

OptiWay(onu-3/3/1)#onu-bandwidth shaper cir

！Configure ONU uplink strategy

OptiWay(onu-3/3/1)#onu-bandwidth police upstream pir drop

！Recover ONU downlink bandwidth original configuration

OptiWay(onu-3/3/1)#no onu-bandwidth downstream

！Disable ONU traffic shaping

OptiWay(onu-3/3/1)#no onu-bandwidth shaper

！Disable ONU downlink strategy

OptiWay(onu-3/3/1)#no onu-bandwidth police downstream

    - onu-encrypt

This command is to enable/disable ONU encryption

onu-encrypt

no onu-encrypt

【Command mode】

ONU configuration mode

【Example】

！Enable ONU encryption

OptiWay(onu-3/3/1)#onu-encryp

！Disable ONU encryption

OptiWay(onu-3/3/1)#no onu-encryp

    - onu-loopback

This command is to enable/disable ONU loopback

onu-loopback oam

no onu-loopback oam

【Command mode】

ONU configuration mode

【Example】

！Enable ONU loopback

OptiWay(onu-3/3/1)#onu-loopback oam

！Disable ONU loopback

OptiWay(onu-3/3/1)#no onu-loopback oam

    - onu-flow-control

This command is to configure ONU port flow control in CTC standard

onu-flow-control

no onu-flow-control

【Command mode】

ONUport configuration mode

【Example】

！Enable ONU port flow control

OptiWay(onu-3/3/1-reth-0/1)#onu-flow-control

！Disable ONU port flow control

OptiWay(onu-3/3/1-reth-0/1)#onu-flow-control

    - onu-shutdown

This command is to configure ONU port enable in CTC standard

onu-shutdown

no onu-shutdown

【Command mode】

ONUport configuration mode

【Example】

！Disable ONU port enable

OptiWay(onu-3/3/1-reth-0/1)#onu-shutdown

！Enable ONU port enable

OptiWay(onu-3/3/1-reth-0/1)#no onu-shutdown

    - onu-speed auto

This command is to configure ONU port auto-negociation in CTC standard

onu-speed auto

no onu-speed auto

【Command mode】

ONUport configuration mode

【Example】

！Enable ONU port auto-negociation

OptiWay(onu-3/3/1-reth-0/1)#onu-speed auto

！Disable ONU port auto-negociation

OptiWay(onu-3/3/1-reth-0/1)#no onu-speed auto

    - onu-bandwidth

This command is to configure ONU uplink and downlink bandwidth in CTC standard

onu-bandwidth ingress cir cir_number cbs cbs_number ebs ebs_number

onu-bandwidth egress cir cir_number pir pir_number

【Parameter】

cir-number: export/inport rate is 64 - 1024000 kbps

cbs-number: ring algorithm deepth is  1523 - 1000000 Byte

ebs-number: additional burst size is 0 - 1522 Byte

pir-number: peak information rate is 64 - 1024000 kbps

【Command mode】

ONUport configuration mode

【Example】

！Configure port uplink bandwidth

OptiWay(onu-3/3/1-reth-0/1)# onu-bandwidth ingress cir 20000 cbs 20000 ebs

123

！Configure port downlink bandwidth

OptiWay(onu-3/3/1-reth-0/1)# onu-bandwidth egress cir 20000 pir 20000

    - onu-bandwidth multicast

This command is to configure private CTC ONU multicast rate

onu-bandwidth multicast rate-limit-value

【Parameter】

rate-limit-value: multicast rate value is :1 – 1048576（kbps）

0 means no limit to multicast

    - onu-bandwidth broadcast

This command is to configure private CTC ONU broadcast rate

onu-bandwidth broadcast { upstream | downstream } rate-limit-value

【Parameter】

rate-limit-value: broadcast rate value is :256-1000000（kbps）

    - onu-vlan-mode

This command is to configure ONU port vlan mode in CTC standard

onu-vlan-mode { transparent | tag vlan vlan-number | translation

{ vlan vlan_number old_vlan vlan_number1 new_vlan vlan_number2 } | { delete old_vlan vlan_number1 new_vlan vlan_number2 } }

【Parameter】

vlan-number: port default vlan ID

vlan_number1: shift list elements 1

vlan_number2: shift list elements 2

【Command mode】

ONU configuration mode,ONUport configuration mode

【Usage】

In ONU configuration mode, only configure two vlan modes, that is transparent mode and tag mode.

In ONU port mode, it configures three vlan modes : transparent mode, tag mode and translation mode.When configuring port translation mode,if need to add new shift entry based on original base, it enters command vlan-mode

translation vlan vlan_number old_vlan vlan_number1 new_vlan

vlan_number2, if add new entry vlan vlan_number is different from original value, the new entry will replace the original entry.

【Example】

！Configure port as translation mode

OptiWay(onu-3/3/1-reth-0/1)#onu-vlan-mode translation vlan 3 old_vlan 5

new_vlan 6

！Add a new shift items

OptiWay(onu-3/3/1-reth-0/1)#onu-vlan-mode translation vlan 3 old_vlan 52

new_vlan 66

！Configure all ports as tag mode

OptiWay(onu-3/3/1)#onu-vlan-mode tag vlan 34

    - onu-classification

This command is to configure ONU port flow classfication in CTC standard

onu-classification precedence precedence-num queMapped queMapped-num ethernet-priority ethernetPriority-num

{ select-filed select-value | select-filed select-value | select-filed

select-value }

Below command is to delete ONU port flow classfication in CTC standard

no onu-classification precedence precedence-num

【Parameter】

precedence-num: priority of flow classification rule

queMapped-num: rule mapping queue num

ethernetPriority-num: Ethernet priority

select-value: select item matching value

【Command mode】

ONU configuration mode/port configuration mode

【Example】

！Add a flow classfication:

OptiWay(onu-3/3/1-reth-0/1)#onu-classification precedence 1 que-mapped 1

ethernet-priority 2 destination-mac 11:22:33:44:55:66

！Delete a flow classfication:

OptiWay(onu-3/3/1-reth-0/1)# no onu-classification precedence 1

    - onu-multicast mode

This command is to configure ONU multicast control mode in CTC standard

onu-multicast mode { onu-igmp-snooping | onu-multicast-ctrl }

【Command mode】

ONU configuration mode

【Example】

！Configure ONU multicast control mode as controllable mode

OptiWay(onu-3/3/1)#onu-multicast mode onu-multicast-ctrl

    - onu-multicast tag

This command is to configure ONU port multicast tag strip

onu-multicast { tag | untag }

【Command mode】

ONUport configuration mode

【Example】

！configure ONU port multicast without VLAN TAG of stripping multicst business packets

OptiWay(onu-3/3/1-reth-0/1)#onu-multicast tag

    - onu-multicast fastleave

This command is to enable/disable fast-leave in CTC standard.

onu-multicast fastleave { enable | disable }

【Command mode】

ONU configuration mode

【Example】

！Configure ONU multicast and enable fast-leave:

OptiWay(onu-3/3/1)#onu-multicast fastleave enable

    - onu-igmp-snooping vlan

This command is to configure port multicast vlan in CTC standard

onu-igmp-snooping vlan vlan-list

This command is to delete ort multicast vlan in CTC standard

no onu-igmp-snooping vlan vlan-list

【Parameter】

vlan-list: configure up to 8 vlan multicast group

【Command mode】

ONUport configuration mode

【Example】

！Configure ONU port multicast vlan group

OptiWay(onu-3/3/1 -reth-0/1)#onu-igmp-snooping vlan 1,2,3-5

！Delete ONU port multicast vlan group

OptiWay(onu-3/3/1 -reth-0/1)# no onu-igmp-snooping vlan 1,2

    - onu-igmp-snooping group-number

This command is to configure the maximum view on line multicast group in igmp-snooping mode

onu-igmp-snooping group-number group-number

【Command mode】

ONUport configuration mode

【Example】

！Configure ONU view on line multicast group in igmp-snooping mode

OptiWay(onu-3/3/1 -reth-0/1)# onu-igmp-snooping group-number 6

    - onu-multicast-ctrl

This command is to configure the privileged control table in the control multicast mode.

onu-multicast-ctrl { permit | preview } { H:H:H:H:H:H portid vlanid }

【Parameter】

H:H:H:H:H:H :multicast mac address

portid:onu port id

vlanid: multicast vlan num

【Command mode】

ONU configuration mode

【Example】

！Configure ONU multicast vlan group

OptiWay(onu-3/3/1)# onu-multicast-ctrl permit 01:00:5e:01:01:01 1 5

    - onu-fec mode

This command is to configure FEC mode in CTC standard

onu-fec mode { enable | disable }

【Command mode】

ONU configuration mode

【Example】

！Enable fec

OptiWay(onu-3/3/1)# onu-fec mode enable

    - no onu-classification

This command is to delete ONU port flow classification in CTC standard

no onu-classification precedence precedence-num

【Parameter】

precedence-num: priority of flow classification rule

【Command mode】

ONU configuration mode/port configuration mode

【Example】

！Delete a flow classification:

OptiWay(onu-3/3/1-reth-0/1)# no onu-classification precedence 1

    - no onu-igmp-snooping vlan

This command is to delete multicast vlan in CTC standard

no onu-igmp-snooping vlan vlan-list

【Parameter】

vlan-list: configure maximum 8 vlan multicast group

【Command mode】

ONUport configuration mode

【Example】

！Delete ONU port multicasdt vlan group

OptiWay(onu-3/3/1 -reth-0/1)# no onu-igmp-snooping vlan 1,2

    - no onu-multicast-ctrl

This command is to delete port multicast vlan group in CTC standard

no onu-multicast-ctrl { H:H:H:H:H:H portid vlanid }

【Parameter】

H:H:H:H:H:H : multicast mac address

portid:onu port id

vlanid: multicast vlan id

【Command mode】

ONU configuration mode

【Example】

！Delete ONU port multicast vlan group

OptiWay(onu-3/3/1)# no onu-multicast-ctrl 01:00:5e:01:01:01 1 5

    - no onu-bandwidth multicast

This command is to recover private CTC ONU multicast default rate :that is no limit to multicast

no onu-bandwidth multicast

    - no onu-bandwidth broadcast

This command is to recover privateCTC ONU broadcast default rate :that is no limit to broadcast

no onu-bandwidth broadcast

    - onu-mac-address-table max-mac-count

This command is to limit ONU port MAC address learning quantity

onu-mac-address-table max-mac-count number

no onu-mac-address-table max-mac-count

【Parameter】

number:MAC address limitation quantity,the range is 1－1000

【Command mode】

ONU configuration mode, it appoints to all ports

ONUport configuration mode,it appoints to single port

【Example】

！Configure ONU 3/3/1 all ports MAC address limitation quantity is 20

OptiWay(onu-3/3/1)#onu-mac-address-table max-mac-count 20

！Configure ONU 3/3/1port3 MAC address limitation quantity is 40

OptiWay(onu-3/3/1)#interface ethernet 3

OptiWay(onu-3/3/1-reth-3)#onu-mac-address-table max-mac-count 40

！Delete ONU 3/3/1 all ports MAC address limitation quantity is 20

OptiWay(onu-3/3/1)#no onu-mac-address-table max-mac-count

    - onu-mac-address-table age-time

This command is to configure ONU MAC address aging time

onu-mac-address-table age-time { number | disable }

no onu-mac-address-table age-time

【Parameter】

number: means MAC address aging time,the range is 1-1048575s

disable: means MAC address table doesn’t be aging

【Default】

The default value of MAC address aging time is 300s

【Command mode】

ONU configuration mode

【Example】

！Configure ONU MAC address aging time is 10s

OptiWay(onu-3/3/1)#onu-mac-address-table age-time 10

！Revert ONU MAC address aging time is the default 300s

OptiWay(onu-3/3/1)#no onu-mac-address-table age-time

！Disable ONU MAC address aging time

OptiWay(onu-3/3/1)#onu-mac-address-table age-time disable

    - onu-queue-scheduler

This command is to configure ONU queue scheduler.

onu-queue-scheduler { strict-priority | wrr queue1-weight queue2-weight queue3-weight queue4-weight }

no queue-scheduler

【Parameter】

strict-priority: means queue executes strict scheduler

queue1-weight queue2-weight queue3-weight queue4-weight: means queue executes WRR .queue1-weight:num 1 queue weight；queue2-weight: num 2queue weight；queue3-weight: num 3 queue weight；queue4-weight: num 4 queue weight

【Command mode】

ONU configuration mode

【Example】

！Configure queue scheduler mode as WRR ,the weight of 8 queues is 15,

14,13,12

OptiWay(onu-3/3/1)#onu-queue-scheduler wrr 15 14 13 12

    - onu-queue-scheduler cos-map

This command is to configure the mapping of 8 queues in hardward system and 8 priorites in IEEE 802.1p .

onu-queue-scheduler cos-map { queue-number packed-priority }

【Parameter】

queue-number: hardware priority queue, the range is 0～4

packed-priority:IEEE 802.1p defines priority,the range is 0～7

【Default】

By default, the mapping is as below :

802.1p:		0	1	2	3	4	5	6	7

Hardware priority :0	0	1	1	2	2	3	3

【Command mode】

ONU configuration mode

【Usage】

By default, EL5600-01-04 has 4 hardware priority queues , from 0～4,4 is the highest priority.

EL5600-01-04 sends buffer packets of higher priority in the hardware priority queues.

【Example】

！Configure hardware priority queues 1 mapping to IEEE 802.1p protocol priority 6

OptiWay(onu-3/3/1)#onu-queue-scheduler cos-map 1 6

    - onu-queue-scheduler cos-remap

This command is to configure the mapping of 8 priority in IEEE 802.1p.With command no means to disable 802.1p re-mapping.

onu-queue-scheduler cos-remap [ old-cos new-cos ]

【Parameter】

old-cos:IEEE 802.1p defined original priority ,the range is 0～7

new-cos:IEEE 802.1p defined new priority,the range is 0～7

【Default】

By default, the mapping disables, when open, COS assigns to different priority.

【Command mode】

ONU configuration mode

【Example】

！Enable 802.1p re-mapping, configure value 2 IEEE 802.1p protocol priority re-mapping to 5

OptiWay(onu-3/3/1)#onu-queue-scheduler cos-remap

OptiWay(onu-3/3/1)#onu-queue-scheduler cos-remap 2 5

    - onu-mac-address-table blackhole

This command is to configure ONU MAC blackhole

onu-mac-address-table blackhole H:H:H:H:H:H [ vlan vlan_id ]

no onu-mac-address-table blackhole { all | H:H:H:H:H:H [ vlan vlan_id ] }

【Parameter】

H:H:H:H:H:H:MAC address

vlan_id:VLAN id,the range is 1－4094

【Command mode】

ONU configuration mode

【Example】

！Configure ONU 3/3/1 blackhole items

OptiWay(onu-3/3/1)#onu-mac-address-table blackhole 1:1:1:1:1:1 vlan 1

！Delete ONU 3/3/1 blackhole items

OptiWay(onu-3/3/1)#no onu-mac-address-table blackhole 1:1:1:1:1:1 vlan 1

！Delete ONU 3/3/1 all blackhole items

OptiWay(onu-3/3/1)#no onu-mac-address-table blackhole all

    - onu-dtag

This command is to configure ONU QINQ

onu-dtag [ flexible-qinq ]

no onu-dtag

onu-dtag outer-tpid tpid_id

no onu-dtag outer-tpid

onu-dtag mode { customer | uplink }

no onu-dtag mode

onu-dtag insert start_vlan end_vlan vlan

no onu-dtag insert [ all ]

onu-dtag psgg-through start_vlan end_vlan

no onu-dtag psgg-through [ all ]

【Parameter】

tpid_id:TPID value,the range is 0x5DD－0xFFFF

start_vlan: start VLAN value,the range is 1－4094

end_vlan: end VLAN value,the range is 1－4094

vlan:VLAN value,the range is 1－4094

【Command mode】

In ONU configuration mode,it is for QINQ mode and outside TPID

In ONU port configuration mode, it is for QINQ entity items

【Example】

！Enable static QINQ

OptiWay(onu-3/3/1)#onu-dtag

！Enable flexible QINQ

OptiWay(onu-3/3/1)#onu-dtag flexible-qinq

！Disable QINQ

OptiWay(onu-3/3/1)#no onu-dtag

！Configure outside TPID

OptiWay(onu-3/3/1)#onu-dtag outer-tpid 999

！Revert to default TPID

OptiWay(onu-3/3/1)#no onu-dtag outer-tpid

！Configure port mode as CUSTOMER mode

OptiWay(onu-3/3/1-reth-0/1)#onu-dtag mode customer

！Revert port mode is the default UPLINK mode

OptiWay(onu-3/3/1-reth-0/1)#no onu-dtag mode

！Configure port INSERT items

OptiWay(onu-3/3/1-reth-0/1)#onu-dtag insert 3 5 6

！Delete port INSERT item

OptiWay(onu-3/3/1-reth-0/1)#no onu-dtag insert 3 5 6

！Configure port PASSTHROUGH items

OptiWay(onu-3/3/1-reth-0/1)#onu-dtag pass-through 7 9

！Delete port all PASSTHROUGH items

OptiWay(onu-3/3/1-reth-0/1)#no onu-dtag pass-through all

    - onu-ip address static

This command is to configure ONU IP

onu-ip address static ip-address ip-address-mask ip-address-gateway

【Command mode】

ONU configuration mode

【Example】

！Configure onu 3/4/1 IP

OptiWay(onu-3/4/1)#onu-ip address static 1.1.1.22 255.255.255.0 1.1.1.1

    - no onu-ip address

This command is to clear ONU IP

no onu-ip address

【Command mode】

ONU configuration mode

【Example】

！Clear onu 3/4/1的IP

OptiWay(onu-3/4/1)#no onu-ip address

    - onu-vlan

- This command is to enter into VLAN configuration mode or create VLAN then enter into VLAN configuration mode

onu-vlan vlan-id

【Command mode】

ONU configuration mode or ONU port mode

【Example】

！Create ONU 3/4/1 VLAN 200

OptiWay(onu-3/4/1)#onu-vlan 200

    - no onu-vlan

- This command is to delete all VLAN except existed VLAN 1 or selected VLAN

no onu-vlan { vlan-id | all }

【Command mode】

ONU configuration mode

【Example】

！Delete ONU 3/4/1 VLAN 200

OptiWay(onu-3/4/1)#no onu-vlan 200

    - onu-swithport

- This command is to add ONU port in current VLAN.

onu-OLTport { interface-list | all }

【Command mode】

ONU VLAN mode

【Example】

！Add ONU3/3/1 port 1,2,3 to current VLAN

OptiWay(onu-3/3/1-rvlan)#onu-OLTport ethernet 2/1 to ethernet 2/3

    - no onu-swithport

- This command is to add ONU port in current VLAN

no onu-OLTport { interface-list | all }

【Command mode】

ONU VLAN mode

【Example】

！Delete ONU3/3/1 port 1 in the current VLAN

OptiWay(onu-3/3/1-rvlan)#no onu-OLTport ethernet 2/1

    - onu-swithport access vlan

- This command is to add current ports to selected VLAN, port default VLAN ID is selected VLAN

onu-OLTport access vlan vlan-id

【Command mode】

ONU port mode

【Example】

！Add ONU3/3/3 access port 3 to VLAN 200

OptiWay(onu-3/3/3-reth-0/3)#onu-OLTport access vlan 200

    - no onu-swithport access vlan

- This command is to delete current access port in selected VLAN

no onu-OLTport access vlan vlan-id

【Command mode】

ONU port mode

【Example】

！Delete ONU3/3/3 access port 3 in VLAN 200

OptiWay(onu-3/3/3-reth-0/3)#no onu-OLTport access vlan 200

    - onu-tag-mode

This command is to configure ONU port in tag mode .

onu-tag-mode [ tag | untag | unmodify ]

【Command mode】

ONU port mode

【Example】

！Configure ONU3/3/3 access port 3 TAG mode as tag

OptiWay(onu-3/3/3-reth-0/3)#onu-tag-mode tag

    - onu-com-session

This command is to configure electricity power ONU console session parameter.

onu-com-session comid { tcpclient | udp } serverip portid [ baudrate [ parity [ databits [ stopbits] ] ] ]

onu-com-session comid tcpserver portid [ baudrate [ parity [ databits [ stopbits ] ] ] ]

【Parameter】

comid: console id ,the range is 1-4

portid: network id,the range is 1-65535

baudrate: baud rate,the range is 0-10,0 means baud rate is 300,1 means baud rate is 600,2 means baud rate is 1200,3 means baud rate is 2400,4 means baud rate is 4800,5 means baud rate is 9600,6 means baud rate is 14400,7 means baud rate is 19200,8 means baud rate is 38400,9 means baud rate is 57600,10 means baud rate is 115200, it will be the default value 115200 if nothing inputs

parity-type: parity type, the range is 0-4,0 means None parity,1 means even parity,2 means odd parity ,3 means symbol parity ,4 means space parity, it will be the default value None parity if nothing inputs

databits: data bits ,the range is 0-3,0 means data bit as 5 bits,1 means data bit 6 bits, 2 means data bit 7 bits,3 means data bit 8 bits, it will be the default value 8 bits if nothing inputs

stopbits:stop bits,the range is 0-2,0 means stop bit as 1bit,1 means stop bit as 1.5 bits,2 means stop bit as 2 bits, it will be the default 1 bit if nothing inputs

【Command mode】

ONU configuration mode

【Example】

！Configue ONU console 1 session is UDP, session port is 9800,session ip is 192.168.1.1, baud rate is 1200,parity type is even parity, databits is 6 bits, stopbits is 1.5 bits

OptiWay(onu-3/4/1)#onu-com-session 1 udp 192.168.1.1 9800 2 1 1 1

    - no onu-com-session

his command is to disable electricity power ONU console session.

no onu-com-session comid

【Command mode】

ONU configuration mode

【Example】

！Disable console 1 session

OptiWay(onu-3/4/1)#no onu-com-session 1

    - clear onu-com-statistic

This command is to clear console traffic statistic

clear onu-com-statistic [ comid ]

【Command mode】

ONU configuration mode

【Example】

！Clear all console traffic statistic

OptiWay(onu-3/4/1)# clear onu-com-statistic

    - onu-event-alarm loopback

This command is to enable/disable onu loopback alarm

onu-event-alarm loopback enable/disable

【Command mode】

ONU configuration mode,All global configuration mode

【Example】

- ！Enable onu loopback alarm

OptiWay(onu-3/3/1)#onu-event-alarm loopback enable

- ！Disable onu loopback alarm

OptiWay(onu-3/3/1)#onu-event-alarm loopback disable

    - onu-ctc-upgrade

This command is to upgrad ONU version based on ctc standard

onu-ctc-upgrade

【Command mode】

ONU configuration mode

【Example】

！Upgrade onu

OptiWay(onu-3/4/1)#onu-ctc-upgrade

    - onu-ctc-upgrade-commit

This command is to ensure onu upgrade had finished

onu-ctc-upgrade-commit

【Command mode】

ONU configuration mode

【Example】

！Ensure onu upgrade had finished

OptiWay(onu-3/4/1)#onu-ctc-upgrade-commit

    - show onu-port-info

This command is to display current ONU port PVID and port TAG mode .

show onu-port-info

【Command mode】

Any configuration mode

【Example】

！Display current ONU3/3/3 port 3 information

OptiWay(onu-3/3/3-reth-0/3)#show onu-port-info

PVID 200

Port mode: Untagged

    - show onu-status

This command is to display receiving ONUregistration,including MAC of ONU,RTT,on line time,ONU type, software version, current online status.

show onu-status [ onu-id | onu-mac ]

【Command mode】

Any configuration mode

【Example】

！Display current online ONU status

OptiWay(onu-3/4/1)#show onu-status

ONU   Mac Address  RTT(TQ) RegisterTime Type    Software   State

3/4/1 00:0a:5a:12:46:59 54 00/021/02 00:40:08 2160 B01D001P005SP1 UP

    - show statistics onu

This command is to display ONU port traffic statistics

show statistics onu olt/pon/onu

【Command mode】

Any configuration mode

【Example】

！Display ONU port 1 traffic statistics

OptiWay(config)#show statistics

OptiWay(onu-3/3/1)#show statistics 3/4/1

SLOT/PON  TOTAL(BYTES)  UNICASTS(packets)

MULTICASTS(packets)    BROADCASTS(packets)

-------------------------------------------------------------------------------------------------

3/4/1 port:pon

input     6389705        511         92443                6176

output    80054           73              0                 498

3/4/1 port:uni

input      549817         73           6907                 498

output    2354053         10          30565                6176

    - show onu-bandwidth

This command is to display the whole ONU uplink and downlink bandwidth,traffic shaping and strategy.

show onu-bandwidth

show onu-bandwidth shaper

show onu-bandwidth police

【Command mode】

ONU configuration mode

【Example】

！Display ONU uplink bandwidth

OptiWay(onu-3/3/1)#show onu-bandwidth

upstream:   fir=0 cir=1000 pir=100000 burst=100 priority=1 delay=10 jitter=10

downstream: fir=0 cir=1000 pir=100000 burst=100 priority=1 delay=10 jitter=10

ONU port 1 bandwidth ingress is Disable

！Display ONU traffic shaping

OptiWay(onu-3/3/1)#show onu-bandwidth shaper

shaper action: disable

！Display ONU traffic strategy

OptiWay(onu-3/3/1)#show onu-bandwidth police

upstream police: disable

downstream police: disable

    - show onu-encrypt

This command is to display ONU encryption

show onu-encrypt

【Command mode】

ONU configuration mode

【Example】

！Display ONU encryption

OptiWay(onu-3/3/1)#show onu-encrypt

encryp: enable

    - show onu-loopback oam

This command is to display ONU loopback

show onu-loopback oam

【Command mode】

ONU configuration mode

【Example】

！Display ONU loopback

OptiWay(onu-3/3/1)#show onu-loopback oam

Loopback oam is disable

    - show onu-interface

This command is to display ONU port information

show onu-interface [ ethernet ethernet_number ]

【Parameter 】

ethernet-number:detailed port num

【Command mode】

ONU configuration mode

【Example】

！Display ONU port1

OptiWay(onu-3/3/1)#show onu-interface ethernet 2/1

onu : 3/3/1

ONU port 1 is Disable, port link is LINK UP

ONU auto negotiate ability :

ability value[1] : 0x00000028

ability value[2] : 0x00000192

ability value[3] : 0x00000142

speed auto is Enable, Flow control is Disable

bandwidth ingress is Disable

bandwidth egress is Disable

    - show onu-sn

This command is to display ONU serial number in CTC standard.

show onu-sn

【Command mode】

ONU configuration mode

【Example】

！Display ONU serial number

OptiWay(onu-3/3/1)#show onu-sn

Vender ID: IMST

MODEL: 8015

ONUID: 0x00 0x0a 0x5a 0x11 0x49 0x5c

HW: 0x35 0x30 0x31 0x31 0x00 0x00 0x00 0x00

SW: 0x32 0x30 0x33 0x00 0x00 0x00 0x00 0x00 0x00 0x00 0x00 0x00 0x00 0x00 0x00 0x00

    - show onu-firmware

This command is to display ONU firmware information in CTC standard.

show onu-firmware

【Command mode】

ONU configuration mode

【Example】

！Display ONU firmware information

OptiWay(onu-3/3/1)#show onu-firmware

onu firmware :

0x48 0xfd 0x97 0x3f 0x00 0x02 0x00 0x00 0x00 0x03 0x00 0x00

    - show onu-pon-chip

This command is to display ONU PON chip information in CTC standard.

show onu-pon-chip

【Command mode】

ONU configuration mode

【Example】

！Display ONU PON chip information

OptiWay(onu-3/3/1)#show onu-pon-chip

onu chip id :  vendor id  IM

onu chip id :  model Id   0x5 0x11

onu chip id :  date       06/17/39

    - show onu-capabilities

This command is to display ONU support in CTC standard

show onu-capabilities

【Command mode】

ONU configuration mode

【Example】

！Display ONU support

OptiWay(onu-3/3/1)#show onu-capabilities

onu 3/3/1 :

onu capability:  serviceSupported			3

onu capability:  numGEPorts              1

onu capability:  geBitmap					0x10000

onu capability:  numFEPorts				16

onu capability:  feBitmap					0xffff

onu capability:  numPOTSPorts			0

onu capability:  numE1Ports				0

onu capability:  numUSQueues			4

onu capability:  maxQueueUSPort 		4

onu capability:  numDSQueues 			4

onu capability:  maxQueueDSPort 		4

onu capability:  BatteryBackup				0

    - show onu-bandwidth

This command is to display ONU uplink and downlink bandwidth in CTC standard.

show onu-bandwidth ingress [ interface ethernet ethernet_number ]

show onu-bandwidth egress [ interface ethernet ethernet_number ]

【Parameter 】

ethernet-number:detailed port num

【Command mode】

ONU configuration mode

【Example】

！Display ONU port uplink bandwidth configuration

OptiWay(onu-3/3/1)#show onu-bandwidth ingress interface ethernet 2/1

onu : 3/3/1

ONU port 1 bandwidth ingress is Enable

cir(ingress rate of port)  20000

cbs(depth of token bucket)  20000

ebs(the extra burst size)  123

！Display all ports downlink bandwidth configuration

OptiWay(onu-3/3/1)#show onu-bandwidth egress

onu : 3/3/1

ONU port 1 bandwidth egress is Disable

ONU port 2 bandwidth egress is Disable

ONU port 3 bandwidth egress is Disable

ONU port 4 bandwidth egress is Disable

ONU port 5 bandwidth egress is Disable

ONU port 6 bandwidth egress is Disable

ONU port 7 bandwidth egress is Disable

ONU port 8 bandwidth egress is Disable

    - show vlan-mode

This command is to display ONU vlan mode in CTC standard

show vlan-mode [ interface ethernet ethernet_num ]

【Parameter 】

ethernet-number: detailed port num

【Command mode】

ONU configuration mode

【Example】

！Display port 1 vlan mode

OptiWay(onu-3/3/1)#show vlan-mode interface ethernet 2/1

onu 3/3/1 :

port  ID :  1  ctc vlan mode : transparent

！Display all ports vlan mode

OptiWay(onu-3/3/1)#show vlan-mode

onu 3/3/1 :

port  ID :  1  ctc vlan mode : transparent

port  ID :  2  ctc vlan mode : transparent

port  ID :  3  ctc vlan mode : transparent

port  ID :  4  ctc vlan mode : transparent

port  ID :  5  ctc vlan mode : transparent

port  ID :  6  ctc vlan mode : transparent

port  ID :  7  ctc vlan mode : transparent

port  ID :  8  ctc vlan mode : transparent

    - show onu-classification

This command is to display ONU port flow classification configuration

show onu-classification

【Command mode】

ONU configuration mode/port configuration mode

【Example】

！Display flow classification configuration

OptiWay(onu-3/3/1)#show onu-classification

Rules 1 Precedence 1 QueMapped 2 ethernet-priority 5

Destination-port 55

Rules 2 Precedence 3 QueMapped 2 ethernet-priority 5

Destination-port 44

Rules 3 Precedence 4 QueMapped 2 ethernet-priority 5

Destination-port 88

Rules 4 Precedence 5 QueMapped 2 ethernet-priority 5

Destination-port 66

    - show onu-multicast mode

This command is to display multicast mode

show onu-multicast mode

【Command mode】

ONU configuration mode

【Example】

！Display ONU multicast control mode as controllable mode

OptiWay(onu-3/3/1)#show onu-multicast mode

    - show onu-multicast tag

This command is to display multicast mode

show onu-multicast tag

【Command mode】

ONUport configuration mode

【Example】

！Display ONU port multicast VLAN TAG strip or not

OptiWay(onu-3/3/1-reth-0/1)# show onu-multicast tag

interface 1:tag

    - show onu-multicast fast-leave

This command is to display port fast-leave status and capacity

show onu-multicast fast-leave

【Command mode】

ONU configuration mode

【Example】

！Display ONU multicast fast-leave function

OptiWay(onu-3/3/1)# show onu-multicast fast-leave

OptiWay(onu-3/3/1 -reth-0/1)# show onu-multicast fastleave

non-fast-leave(igmp-snooping) ability:support

fast-leave(igmp-snooping) ability:support

non-fast-leave(multicast-control) ability:support

fast-leave(multicast-control) ability:support

fase-leave state disable

    - show onu-igmp-snooping vlan

This command is to display multicast vlan group and multicast group quantit y on line at the same time.

show onu-igmp-snooping vlan

【Command mode】

ONU configuration mode

【Example】

！Display igmp-snooping multicast vlan group in the multicast mode

OptiWay(onu-3/3/1 -reth-0/1)#show onu-igmp-snooping vlan

interface 1

multicast-vlan:no vlan

group-number:255

    - show onu-multicast-ctrl local-ctrl

This command is to display privilege control table in the  local controllable mode

show onu-multicast-ctrl local-ctrl

【Command mode】

ONU configuration mode

【Example】

！Display local controllable multicast configuration

OptiWay(onu-3/3/1)# show onu-multicast-ctrl local-ctrl

Port     Multicast MAC      Multicast Vlan    Type             online

5      01:00:5e:01:01:01            5          permit             N

    - show onu-multicast-ctrl

This command is to display privilege control table in the remote controllable mode

show onu-multicast-ctrl

【Command mode】

ONU configuration mode

【Example】

！Display remote controllable multicast configuration

OptiWay(onu-3/3/1)# show onu-multicast-ctrl

the channel number at one time : 5

the live time : 300

preview time once : 180

preview interval : 300

preview times : 5

Port     Multicast MAC        Multicast Vlan

5      01:00:5e:01:01:01             5

    - show onu-fec

This command is to display ONU forward error correction

show onu-fec

【Command mode】

ONU configuration mode

【Example】

！Display fec status and capacity

OptiWay(onu-3/3/1)#show onu-fec

onu fec ability: supported   mode:disable

    - show onu-mac-address-table max-mac-count

This command is to display ONU port MAC address limitation quantity

show onu-mac-address-table max-mac-count

【Command mode】

ONU configuration mode, display all port

ONUport configuration mode, display single port

【Example】

！Display ONU all ports MAC address limitation quantity

OptiWay(onu-3/3/1)#show onu-mac-address-table max-mac-count

Port      limit OLT      Max mac address number

1         disable           0

2         disable           0

3         disable           0

4         disable           0

5         disable           0

6         disable           0

7         disable           0

.

.

.

！Display ONU port 3 MAC address limitation quantity

OptiWay(onu-3/3/1)#interface ethernet 3

OptiWay(onu-3/3/1-reth-0/3)#show onu-mac-address-table max-mac-count

Port      limit OLT      Max mac address number

3         disable           0

    - show onu-mac-address-table age-time

This command is to display ONU MAC address aging time

show onu-mac-address-table age-time

【Command mode】

ONU configuration mode

【Example】

！Display ONU MAC address aging time

OptiWay(onu-3/3/1)#show onu-mac-address-table age-time

mac address table agingtime is 300 second!

    - show onu-queue-scheduler

This command is to display ONU queue scheduler mode and mapping

show onu-queue-scheduler

show onu-queue-scheduler cos-map

show onu-queue-scheduler cos-remap

【Command mode】

ONU configuration mode

【Example】

！Display ONU queue scheduler mode

OptiWay(onu-3/3/1)# show onu-queue-scheduler

Queue scheduler mode: first come first serve

    - show onu-mac-address-table blackhole

This command is to display ONU MAC blackhole

show onu-mac-address-table blackhole

【Command mode】

ONU configuration mode

【Example】

！Display ONU 3/3/1 blackhole items

OptiWay(onu-3/3/1)#show onu-mac-address-table blackhole

Show MAC blackhole table information

MAC Address            VLAN ID

01:01:01:01:01:01        1

Total entries: 1 .

    - show onu-bandwidth multicast

This command is to display ONU multicast rate

show onu-bandwidth multicast

【Command mode】

ONU configuration mode

【Example】

！Display ONU 3/3/1 multicast rate

OptiWay(onu-3/3/1)#show onu-bandwidth multicast

ONU 4/2/3 Multicast:  Downstream = 1000000(kbps)   Upstream = 1000000(kbps)

    - show onu-bandwidth broadcast

This command is to display ONU broadcast rate

show onu-bandwidth broadcast

【Command mode】

ONU configuration mode

【Example】

！Display ONU 3/3/1 broadcast rate

OptiWay(onu-3/3/1)# show onu-bandwidth broadcast

ONU 4/2/3 Broadcast:  Downstream = 1000000(kbps)   Upstream = 1000000(kbps)

    - show onu-dtag

This command is to display ONU QINQ information

show onu-dtag

show onu-dtag insert

show onu-dtag pass-througn

【Command mode】

ONU configuration mode

ONUport configuration mode

【Example】

！Display QINQ mode

OptiWay(onu-3/3/1)#show onu-dtag

Current double tag style is : flexible-qinq

Current outer-tpid is      : 0x999

Ethernet 01 is a customer port

Ethernet 02 is a uplink port

Ethernet 03 is a uplink port

Ethernet 04 is a uplink port

.

.

.

Ethernet 16 is a uplink port

！Display port INSERT items

OptiWay(onu-3/3/1-reth-0/1)#show onu-dtag insert

port      inner start vlan      inner end vlan      outer vlan

01        3                     5                   6

Total entries: 1 .

！Display port PASSTHROUGH items

OptiWay(onu-3/3/1-reth-0/1)#show onu-dtag pass-through

port      pass through start vlan      pass through end vlan

01        7                            9

Total entries: 1 .

    - show onu-mac-address-table blackhole

This command is to display ONU MAC blackhole

show onu-mac-address-table blackhole

【Command mode】

ONU configuration mode

【Example】

！Display port INSERT items

OptiWay(onu-3/3/1-reth-0/1)#show onu-dtag insert

    - show onu-ip address onu

This command is to display ONU IP

show onu-ip address onu { onu-id | mac-address }

【Command mode】

Any configuration mode

【Example】

！Display onu 3/4/1 IP configuration

OptiWay(config)#show onu-ip address onu 3/4/1

ONU   Mac Address       Ipaddress Subnet Mask   Gateway Status

3/4/1   11:22:33:44:55:66   1.1.1.22  255.255.255.0  1.1.1.1  static

    - show onu-com-session

This command is to display electricity power ONU console session configuration parameter

show onu-com-session [ comid ]

【Command mode】

ONU configuration mode

【Example】

！Display onu 3/3/1 console session configuration

OptiWay(onu-3/3/1)#show onu-com-session

com1

com session type: udp

com session port: 9800

com session serverip:192.168.1.1

com settings baudrate:1200

com settings parity: even

com settings databits:6 bit

com settings stopbits:2 bit

com2

com session:DISABLE

com3

com session:DISABLE

com4

com session:DISABLE

    - show onu-com-statistic

This command is to display console traffic statistics

show onu-com-statistic [ comid ]

【Command mode】

ONU configuration mode

【Example】

！Display onu 3/3/1 console traffic statistics

OptiWay(onu-3/3/1)#show onu-com-statistic

com1

com bytesrecv: 0

com bytessend:0

    - show onu-event-alarm loopback

This command is to display onu loopback alarm status

show onu-event-alarm loopback

【Command mode】

ONU configuration mode,All global configuration mode

【Example】

OptiWay(onu-3/3/1)#show onu-event-alarm loopback

# PSG Management Configuration Command

  - PSG Management Configuration Command

PSG(Protection OLTing Group) configuration command includes:

- mpcp-delay-time

- admin-enable-pon

- psg creat

- psg delete

- psg OLT

- admin-enable-psg

- show mpcp-delay-time

- show admin-enable-pon

- show psg

- show admin-enable-psg

    - mpcp-delay-time

- This command is to configure/delete MPCP delay time of ONU registration

- mpcp-delay-time slot slot_id pon pon_id time delay_time

- no mpcp-delay-time slot slot_id pon pon_id

【Parameter】

slot_id:slot id

- pon_id:pon port id

- delay_time:delay time

【Command mode】

All global configuration mode

【Example】

- ！Configure PON 3 delay time in slot3

- OptiWay(config)# mpcp-delay-time slot 3 pon 3 time 10

- ！Delete PON 3 delay time in slot3

- OptiWay(config)# no mpcp-delay-time slot 3 pon 3

    - admin-enable-pon

This command is to enable/disable PON status

admin-enable-pon slot slot_id pon pon_id

no admin-enable-pon slot slot_id pon pon_id

【Parameter】

slot_id:slot id

- pon_id:pon port id

【Command mode】

All global configuration mode

【Example】

- ！Enable PON 3 status in slot3

- OptiWay(config)#admin-enable-pon slot 3 pon 3

- ！Disable PON 3 status in slot3

OptiWay(config)#no admin-enable-pon slot 3 pon 3

    - psg creat

This command is to creat PSG group

psg creat slot slot_id psg-id psg_id active-pon pon_index standby-pon pon_index

【Parameter】

psg_id:psg id

slot_id:slot id

- pon_index:pon id

【Command mode】

All global configuration mode

【Usage】

Creat PSG, system will assign PSG id automatically in the current PSG.

Before creating PSG, disable PON status.

【Example】

- ！Creat PSG of master PON 1 backup PON2

- OptiWay(config)#psg creat slot 3 active-pon 1 standby-pon 2

    - psg delete

This command is to delete PSG group

psg delete slot slot_id psg-id psg_id

【Parameter】

slot_id:slot id

- psg_id:psg id

【Command mode】

All global configuration mode

【Example】

- ！Delete PSG group

OptiWay(config)#psg delete slot 3 psg-id 1

    - psg OLT

This command is to shift master/backup PON in PSG manually

psg OLT slot slot_id psg-id psg_id

【Parameter】

slot_id:slot id,the range is 2--5

- psg_id:psg id,the range is 1--2

【Command mode】

All global configuration mode

【Example】

- ！Shift master/backup PON in PSG 1

OptiWay(config)#psg OLT slot 3 psg-id 1

    - admin-enable-psg

This command is to enable/disable PSG group

admin-enable-psg slot slot_id psg-id psg_id

no admin-enable-psg slot slot_id psg-id psg_id

【Parameter】

slot_id:slot id

- psg_id:psg id

【Command mode】

All global configuration mode

【Example】

- ！Enable PSG group 1

OptiWay(config)#admin-enable-psg slot 3 psg-id 1

！Disable PSG group 1

OptiWay(config)#no admin-enable-psg slot 3 psg-id 1

    - show mpcp-delay-time

This command is to display MPCP echo delay time in ONU registration

show mpcp-delay-time slot slot_id pon pon_id

【Parameter】

slot_id:slot id

- pon_id:pon id

【Command mode】

All global configuration mode

【Example】

！Display PON 3 echo delay time in slot3

OptiWay(config)#show mpcp-delay-time slot 3 pon 3

Port          limit delay      delay time

pon 3/3:      disable          10

    - show admin-enable-pon

This command is to display PON

show admin-enable-pon slot slot_id

【Parameter】

slot_id:slot id

【Command mode】

All global configuration mode

【Example】

！display PON in slot 3

OptiWay(config)#show admin-enable-pon slot 3

PON port      administrative status

pon 3/1:       admin-up

pon 3/2:       admin-up

pon 3/3:       admin-down

pon 3/4:       admin-up

    - show psg

This command is to display PSG group

show psg slot slot_id

【Parameter】

slot_id:slot id,the range is 2--5

【Command mode】

All global configuration mode

【Example】

！Display PSG 1

OptiWay(config)#show psg slot 3

PSG ID     active pon      standby pon     administrative status

psg-3/1      pon-3/1        pon-3/2          admin-down

    - show admin-enable-psg

This command is to display PSG group enable

show admin-enable-psg slot slot_id [ psg-id psg_id ]

【Parameter】

slot_id:slot id

- psg_id:psg id

【Command mode】

All global configuration mode

【Example】

！Display PSG group enable

OptiWay(config)#show admin-enable-psg slot 3 psg-id 1

PSG port      administrative status

psg 3/1:       admin-up

# Controllable Multicast Profile Managmenet Commands

  - Controllable multicast profile management commands

Controllable multicast profile management commands includs:

- multicast-ctrl profile

- multicast-ctrl

- show multicast-ctrl profile

- enable/disable multicast-ctrl profile

- onu-multicast-ctrl profile-binding

    - multicast-ctrl profile

This command is to creat/delete controllable multicast profile

multicast-ctrl profile  profile_id

no multicast-ctrl profile  profile_id

【Parameter】

profile_id:controllable profile number,the range is 1－32

【Command mode】

All global configuration mode

【Example】

！Creat controllable multicast profile 1

OptiWay(config)# multicast-ctrl profile 1

！Delete controllable multicast profile 1

OptiWay(config)# no multicast-ctrl profile 1

    - multicast-ctrl

This command is to add/delete controllable multicast profile privilege control table

multicast-ctrl  { permit | preview } { H:H:H:H:H:H vlan_id }

no  multicast-ctrl  { H:H:H:H:H:H | vlan_id }

【Parameter】

H:H:H:H:H:H: means MAC address

vlan_id:means VLAN id,the range is 1－4094

【Command mode】

Profile mode

【Example】

！Configure controllable multicast profile privilege:

OptiWay(multicast-profile-1)#multicast-ctrl permit 01:00:5e:01:02:03 2

！Delete controllable multicast profile privilege:

OptiWay(multicast-profile-1)#no multicast-ctrl 01:00:5e:01:02:03

    - show multicast-ctrl profile

This command is to display controllable multicast profile privilege control table

show multicast-ctrl profile profile_id

【Parameter】

profile_id:controllable multicast profile id,the range is 1－32

【Command mode】

Any configuration mode

【Example】

！Display controllable multicast profile 1 privilege control table

OptiWay(onu-3/3/1)#show multicast-ctrl profile 1

    - enable multicast-ctrl profile

This command is to enable controllable multicast profile privilege control table

enable multicast-ctrl profile profile_id

【Parameter】

profile_id:controllable multicast profile id,the range is 1－32

【Command mode】

Profile mode

【Example】

！Enable controllable multicast profile 1:

OptiWay(multicast-profile-1)#enable multicast-ctrl profile 1

    - onu-multicast-ctrl profile-binding

This command is to bind/delete onu and controllable multicast profile privilege control table

onu-multicast-ctrl profile-binding  profile_id  port_id

no onu-multicast-ctrl profile-binding  profile_id  port_id

【Parameter】

profile_id:controllable multicast profile id,the range is 1－32

port_id:onu port id

【Command mode】

ONU configuration mode

【Example】

！Bind onu3/3/1 controllable multicast profile 1 privilege control table:

OptiWay(onu-3/3/1)#onu-multicast-ctrl profile-binding 1 0/2

！Delete onu3/3/1 controllable multicast profile 1 privilege control table:

OptiWay(onu-3/3/1)#no onu-multicast-ctrl profile-binding 1 0/2